# DPTrainer — Curriculum Review

DPTrainer is a structured session companion for Trainee Dynamic Positioning Operators undertaking Nautical Institute passive simulator days. It gives a TDPO a task to work through, a standard to meet, fields to record against, and questions to answer, for each of the 30 days the scheme allows. It is not a training provider. It does not instruct, assess, certify or sign off. The supervising DPO does all of that; DPTrainer only structures the time.

## Source documents behind the curriculum

- The Nautical Institute — DP Operator’s Certification Scheme (New Offshore Scheme), logbook task sections 1–12

- DP Operator’s Handbook, 3rd Edition — Bray, Daniels, Fiander, Foster (The Nautical Institute)

- IMCA M117 Rev. 3.3 — The Training and Experience of Key DP Personnel

- IMCA M220 Rev. 3.0 (Jan 2025) — Recommended Practice on Operational Activity Planning (ASOG, CAM/TAM, TAGOS)

- IMCA M249 Rev. 1 — DP Practitioner Accreditation Scheme Handbook

- IMCA M273 (Jan 2026) — A Guide to Conducting DP Drills and Ensuring Preparedness for DP Failures

- IMO MSC.1/Circ.1580 (Jun 2017) — Guidelines for vessels and units with dynamic positioning (DP) systems

## Scope

19 domains · 72 tasks · 30 training days

## Reviewer instruction

Please mark anything unrealistic, missing, or wrongly placed within the 30 days. Marginal notes are more useful than a summary.

## Reviewer

| Name |  |
| --- | --- |
| Position |  |
| Organisation |  |
| Date |  |

Generated 10 September 2026

# Day 1 — System Setup and Basic Vessel Control

120 min across 5 slots · target 120 min

## Slot 1 · setup_001 — Full System Setup From Scratch

| Domain | System Setup and Initialisation |
| --- | --- |
| Competency | System initialisation |
| Difficulty | 1 of 5 |
| Duration | 20 min |
| Vessel modes | Off → DP |

### NI task references

Reference

1.1

1.2

4.12

### Setup

Cold screen. No pre-configuration. Start from the beginning.

### Execute

Complete the full simulator setup procedure without using any guide. Enter real vessel position from GPS display, heading from gyrocompass, draft from stability display, water depth from echosounder. Confirm all sensors active and healthy. Confirm all thrusters enabled. Set environment to calm. Run a 3-minute steady state check — vessel must hold position with minimal thruster activity before you declare setup complete.

### Professional standard

- All fields correctly populated from live vessel data

- All sensors showing green — no rejections or warnings

- All thrusters enabled and showing in schematic

- Vessel stable after 3-minute steady state check

- Setup completed within 15 minutes

### Observe and record

| Field | Type |
| --- | --- |
| Time to complete setup (minutes) | number |
| Any items missed or incorrect on first attempt | text |
| Sensor status at completion — all green? | text |
| Thruster status at completion — all enabled? | text |
| Steady state check result — vessel stable? | text |
| Notes | text |

### Debrief questions

1. What would happen to the vessel model if you entered the wrong water depth — how does an incorrect depth affect the simulation?

2. Why must the initial heading entered match the actual vessel heading precisely — what goes wrong if it does not?

3. What does the 3-minute steady state check confirm — and what does it tell you if the vessel is not stable?

4. Which sensor is most critical for DP position keeping and why?

5. If one gyro was showing a 2 degree difference from the other at setup — would you proceed? What would you do and why?

### Variants (2)

setup_001a — Setup Against the Clock

| Change | Value |
| --- | --- |
| Target time minutes | 10 |

Same setup, harder target. A professional DPO can set up efficiently. Aim for 10 minutes without sacrificing accuracy.

setup_001b — Setup With One Deliberate Error

| Change | Value |
| --- | --- |
| Dpo introduces error | yes |

Your DPO will introduce one deliberate misconfiguration during setup. Find it before starting any drill. This simulates inheriting a badly configured desk.

## Slot 2 · jc_001 — Pure Drift Stop

| Domain | Joystick Control and Vessel Handling |
| --- | --- |
| Competency | Stopping and speed management |
| Difficulty | 1 of 5 |
| Duration | 20 min |
| Vessel modes | DP → JSMH |

### NI task references

Reference

7.5

7.14

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Note your exact current position on the motion page. This is your datum. You will not be returning to it in this task — the only goal is stopping.

### Execute

Select JSMH and set all joystick inputs to zero. Let the vessel drift freely for exactly 4 minutes. Do not touch any controls. When 4 minutes expires, using joystick only, bring the vessel to a complete stop. Both velocity axes must read below 0.1kts. Do not return to datum — stopping is the entire task.

### Professional standard

- Stopped within 3 deliberate joystick inputs

- Residual velocity below 0.1kts on both axes

- Heading maintained throughout — no yaw during the stop

- No overcorrection after initial input

### Observe and record

| Field | Type |
| --- | --- |
| Drift direction (degrees) | number |
| Drift distance (metres) | number |
| Vessel speed at first input (kts) | number |
| Total joystick inputs | number |
| Residual velocity at stop (kts) | number |
| Heading change during stop (degrees) | number |
| Time from first input to stop (seconds) | number |
| Notes | text |

### Debrief questions

1. Which direction did the vessel drift and which environmental force dominated — wind, wave or current? How do you know?

2. At the speed your vessel was moving when you intervened, and knowing your stopping distance — at what range from a hard exclusion zone would you have needed to begin your deceleration to stop cleanly?

3. Why does stopping a vessel into the weather require different technique to stopping beam-on to it?

4. If wind increased from 15kts to 25kts mid-stop, what would change about your technique and why?

5. The DP mathematical model builds its understanding of environmental forces partly from thruster activity. After a large joystick transit, what does the model incorrectly learn — and why does selecting Monitor briefly before DP help the model reset more accurately than going straight from JSMH to DP?

### Variants (4)

jc_001a — Extended Drift Stop

| Change | Value |
| --- | --- |
| Drift minutes | 6 |

More speed to manage. Start your deceleration earlier than you think you need to.

jc_001b — Beam Drift Stop

| Change | Value |
| --- | --- |
| Environment — Wind dir deg | 90 |

The vessel will behave completely differently. Tunnel thruster geometry means beam stopping authority differs from head/stern stopping authority. Notice which thrusters the system uses.

jc_001c — Drift Stop with Position Constraint

| Change | Value |
| --- | --- |
| Position constraint metres | 50 |

Now you have two objectives. Stopping cleanly AND where you stop matters. This begins introducing position awareness into speed management.

jc_001d — Drift, Stop and Return

| Change | Value |
| --- | --- |
| Add return phase | yes |
| Return accuracy metres | 2 |
| Final mode | DP |

This is the complete manoeuvre. Three distinct phases. Do not start the return until velocity is below 0.1kts. Do not engage DP until you are within the DP operating radius and below 0.1kts.

## Slot 3 · jc_002 — Controlled Surge Move

| Domain | Joystick Control and Vessel Handling |
| --- | --- |
| Competency | Controlled position moves |
| Difficulty | 1 of 5 |
| Duration | 25 min |
| Vessel modes | DP position move or JSMH |

### NI task references

Reference

7.5

7.5.1

### Environment

| Setting | Value |
| --- | --- |
| First run | calm |
| Subsequent runs — Wind speed kts | 15 |
| Subsequent runs — Wind dir deg | 37 |

### Setup

Confirm your exact datum position. You will return to within 2m of this position at the end of each run.

### Execute

Move the vessel exactly 100m North from datum. Stop with residual velocity below 0.1kts. Return to within 2m of original datum position. This is one run. Complete three runs.

### Professional standard

- Overshoot less than 5m on return each run

- Residual velocity below 0.1kts at stop each run

- Improvement visible between run 1 and run 3

- Heading maintained throughout

### Observe and record

Run 1

| Field | Type |
| --- | --- |
| Overshoot (metres) | number |
| Residual velocity (kts) | number |
| Total inputs | number |
| Return accuracy — distance from exact datum (metres) | number |
| Notes | text |

Run 2

| Field | Type |
| --- | --- |
| Overshoot (metres) | number |
| Residual velocity (kts) | number |
| Total inputs | number |
| Return accuracy — distance from exact datum (metres) | number |
| Notes | text |

Run 3

| Field | Type |
| --- | --- |
| Overshoot (metres) | number |
| Residual velocity (kts) | number |
| Total inputs | number |
| Return accuracy — distance from exact datum (metres) | number |
| Notes | text |

### Debrief questions

1. Did your overshoot reduce between run 1 and run 3? What specifically changed in your technique?

2. At what point in each run did you decide to begin decelerating — and in hindsight was that the right point?

3. Why is a 100m surge move (North/South) typically easier to execute cleanly than a 100m sway move (East/West) on most DP vessels? Think about thruster geometry.

4. In a real operation near a rig, what would be the consequence of a 5m overshoot versus a 15m overshoot?

### Variants (4)

jc_002a — Short Surge Move

| Change | Value |
| --- | --- |
| Move distance metres | 50 |

Shorter distance, less time to manage deceleration. Harder than it sounds.

jc_002b — Long Surge Move

| Change | Value |
| --- | --- |
| Move distance metres | 200 |

More speed builds, earlier deceleration decision required.

jc_002c — Surge Move in Wind

| Change | Value |
| --- | --- |
| Environment — Wind speed kts | 15 |
| Environment — Wind dir deg | 37 |

Same task, different feel. Apply all three runs in the same wind condition.

jc_002d — Controlled Sway Move

| Change | Value |
| --- | --- |
| Move direction | East |
| Move distance metres | 100 |

Direct comparison of surge vs sway handling. Same distance, same standard. Notice what is different.

## Slot 4 · jc_003 — Controlled Sway Move

| Domain | Joystick Control and Vessel Handling |
| --- | --- |
| Competency | Controlled position moves |
| Difficulty | 2 of 5 |
| Duration | 25 min |
| Vessel modes | JSMH or DP position move |

### NI task references

Reference

7.5

7.5.1

### Environment

| Setting | Value |
| --- | --- |
| First run | calm |
| Subsequent runs — Wind speed kts | 15 |
| Subsequent runs — Wind dir deg | 37 |

### Setup

Confirm exact datum position. You will return to within 2m of this position at the end of each run.

### Execute

Move the vessel exactly 100m East from datum. Stop with residual velocity below 0.1kts. Return to within 2m of original datum. This is one run. Complete three runs. Compare your results to jc_002.

### Professional standard

- Overshoot less than 5m on return each run

- Residual velocity below 0.1kts at stop each run

- Improvement visible between run 1 and run 3

- Heading maintained throughout

### Observe and record

Run 1

| Field | Type |
| --- | --- |
| Overshoot (metres) | number |
| Residual velocity (kts) | number |
| Total inputs | number |
| Return accuracy — distance from datum (metres) | number |
| Time to complete run (seconds) | number |
| Notes | text |

Run 2

| Field | Type |
| --- | --- |
| Overshoot (metres) | number |
| Residual velocity (kts) | number |
| Total inputs | number |
| Return accuracy — distance from datum (metres) | number |
| Time to complete run (seconds) | number |
| Notes | text |

Run 3

| Field | Type |
| --- | --- |
| Overshoot (metres) | number |
| Residual velocity (kts) | number |
| Total inputs | number |
| Return accuracy — distance from datum (metres) | number |
| Time to complete run (seconds) | number |
| Notes | text |

### Debrief questions

1. Was the sway move harder or easier than the surge move in jc_002? What specifically explains the difference?

2. Which thrusters did the most work during the sway move — and why those ones based on their position on the vessel?

3. What does this tell you about the vessel's athwartship stopping authority compared to fore-aft?

4. In a real operation, when would you need to make a pure sway move rather than repositioning on DP?

### Variants (2)

jc_003a — Short Sway Move

| Change | Value |
| --- | --- |
| Move distance metres | 50 |

Shorter distance — less time to manage deceleration. Harder than it sounds in the athwartship axis.

jc_003b — Sway Move in Wind

| Change | Value |
| --- | --- |
| Environment — Wind speed kts | 15 |
| Environment — Wind dir deg | 37 |

Same task in wind. Notice how beam-on wind affects sway control differently to how head wind affected surge control.

## Slot 5 · buffer_001 — buffer

Buffer slot · 30 min — Additional joystick practice — repeat jc_001 or jc_002 variant

# Day 2 — Joystick Precision and Environmental Introduction

120 min across 5 slots · target 120 min

## Slot 1 · setup_002 — Checklist Discipline

| Domain | System Setup and Initialisation |
| --- | --- |
| Competency | System initialisation |
| Difficulty | 1 of 5 |
| Duration | 15 min |
| Vessel modes | Off → DP |

### NI task references

Reference

1.1.4

1.3.4

### Setup

Paper and pen only. App closed.

### Execute

Before opening the app, write down every item on the pre-session checklist from memory. All 16 items, in order, with a one-sentence explanation of why each item matters. Then open the app and compare your list. Note any items missed, any items in wrong order, any explanations that were incomplete.

### Professional standard

- Recall minimum 14 of 16 items without reference

- Correct explanation for each item recalled

- Identify own gaps accurately

### Observe and record

| Field | Type |
| --- | --- |
| Items recalled without reference | number |
| Items missed — which ones? | text |
| Items recalled but in wrong order | text |
| Which item was hardest to explain and why? | text |
| Notes | text |

### Debrief questions

1. Which checklist items have the highest immediate safety significance if skipped?

2. What is the consequence of starting a drill with a reference system misconfigured?

3. Why does the checklist require DPO presence to be confirmed before anything else — not after setup?

4. If you could only check five items due to time pressure, which five would you choose and why?

### Variants (1)

setup_002a — Checklist From Memory — No Mistakes

| Change | Value |
| --- | --- |
| Target score | 16 |

All 16 items, correct order, correct explanation. This is the standard a DPO would expect before signing off Section 1 of your logbook.

## Slot 2 · jc_004 — Diagonal Move

| Domain | Joystick Control and Vessel Handling |
| --- | --- |
| Competency | Controlled position moves |
| Difficulty | 2 of 5 |
| Duration | 25 min |
| Vessel modes | JSMH |

### NI task references

Reference

7.5

7.5.1

### Environment

Wind 0 kt @ 000° · Sea 0 m · Current 0 kt

### Setup

Confirm datum. Calm conditions only for first run.

### Execute

Move the vessel 100m Northeast simultaneously — not North then East in sequence, but both axes at the same time. Stop with residual below 0.1kts on both axes. Return to datum. Three runs.

### Professional standard

- Heading maintained within 3 degrees throughout

- Both axes within 5m overshoot on return

- Residual velocity below 0.1kts at stop

- Smooth coordinated inputs — no axis fighting

### Observe and record

Run 1

| Field | Type |
| --- | --- |
| Heading drift during move (degrees) | number |
| Which axis was harder to control? | text |
| Overshoot North axis (metres) | number |
| Overshoot East axis (metres) | number |
| Total joystick inputs | number |
| Notes | text |

Run 2

| Field | Type |
| --- | --- |
| Heading drift during move (degrees) | number |
| Which axis was harder to control? | text |
| Overshoot North axis (metres) | number |
| Overshoot East axis (metres) | number |
| Total joystick inputs | number |
| Notes | text |

Run 3

| Field | Type |
| --- | --- |
| Heading drift during move (degrees) | number |
| Which axis was harder to control? | text |
| Overshoot North axis (metres) | number |
| Overshoot East axis (metres) | number |
| Total joystick inputs | number |
| Notes | text |

### Debrief questions

1. Which axis was harder to control simultaneously and what caused the difficulty?

2. Did your heading drift during the move — what caused it and how did you correct?

3. In what real operational scenario would a diagonal move be required rather than sequential axis moves?

4. How does the DP thrust allocation algorithm handle simultaneous surge and sway demand — does it prioritise one axis?

### Variants (1)

jc_004a — Diagonal Move in Moderate Conditions

| Change | Value |
| --- | --- |
| Environment — Wind speed kts | 15 |
| Environment — Wind dir deg | 37 |

Same task with wind. The wind vector intersects your movement vector — notice how this changes which axis is harder.

## Slot 3 · env_001 — Environment Introduction — Progressive Build

| Domain | Environmental Awareness and Model Behaviour |
| --- | --- |
| Competency | Environmental awareness |
| Difficulty | 1 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

7.13

7.17

### Setup

Vessel on DP in calm conditions. Record baseline thruster output for all units — this is your zero reference. Before adding each stage, write down your prediction of which thrusters will increase and by how much.

### Execute

Add environmental conditions in three stages. Stage 1: wind 8kts at 037 degrees. Let settle 3 minutes. Record thruster response. Compare to prediction. Stage 2: add current 0.3kts at 090 degrees. Let settle 3 minutes. Record changes. Compare to prediction. Stage 3: add wave 1.0m at 048 degrees. Let settle 3 minutes. Record changes. Compare to prediction.

### Professional standard

- Accurate prediction of which thrusters increase before each stage

- Correct identification of dominant force at each stage

- Explanation of why specific thrusters responded

- Current vector reading compared to set conditions

### Observe and record

| Field | Type |
| --- | --- |
| Baseline thruster output all units at calm (percent each) | text |
| Prediction before Stage 1 — which thrusters increase? | text |
| Actual thruster response after Stage 1 wind 8kts | text |
| Current vector reading after Stage 1 | text |
| Prediction before Stage 2 — which thrusters increase? | text |
| Actual thruster response after Stage 2 current added | text |
| Current vector reading after Stage 2 | text |
| Prediction before Stage 3 — which thrusters increase? | text |
| Actual thruster response after Stage 3 wave added | text |
| Your assessment of combined environmental force direction | text |
| Notes | text |

### Debrief questions

1. Which thruster increased most when you added wind — and does its position on the vessel explain why?

2. Why did the current vector reading change when you added actual current, even though the tidal current was already there?

3. What is the combined environmental force vector direction given wind from 037, wave from 048, and current from 090?

4. How does wave loading differ from wind loading in terms of how the DP responds to it?

### Variants (1)

env_001a — Adverse Environment Build

| Change | Value |
| --- | --- |
| Wind speed kts | 25 |
| Current speed kts | 0.8 |

Higher forces — predictions need to account for potential saturation. Watch which thruster hits its limit first.

## Slot 4 · jc_005 — Heading Change at Controlled Rate

| Domain | Joystick Control and Vessel Handling |
| --- | --- |
| Competency | Heading control |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP auto heading |

### NI task references

Reference

7.5

7.12

### Environment

| Setting | Value |
| --- | --- |
| First run — Wind speed kts | 0 |
| Second run — Wind speed kts | 15 |
| Second run — Wind dir deg | 37 |

### Setup

Note current heading. This is your start heading. Your target is current heading plus 45 degrees.

### Execute

Change vessel heading by 45 degrees at no more than 2 degrees per minute. Stop exactly on the new heading. Record position excursion during the turn. Then return to original heading at 5 degrees per minute. Record excursion on return.

### Professional standard

- Rate of turn does not exceed 2 degrees per minute on slow turn

- Position excursion under 5m during turn

- Stop within 1 degree of target heading

- Heading maintained on return

### Observe and record

| Field | Type |
| --- | --- |
| Start heading (degrees) | number |
| Target heading (degrees) | number |
| Maximum position excursion during slow turn (metres) | number |
| Heading accuracy at stop — degrees off target | number |
| Maximum position excursion during fast return (metres) | number |
| Describe thruster pattern during turn | text |
| Notes | text |

### Debrief questions

1. Why did position keeping degrade during the heading change even though you were on DP auto position?

2. Why would you limit rate of turn to 2 degrees per minute near a structure?

3. What happens to thruster allocation geometry as the vessel rotates — which thrusters change role?

4. When would you use a fast heading change versus a slow controlled turn operationally?

### Variants (2)

jc_005a — 90 Degree Heading Change

| Change | Value |
| --- | --- |
| Heading change degrees | 90 |

Larger rotation means longer period of degraded position keeping. Plan for more excursion and debrief on where the worst excursion occurred in the turn.

jc_005b — Heading Change Near Structure

| Change | Value |
| --- | --- |
| Virtual structure active | yes |
| Standoff metres | 150 |

Same turn but with a virtual structure 150m away. Your excursion margin is now constrained. Does your technique change?

## Slot 5 · buffer_002 — buffer

Buffer slot · 30 min — Repeat env_001 with different heading to compare thruster response

# Day 3 — Mode Transitions and Environmental Loading

150 min across 6 slots · target 150 min

## Slot 1 · mode_003 — JSAH vs JSMH Comparison

| Domain | Mode Transitions and Model Management |
| --- | --- |
| Competency | Mode transitions |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → JSMH and JSAH alternating |

### NI task references

Reference

7.4

7.5

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP at datum. Moderate conditions settled.

### Execute

Part 1 — Surge move in JSMH: Select JSMH and make a controlled 100m surge move North. Record heading drift during the move. Return to datum. Part 2 — Same surge move in JSAH: Select JSAH and repeat the identical move. Record heading drift. Compare. Part 3 — Heading change in each mode: Attempt a 20 degree heading change in JSMH, then in JSAH. Compare the experience and result.

### Professional standard

- Correctly identify heading behaviour difference between modes

- Articulate when each mode is appropriate

- Complete all three parts with observations recorded

- Correct statement of fundamental difference between modes

### Observe and record

| Field | Type |
| --- | --- |
| Heading drift during surge move in JSMH (degrees) | number |
| Heading drift during surge move in JSAH (degrees) | number |
| Result of heading change attempt in JSMH | text |
| Result of heading change attempt in JSAH | text |
| In your own words — what is the fundamental difference between JSMH and JSAH? | text |
| JSMH is best suited for which situations? | text |
| JSAH is best suited for which situations? | text |
| Notes | text |

### Debrief questions

1. What is the fundamental difference between JSMH and JSAH in terms of what the DP system controls?

2. During the surge move in wind — which mode gave better heading control and why?

3. When would you choose JSMH over JSAH during a real approach to a structure?

4. What is the risk of using JSAH when close to a fixed structure in a cross-wind condition?

### Variants (1)

mode_003a — JSAH vs JSMH in Heavy Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 25 |

Same comparison in heavier conditions. The difference between modes becomes more pronounced as environmental loading increases.

## Slot 2 · jc_001 — Pure Drift Stop

| Domain | Joystick Control and Vessel Handling |
| --- | --- |
| Competency | Stopping and speed management |
| Difficulty | 1 of 5 |
| Duration | 20 min |
| Vessel modes | DP → JSMH |

### NI task references

Reference

7.5

7.14

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Note your exact current position on the motion page. This is your datum. You will not be returning to it in this task — the only goal is stopping.

### Execute

Select JSMH and set all joystick inputs to zero. Let the vessel drift freely for exactly 4 minutes. Do not touch any controls. When 4 minutes expires, using joystick only, bring the vessel to a complete stop. Both velocity axes must read below 0.1kts. Do not return to datum — stopping is the entire task.

### Professional standard

- Stopped within 3 deliberate joystick inputs

- Residual velocity below 0.1kts on both axes

- Heading maintained throughout — no yaw during the stop

- No overcorrection after initial input

### Observe and record

| Field | Type |
| --- | --- |
| Drift direction (degrees) | number |
| Drift distance (metres) | number |
| Vessel speed at first input (kts) | number |
| Total joystick inputs | number |
| Residual velocity at stop (kts) | number |
| Heading change during stop (degrees) | number |
| Time from first input to stop (seconds) | number |
| Notes | text |

### Debrief questions

1. Which direction did the vessel drift and which environmental force dominated — wind, wave or current? How do you know?

2. At the speed your vessel was moving when you intervened, and knowing your stopping distance — at what range from a hard exclusion zone would you have needed to begin your deceleration to stop cleanly?

3. Why does stopping a vessel into the weather require different technique to stopping beam-on to it?

4. If wind increased from 15kts to 25kts mid-stop, what would change about your technique and why?

5. The DP mathematical model builds its understanding of environmental forces partly from thruster activity. After a large joystick transit, what does the model incorrectly learn — and why does selecting Monitor briefly before DP help the model reset more accurately than going straight from JSMH to DP?

### Variants (4)

jc_001a — Extended Drift Stop

| Change | Value |
| --- | --- |
| Drift minutes | 6 |

More speed to manage. Start your deceleration earlier than you think you need to.

jc_001b — Beam Drift Stop

| Change | Value |
| --- | --- |
| Environment — Wind dir deg | 90 |

The vessel will behave completely differently. Tunnel thruster geometry means beam stopping authority differs from head/stern stopping authority. Notice which thrusters the system uses.

jc_001c — Drift Stop with Position Constraint

| Change | Value |
| --- | --- |
| Position constraint metres | 50 |

Now you have two objectives. Stopping cleanly AND where you stop matters. This begins introducing position awareness into speed management.

jc_001d — Drift, Stop and Return

| Change | Value |
| --- | --- |
| Add return phase | yes |
| Return accuracy metres | 2 |
| Final mode | DP |

This is the complete manoeuvre. Three distinct phases. Do not start the return until velocity is below 0.1kts. Do not engage DP until you are within the DP operating radius and below 0.1kts.

## Slot 3 · env_002 — Minimum Power Heading

| Domain | Environmental Awareness and Model Behaviour |
| --- | --- |
| Competency | Environmental awareness |
| Difficulty | 2 of 5 |
| Duration | 25 min |
| Vessel modes | DP → DP |

### NI task references

Reference

7.12

7.13

### Environment

Wind 20 kt @ 037° · Current 0.5 kt @ 090°

### Setup

Set environment as specified. Allow 5 minutes to settle. Record total thruster demand at current heading — this is your baseline.

### Execute

Test four different headings spaced 45 degrees apart. Hold each heading for 3 minutes and record total thruster demand. Find the heading that gives minimum total demand. Hold that heading for 5 minutes and confirm it is stable.

### Professional standard

- Test minimum four headings systematically

- Correctly identify minimum power heading

- Achieve measurable demand reduction from starting heading

- Hold minimum power heading stably for 5 minutes

### Observe and record

| Field | Type |
| --- | --- |
| Starting heading (degrees) | number |
| Total thruster demand at starting heading (percent) | number |
| Heading 1 tested (degrees) | number |
| Demand at heading 1 (percent) | number |
| Heading 2 tested (degrees) | number |
| Demand at heading 2 (percent) | number |
| Heading 3 tested (degrees) | number |
| Demand at heading 3 (percent) | number |
| Heading 4 tested (degrees) | number |
| Demand at heading 4 (percent) | number |
| Minimum power heading found (degrees) | number |
| Demand reduction achieved vs starting heading (percent) | number |
| Notes | text |

### Debrief questions

1. What is the minimum power heading you found and why does that specific heading minimise demand?

2. How does this concept apply to real operations where your heading is partly constrained by the work being done?

3. What is weathervaning and when would you use Minimum Power mode instead of holding a fixed heading?

4. How would the minimum power heading change if the current direction shifted by 45 degrees?

### Variants (1)

env_002a — Minimum Power Heading — Heavy Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 30 |
| Current speed kts | 0.8 |

Higher forces make heading selection more critical. The difference in demand between best and worst heading will be much larger.

## Slot 4 · jc_006 — Emergency Stop from Speed

| Domain | Joystick Control and Vessel Handling |
| --- | --- |
| Competency | Stopping and speed management |
| Difficulty | 3 of 5 |
| Duration | 20 min |
| Vessel modes | JSMH |

### NI task references

Reference

7.5

7.14

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Moderate conditions set. Vessel on DP at datum. Identify a clear stopping lane with no constraints.

### Execute

Deliberately build the vessel to 1.0kt forward speed using joystick. Then stop as quickly and cleanly as possible. Record stopping distance. Reset to datum. Repeat three times — try to improve stopping distance each run.

### Professional standard

- Stop within 30m from 1.0kt in moderate conditions

- Residual velocity below 0.1kts at stop

- No heading change greater than 5 degrees during stop

- Improvement between run 1 and run 3

### Observe and record

Run 1

| Field | Type |
| --- | --- |
| Speed at intervention (kts) | number |
| Stopping distance (metres) | number |
| Residual velocity at stop (kts) | number |
| Heading change during stop (degrees) | number |
| Joystick inputs used | number |
| Notes | text |

Run 2

| Field | Type |
| --- | --- |
| Speed at intervention (kts) | number |
| Stopping distance (metres) | number |
| Residual velocity at stop (kts) | number |
| Heading change during stop (degrees) | number |
| Joystick inputs used | number |
| Notes | text |

Run 3

| Field | Type |
| --- | --- |
| Speed at intervention (kts) | number |
| Stopping distance (metres) | number |
| Residual velocity at stop (kts) | number |
| Heading change during stop (degrees) | number |
| Joystick inputs used | number |
| Notes | text |

### Debrief questions

1. What was your stopping distance from 1.0kt? Is that a safe margin at 100m from a hard exclusion zone?

2. What specific technique change reduced your stopping distance between runs?

3. How does heading relative to wind affect stopping efficiency — is it easier to stop into the weather or with it?

4. At what speed would you consider the vessel out of safe control in a DP operational context near structure?

### Variants (2)

jc_006a — Emergency Stop from Higher Speed

| Change | Value |
| --- | --- |
| Target speed kts | 1.5 |

Faster entry speed. Stopping distance will increase non-linearly. Notice how much earlier you need to intervene.

jc_006b — Emergency Stop in Heavy Weather

| Change | Value |
| --- | --- |
| Environment — Wind speed kts | 25 |
| Environment — Wave height m | 2 |

Heavy conditions change everything about stopping. Environmental forces may assist or resist depending on your heading. Assess before you act.

## Slot 5 · act_001 — ASOG Traffic Light Interpretation

| Domain | ASOG, CAM & TAM Decision Support |
| --- | --- |
| Competency | Decision support interpretation |
| Difficulty | 1 of 5 |
| Duration | 30 min |
| Task type | Applied exercise |
| Vessel modes | DP → DP — sustained hold while working scenarios |

### NI task references

Reference

7.21

7.23

7.24

### Setup

Vessel settled on DP holding position at a safe open-water datum with the model built; the vessel remains on a sustained DP hold while the scenarios are worked at the desk. TDPO reviews the vessel's actual ASOG table (or a representative excerpt) prior to the session. DPO or supervisor prepares 3-4 short written scenarios describing a fault, alarm, or environmental change.

### Execute

For each scenario presented, the TDPO must: (1) classify the correct DP status colour, (2) state who must be notified and by when, (3) state the required response action, and (4) explain the reasoning using the ASOG's own criteria rather than guesswork. Throughout, the TDPO references the live thruster loading, position footprint and environmental readings on the DP desk when justifying each answer.

### Professional standard

- Correctly classifies status for at least 3 of 4 scenarios without prompting

- Does not confuse Blue (advisory, operations continue) with Yellow (degraded, prepare to suspend)

- Correctly identifies that a change to Blue does NOT require ceasing ongoing DP operations

- States the notification chain accurately for the vessel's own management structure

### Observe and record

| Field | Type |
| --- | --- |
| Scenario presented | text |
| Status identified (Green/Blue/Yellow/Red) | text |
| Correctly classified? | boolean |
| Notifications stated | text |
| Response action stated | text |
| Notes | text |

### Debrief questions

1. What is the practical difference between the Blue and Yellow response — why does Yellow require preparing to suspend operations while Blue does not?

2. If the DP system generates a consequence-analysis drift-off warning, which status does that typically trigger, and why does that differ from an ordinary equipment alarm?

3. The operation must not resume after a Yellow status until redundancy is regained — who makes that call on your vessel, and what evidence would they want to see first?

4. Can you think of a fault on your vessel's actual configuration that would immediately jump to Red rather than passing through Yellow first? Why does it skip a step?

### Variants (3)

act_001a — Single Thruster Loss Scenario

Loss of one bow thruster where two are fitted is the textbook Yellow example — redundancy lost, position still held. Confirm you can articulate why this isn't Red.

act_001b — Weather Forecast Advisory Scenario

| Change | Value |
| --- | --- |
| Scenario type | forecast_based |

A 6-hour forecast showing conditions that would compromise station-keeping after a worst-case failure is a Blue trigger even though nothing has actually failed yet. This tests whether you're waiting for a hard failure before reacting.

act_001c — SIMOPS-Specific Row Scenario

If your vessel's ASOG has a separate SIMOPS section, use a scenario where another vessel or structure in the field changes status — not your own equipment. Notice the response differs (escape routes, adjacent vessel comms).

## Slot 6 · ds_001 — Model-Build and Settling-Time Observation

| Domain | DP System Behaviour, Modes and Performance Monitoring |
| --- | --- |
| Competency | System understanding and model behaviour |
| Difficulty | 1 of 5 |
| Duration | 35 min |
| Vessel modes | JSMH → DP |

### NI task references

Reference

7.16

7.17

4.12

### Environment

Wind 12 kt @ 040° · Sea 0.8 m @ 055° · Current 0.2 kt @ 100°

### Setup

Have the vessel stationary or slow-moving under manual (JSMH) control. Note the exact time on the DP desk clock before you do anything else.

### Execute

Select DP. Do not touch any controls once selected. Watch the position and heading trend on the DP display for the full settling period — typically up to 30 minutes on older systems, sometimes 10-20 on newer ones. Record how the excursions reduce over time and note the point at which the vessel visibly stops hunting and settles to a steady residual.

### Professional standard

- No manual intervention once DP was selected

- Correctly identified the point the model appeared to settle, not just when the timer ran out

- Can explain in own words why a freshly-engaged DP system behaves like a helmsman just taking the wheel

- Logged actual settling time, not the textbook figure

### Observe and record

| Field | Type |
| --- | --- |
| Time DP selected | text |
| Time to apparent settling (minutes) | number |
| Max position excursion during settling (m) | number |
| Max heading excursion during settling (degrees) | number |
| Notes | text |

### Debrief questions

1. The DP model takes time to adapt to the vessel and the present environment, in the same way a helmsman takes a few minutes to get the feel of a vessel after taking the wheel. Why would selecting DP straight after a large heading change give you a worse settling period than selecting DP from steady, stationary conditions?

2. Some newer systems have shortened this settling period from 30 minutes to as little as 10. What does a DPO gain from that, and what might a DPO lose by relying on it during a safety-critical operation?

3. Your captain's technique after a large JSMH transit — slow to near zero, brief Monitor selection, then DP — relies on this same model-build principle. In your own words, why does that sequence give the model cleaner data to build from than going straight from JSMH to DP?

4. If you had watched this same exercise in deteriorating weather instead of steady conditions, what would you expect to look different about the settling trace?

### Variants (2)

ds_001a — Model-Build After Large Heading Change

| Change | Value |
| --- | --- |
| Pre dp heading change deg | 90 |

Execute a 90° manual heading change immediately before selecting DP, then run the same observation. Compare the settling trace directly against your ds_001 record.

ds_001b — Model-Build in Rising Wind

| Change | Value |
| --- | --- |
| Wind rising during settling | yes |
| Wind speed kts end | 22 |

Wind rises steadily through the settling window. Watch particularly for whether the model chases the change or the wind feed-forward function appears to compensate directly.

# Day 4 — Model Management and Mode Transitions

120 min across 5 slots · target 120 min

## Slot 1 · mode_001 — Clean Mode Transition — JSMH to Monitor to DP

| Domain | Mode Transitions and Model Management |
| --- | --- |
| Competency | Mode transitions |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → JSMH then Monitor then DP |

### NI task references

Reference

7.4

7.16

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP at datum. Moderate environment settled. Note current vector value before starting.

### Execute

Three runs. Each run: select JSMH with zero inputs and allow vessel to drift for 2 minutes. Then slow to near zero. Select Monitor briefly. Select DP. Record capture quality. Vary the vessel speed at the Monitor transition between runs — Run 1 transition at 0.3kts, Run 2 at 0.1kts, Run 3 at near zero. Compare capture quality across all three runs.

### Professional standard

- Cleanest capture when vessel nearest zero at Monitor transition

- Correct explanation of what Monitor does to the model

- Improvement in capture quality from Run 1 to Run 3

- Time to stable position reducing each run

### Observe and record

Run 1

| Field | Type |
| --- | --- |
| Vessel speed at Monitor selection (kts) | number |
| DP capture quality — smooth or hunting? | text |
| Time to stable position after DP selected (seconds) | number |
| Current vector value at moment of DP selection | text |
| Notes | text |

Run 2

| Field | Type |
| --- | --- |
| Vessel speed at Monitor selection (kts) | number |
| DP capture quality — smooth or hunting? | text |
| Time to stable position after DP selected (seconds) | number |
| Current vector value at moment of DP selection | text |
| Notes | text |

Run 3

| Field | Type |
| --- | --- |
| Vessel speed at Monitor selection (kts) | number |
| DP capture quality — smooth or hunting? | text |
| Time to stable position after DP selected (seconds) | number |
| Current vector value at moment of DP selection | text |
| Notes | text |

### Debrief questions

1. Why did DP capture quality improve when the vessel was closer to zero speed at the Monitor transition?

2. What does Monitor actually do to the DP mathematical model — is it a hard reset or something else?

3. How long did Monitor need to be active before DP capture improved — what does this tell you about the model?

4. When would you use this JSMH to Monitor to DP sequence in a real operational scenario?

### Variants (1)

mode_001a — Transition After Large Transit

| Change | Value |
| --- | --- |
| Pre transit distance metres | 500 |

Complete a 500m JSMH transit first, then apply the clean transition technique. This simulates moving between work locations. Model corruption will be greater — observe how this affects the required Monitor duration.

## Slot 2 · env_005 — Model Corruption and Recovery

| Domain | Environmental Awareness and Model Behaviour |
| --- | --- |
| Competency | Model behaviour |
| Difficulty | 3 of 5 |
| Duration | 25 min |
| Vessel modes | DP → JSMH then DP |

### NI task references

Reference

7.16

7.17

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Moderate environment set and settled. Vessel on DP at datum. Note current vector value before starting.

### Execute

Run 1 — Corrupted transition: Make an aggressive 300m JSMH transit with large inputs and abrupt stops. Immediately select DP. Record thruster behaviour and current vector for 2 minutes. Note time to settle. Run 2 — Clean transition: Reset. Make the same 300m transit but slow to near zero before selecting Monitor briefly, then DP. Record settle time and capture quality. Compare both runs.

### Professional standard

- Observe measurable difference in capture quality between runs

- Correctly identify current vector corruption after aggressive transit

- Explain the mechanism of model corruption

- Record settle time difference between methods

### Observe and record

| Field | Type |
| --- | --- |
| Current vector before transit (kts and direction) | text |
| Run 1 — Current vector immediately after aggressive transit | text |
| Run 1 — Describe thruster behaviour first 30 seconds on DP | text |
| Run 1 — Time to stable position (seconds) | number |
| Run 2 — Current vector after clean transition | text |
| Run 2 — Describe thruster behaviour first 30 seconds on DP | text |
| Run 2 — Time to stable position (seconds) | number |
| Difference in settle time between runs (seconds) | number |
| Notes | text |

### Debrief questions

1. What did the current vector show immediately after the aggressive transit and what caused that reading?

2. Why does slowing to near zero before the Monitor transition improve DP capture quality?

3. What does the Kalman filter inherit from a large JSMH transit — and why is that a problem?

4. How long did the model take to settle in each method — and what explains the difference in time?

### Variants (1)

env_005a — Model Corruption — Large Transit

| Change | Value |
| --- | --- |
| Transit distance metres | 1000 |

A much larger transit produces much greater corruption. This simulates moving between rigs. Notice how much longer the model takes to settle.

## Slot 3 · mode_002 — Minimum Power Mode Observation

| Domain | Mode Transitions and Model Management |
| --- | --- |
| Competency | Mode transitions |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP Minimum Power |

### NI task references

Reference

7.12

7.15

### Environment

Wind 25 kt @ 037° · Sea 1.5 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP holding a fixed heading. Record current heading and total thruster demand as baseline.

### Execute

Select Minimum Power mode. Observe as the vessel weathervanes toward the minimum resistance heading. Record what heading it settles on. Compare this to the minimum power heading you found manually in env_002 — do they agree? Hold the settled heading for 10 minutes and record demand reduction achieved compared to the fixed heading baseline.

### Professional standard

- Correctly observe and record weathervane behaviour

- Compare settled heading to env_002 result

- Record quantified demand reduction

- Understand operational limitations of this mode

### Observe and record

| Field | Type |
| --- | --- |
| Starting heading before Minimum Power selected (degrees) | number |
| Total thruster demand at starting heading (percent) | number |
| Heading vessel settled on in Minimum Power mode (degrees) | number |
| Total thruster demand at settled heading (percent) | number |
| Demand reduction achieved (percent) | number |
| Does settled heading agree with env_002 result? If not — why not? | text |
| Time for vessel to settle on minimum power heading (minutes) | number |
| Notes | text |

### Debrief questions

1. Did Minimum Power mode find the same heading you identified manually in env_002 — and if not what explains the difference?

2. When would you use Minimum Power mode operationally and when would you not use it?

3. What are the risks of selecting Minimum Power mode when near a structure?

4. How does weathervaning affect your vessel position relative to any nearby installation or subsea asset?

### Variants (1)

mode_002a — Minimum Power in Changing Conditions

| Change | Value |
| --- | --- |
| Wind direction change mid task | yes |

Your DPO will change the wind direction mid-task. Observe whether the vessel re-weathervanes. How quickly does it respond? What are the implications if this happens during an operation?

## Slot 4 · env_004 — Current Vector Analysis

| Domain | Environmental Awareness and Model Behaviour |
| --- | --- |
| Competency | Environmental awareness |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

7.13

7.17

### Setup

Set calm wind and wave. Add only current at 0.5kts from 090 degrees. Allow 10 minutes for model to fully settle before recording anything.

### Execute

After 10 minutes, record the current vector displayed by the DP system. Compare the displayed value to the actual current you set. Then add wind at 15kts from 180 degrees — directly opposing current. Observe how the displayed current vector changes over 5 minutes. Record the new value. Explain the difference.

### Professional standard

- Correctly record displayed vs set current values

- Observe and record change when wind added

- Correctly explain why displayed current changed when wind was added

- Understand that DP current vector represents all unmodelled forces

### Observe and record

| Field | Type |
| --- | --- |
| Current set in simulation (kts) | number |
| Current set direction (degrees) | number |
| DP displayed current vector after 10 minutes (kts) | number |
| DP displayed current vector direction after 10 minutes (degrees) | number |
| Did displayed value match set value — if not why not? | text |
| Displayed current vector after adding wind (kts) | number |
| Displayed current vector direction after adding wind (degrees) | number |
| Your explanation of why the vector changed when wind was added | text |
| Notes | text |

### Debrief questions

1. Did the displayed current vector match what you set in the simulation? If not — why not and what does this tell you?

2. When you added wind, why did the current vector change even though the tidal current did not change?

3. What does the DP current vector actually represent — is it purely tidal current?

4. How does this understanding affect how you interpret the current vector during a real operation?

### Variants (1)

env_004a — Current Vector in Complex Environment

| Change | Value |
| --- | --- |
| Wind speed kts | 20 |
| Wave height m | 1.5 |

Add all three environmental forces and analyse the current vector. How much of what you see is tidal current vs wind vs wave force?

## Slot 5 · buffer_004 — buffer

Buffer slot · 35 min — Repeat mode_001 after longer JSMH transit

# Day 5 — Foundation Review and Emergency Stopping

170 min across 6 slots · target 170 min

## Slot 1 · jc_006 — Emergency Stop from Speed

| Domain | Joystick Control and Vessel Handling |
| --- | --- |
| Competency | Stopping and speed management |
| Difficulty | 3 of 5 |
| Duration | 20 min |
| Vessel modes | JSMH |

### NI task references

Reference

7.5

7.14

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Moderate conditions set. Vessel on DP at datum. Identify a clear stopping lane with no constraints.

### Execute

Deliberately build the vessel to 1.0kt forward speed using joystick. Then stop as quickly and cleanly as possible. Record stopping distance. Reset to datum. Repeat three times — try to improve stopping distance each run.

### Professional standard

- Stop within 30m from 1.0kt in moderate conditions

- Residual velocity below 0.1kts at stop

- No heading change greater than 5 degrees during stop

- Improvement between run 1 and run 3

### Observe and record

Run 1

| Field | Type |
| --- | --- |
| Speed at intervention (kts) | number |
| Stopping distance (metres) | number |
| Residual velocity at stop (kts) | number |
| Heading change during stop (degrees) | number |
| Joystick inputs used | number |
| Notes | text |

Run 2

| Field | Type |
| --- | --- |
| Speed at intervention (kts) | number |
| Stopping distance (metres) | number |
| Residual velocity at stop (kts) | number |
| Heading change during stop (degrees) | number |
| Joystick inputs used | number |
| Notes | text |

Run 3

| Field | Type |
| --- | --- |
| Speed at intervention (kts) | number |
| Stopping distance (metres) | number |
| Residual velocity at stop (kts) | number |
| Heading change during stop (degrees) | number |
| Joystick inputs used | number |
| Notes | text |

### Debrief questions

1. What was your stopping distance from 1.0kt? Is that a safe margin at 100m from a hard exclusion zone?

2. What specific technique change reduced your stopping distance between runs?

3. How does heading relative to wind affect stopping efficiency — is it easier to stop into the weather or with it?

4. At what speed would you consider the vessel out of safe control in a DP operational context near structure?

### Variants (2)

jc_006a — Emergency Stop from Higher Speed

| Change | Value |
| --- | --- |
| Target speed kts | 1.5 |

Faster entry speed. Stopping distance will increase non-linearly. Notice how much earlier you need to intervene.

jc_006b — Emergency Stop in Heavy Weather

| Change | Value |
| --- | --- |
| Environment — Wind speed kts | 25 |
| Environment — Wave height m | 2 |

Heavy conditions change everything about stopping. Environmental forces may assist or resist depending on your heading. Assess before you act.

## Slot 2 · env_003 — Environmental Ramp to Saturation

| Domain | Environmental Awareness and Model Behaviour |
| --- | --- |
| Competency | Capability and limits |
| Difficulty | 3 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

7.13

7.17

7.21

### Setup

Vessel on DP in calm conditions. Enable Ramp mode for wind and wave. Set wind target 35kts at 037 degrees, wave target 3.0m at 048 degrees.

### Execute

Start the ramp and allow conditions to increase gradually. Monitor thruster loading continuously. Record at what wind speed the first thruster reaches saturation. Record at what point position footprint begins to degrade. Record at what point you would recommend to the OIM that operations should be reviewed.

### Professional standard

- Identify first saturation before position degrades

- Correct identification of which thruster saturates first

- Reasoned recommendation point with evidence

- Understanding of relationship between saturation and position accuracy

### Observe and record

| Field | Type |
| --- | --- |
| Wind speed at first thruster saturation (kts) | number |
| Which thruster saturated first? | text |
| Wind speed when position footprint began to degrade (kts) | number |
| Wind speed at which you would recommend reviewing operations (kts) | number |
| Reasoning for your recommendation | text |
| Total thruster demand at maximum conditions (percent) | number |
| Notes | text |

### Debrief questions

1. Which thruster saturated first and does its position on the vessel explain why it was first?

2. At what sea state would you recommend the OIM consider changing heading or suspending operations on this vessel?

3. What is a capability plot and how would it have predicted what you just observed in this exercise?

4. What is the relationship between thruster saturation and position keeping accuracy — why does one cause the other?

### Variants (1)

env_003a — Ramp at Non-Optimal Heading

| Change | Value |
| --- | --- |
| Heading beam on | yes |

Same ramp but with weather beam-on to the vessel. Saturation will occur earlier. Compare the result to your head-on ramp.

## Slot 3 · review_001 — Self-Assessment — End of Foundation Phase

| Domain | Self-Assessment and Progress Review |
| --- | --- |
| Competency | Self assessment |
| Difficulty | 1 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP — sustained hold while conducting review |

### NI task references

None recorded.

### Setup

Vessel on DP in calm conditions. Let the system run while you reflect.

### Execute

Hold position on DP. While the vessel holds, answer the following from memory without reference to notes. What have you learned in sessions 1 to 5 that surprised you? What skills feel genuinely confident? What feels uncertain? What questions remain unanswered? Discuss with your DPO at the end of the hold.

### Professional standard

- Honest assessment — not optimistic, not falsely modest

- Specific skills named not vague categories

- Genuine unanswered questions identified

- Agreed focus areas written down before ending the session

### Observe and record

| Field | Type |
| --- | --- |
| Skills you feel genuinely confident about after days 1-5 | text |
| Skills you feel uncertain or weak on | text |
| What surprised you in sessions 1-5? | text |
| Questions that remain unanswered — ask your DPO | text |
| Agreed focus areas for days 6-10 — agreed with DPO | text |
| Notes | text |

### Debrief questions

1. Are the skills you listed as confident ones you could demonstrate right now without preparation?

2. Are the skills you listed as weak ones you have actually practised or just read about?

3. Which of your unanswered questions is most important to resolve before the next phase?

4. What would a senior DPO say if they watched your best session from days 1 to 5?

## Slot 4 · act_002 — CAM vs TAM Configuration Recognition

| Domain | ASOG, CAM & TAM Decision Support |
| --- | --- |
| Competency | Decision support interpretation |
| Difficulty | 2 of 5 |
| Duration | 30 min |
| Task type | Applied exercise |
| Vessel modes | DP → DP — sustained hold while working scenarios |

### NI task references

Reference

7.21

7.22

7.24

### Setup

Vessel settled on DP holding position; the hold is sustained while the exercise is worked at the desk. TDPO reviews the vessel's CAM (and TAM, if the vessel operates one) configuration table. DPO presents a description of a planned activity (e.g. within 500m of a structure, IRM work, standby duty outside the zone).

### Execute

For each activity described, TDPO must state whether CAM or TAM applies, justify it against the vessel's documented criteria (not general assumption), and identify the minimum thruster/generator configuration required. When justifying each answer, the TDPO references the live thruster loading, position footprint and environmental readings, comparing the running configuration against the minimum the table requires.

### Professional standard

- Correctly defaults to CAM unless a documented, signed-off risk assessment permits TAM

- Does not treat TAM as merely 'a lower-effort mode' — can state the specific risk-acceptance basis

- Correctly identifies minimum generator/thruster count from the vessel's own CAM/TAM table

- Recognises that TAM does not remove the requirement for mitigating controls

### Observe and record

| Field | Type |
| --- | --- |
| Activity described | text |
| CAM or TAM selected | text |
| Justification given | text |
| Minimum generator/thruster config stated | text |
| Notes | text |

### Debrief questions

1. Why should CAM be the default, and TAM the exception requiring sign-off, rather than the other way round?

2. What's the difference between 'the vessel can lose position in TAM and that's accepted' versus 'TAM removes the need for controls' — why is that distinction important?

3. On your vessel, is there more than one CAM depending on which industrial mission is being performed? If so, what changes between them?

4. If you were only given the CAM/TAM table with no further context, could you tell from the table alone what triggers a return to CAM from TAM?

### Variants (1)

act_002a — IRM During Standby

Inspection/Repair/Maintenance work onboard is often explicitly prohibited during CAM operations. Check whether your vessel's ASOG title block states this — it usually does.

## Slot 5 · ds_002 — Gain and Footprint Comparison

| Domain | DP System Behaviour, Modes and Performance Monitoring |
| --- | --- |
| Competency | Power precision tradeoff |
| Difficulty | 2 of 5 |
| Duration | 40 min |
| Vessel modes | DP → DP |

### NI task references

Reference

7.15

8.5

7.17

### Environment

Wind 20 kt @ 030° · Sea 1.5 m @ 040° · Current 0.4 kt @ 095°

### Setup

With the vessel settled on DP in the given environment, select low gain and allow ten minutes for the system to settle into that setting before recording anything.

### Execute

Record footprint (position scatter) and average thruster load over a ten-minute period on low gain. Without changing the environment, switch to high gain, allow ten minutes to bed in, and record the same two measures again.

### Professional standard

- Environment held constant between the two recording periods

- Allowed a full settling period after each gain change before recording

- Could state, in figures, the trade-off between footprint tightness and thrust/fuel cost

- Correctly distinguished relaxed gain from simple low gain when asked

### Observe and record

| Field | Type |
| --- | --- |
| Low gain — position scatter radius (m) | number |
| Low gain — average thrust (%) | number |
| High gain — position scatter radius (m) | number |
| High gain — average thrust (%) | number |
| Notes | text |

### Debrief questions

1. Which gain setting held the tighter footprint, and what did that cost you in thrust and, by extension, fuel? Describe a real operation where paying that cost would clearly be worth it, and one where it wouldn't.

2. Relaxed gain lets the vessel drift freely within a set radius before the system intervenes at all. How is that functionally different from simply running on low gain?

3. If you were holding station 15m from a structure for a lift, which setting would you choose, and why would you accept the higher fuel burn that comes with it?

4. Under relaxed gain, power only increases once the vessel nears the edge of the set circle. What would tell you, before any alarm fires, that the vessel is approaching that edge?

### Variants (1)

ds_002a — Gain Comparison in Calm Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 8 |
| Wave height m | 0.4 |

Repeat the same comparison in calm conditions. The gap between low and high gain performance should be much smaller here — notice by how much, and why.

## Slot 6 · drill_001 — Auto/Manual Human Intervention Familiarisation

| Domain | DP Emergency Drills & Preparedness |
| --- | --- |
| Competency | Failure response procedural |
| Difficulty | 1 of 5 |
| Duration | 30 min |
| Task type | Live simulator drill |
| Vessel modes | DP → DP |

### NI task references

Reference

7.19

11.10

1.5

### Setup

Vessel settled on auto DP in a safe open area of the simulated environment, no proximity hazards, no risk of unsafe position excursion.

### Execute

One piece of running equipment at a time (thruster, engine, pump, controller) is switched to local/manual control by the supervising DPO while the TDPO watches the DP control screens. Before each switch, TDPO predicts aloud what should happen. After each switch, TDPO records what actually happened, then the DPO reinstates the equipment.

### Professional standard

- Predicts correctly, before the action, whether DP control will be affected

- Correctly identifies which alarms should appear and does appear

- Does not freeze or hesitate when an unexpected effect occurs — verbalises what is being observed

- Can state afterwards which of the tested items rely on operator intervention as part of the vessel's redundancy concept

### Observe and record

| Field | Type |
| --- | --- |
| Equipment tested | text |
| TDPO's prediction | text |
| Actual DP effect observed | text |
| Alarm(s) generated | text |
| Prediction correct? | boolean |
| Notes | text |

### Debrief questions

1. Were all the effects you observed ones you actually understood, or were some a surprise even after seeing them happen?

2. What should your response be, as a watchkeeper, if you saw someone switch a running thruster to local without warning you first?

3. Discuss the alternative actions available in response to an unplanned local-control switch — is there one correct response, or several defensible ones?

4. What gap in your own understanding did this drill expose that you'd want to close before doing this for real?

### Variants (2)

drill_001a — Thrusters Only

Restrict to thruster local/manual switching only. Good first pass before adding engines and pumps.

drill_001b — Power System Only (Engines, Pumps)

Restrict to engines and cooling/fuel pumps. Notice these often have less visible immediate effect on DP than thrusters do — that's the point.

# Day 6 — Sensor Management and Reference Systems

120 min across 5 slots · target 120 min

## Slot 1 · sen_001 — Reference System Weighting

| Domain | Sensor and Reference System Management |
| --- | --- |
| Competency | Reference management |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

5.1

5.2

7.9

7.10

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Two DGPS references active. Vessel stable on DP. Open the Sensors tab and observe the weighting assigned to each reference before doing anything.

### Execute

Record initial weights for both references. Ask your DPO to introduce simulated HDOP degradation on one reference. Observe how the weighting changes. Record the new weights. Note how position quality changes. Assess at what point you would manually inhibit the degraded reference rather than waiting for auto-rejection. Re-enable and confirm both references return to healthy status.

### Professional standard

- Correctly record initial reference weights

- Observe and record weight change after degradation

- Correct threshold assessment for manual inhibit

- Correct procedure for re-enabling recovered reference

### Observe and record

| Field | Type |
| --- | --- |
| Reference 1 initial weight | text |
| Reference 2 initial weight | text |
| Reference 1 weight after degradation introduced | text |
| Reference 2 weight after degradation introduced | text |
| How did position quality change after degradation? | text |
| At what point would you manually inhibit — and why that point? | text |
| Did both references return to healthy status on re-enable? | text |
| Notes | text |

### Debrief questions

1. How does the DP Kalman filter decide how much weight to assign each reference — what factors influence this?

2. What HDOP value would concern you and at what value would you act?

3. At what point would you manually inhibit a reference rather than waiting for auto-rejection — and what is the risk of waiting?

4. What is the difference between a reference being auto-rejected and manually inhibited — and does it matter which way it happens?

### Variants (1)

sen_001a — Reference Weighting with Three References

| Change | Value |
| --- | --- |
| Reference count | 3 |

Add a third reference and observe how the weighting distributes across three. Then degrade one — how do the remaining two respond?

## Slot 2 · sen_004 — Wind Feedforward Disable

| Domain | Sensor and Reference System Management |
| --- | --- |
| Competency | Sensor management |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

6.3

7.8

### Environment

Wind 20 kt @ 037°

| Setting | Value |
| --- | --- |
| Ramp enabled | yes |
| Ramp variation kts | 5 |

### Setup

Wind 20kts with ramp variation to simulate gusting. Vessel stable on DP with wind feedforward enabled. Record position footprint and thruster activity as baseline.

### Execute

With feedforward enabled, observe and record position footprint size and thruster response character during gusting for 3 minutes. Then disable wind feedforward. Observe the change in position keeping. Record how the position footprint changes and how thruster response changes from anticipatory to reactive. Re-enable feedforward and compare recovery.

### Professional standard

- Observe and record quantifiable difference in position footprint

- Correctly characterise thruster response as anticipatory vs reactive

- Understand the source of feedforward data

- Correct explanation of why feedforward improves gusty condition performance

### Observe and record

| Field | Type |
| --- | --- |
| Position footprint size with feedforward enabled | text |
| Describe thruster response character with feedforward on | text |
| Position footprint size with feedforward disabled | text |
| Describe thruster response character with feedforward off | text |
| Observed time lag between gust and correction without feedforward (seconds) | number |
| Your assessment of the difference — was it significant? | text |
| Notes | text |

### Debrief questions

1. What specifically changed about the thruster response pattern when feedforward was disabled?

2. Why does feedforward give better position keeping in gusty conditions compared to feedback alone?

3. What is the source of wind feedforward data — and what happens to position keeping if that sensor fails mid-operation?

4. In what sea conditions would losing wind feedforward be most operationally significant?

### Variants (1)

sen_004a — Wind Sensor Failure

| Change | Value |
| --- | --- |
| Wind sensor fails | yes |

Rather than disabling feedforward manually, your DPO will fail the wind sensor. Observe the alarm, assess the impact on position keeping, and decide whether to continue.

## Slot 3 · env_004 — Current Vector Analysis

| Domain | Environmental Awareness and Model Behaviour |
| --- | --- |
| Competency | Environmental awareness |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

7.13

7.17

### Setup

Set calm wind and wave. Add only current at 0.5kts from 090 degrees. Allow 10 minutes for model to fully settle before recording anything.

### Execute

After 10 minutes, record the current vector displayed by the DP system. Compare the displayed value to the actual current you set. Then add wind at 15kts from 180 degrees — directly opposing current. Observe how the displayed current vector changes over 5 minutes. Record the new value. Explain the difference.

### Professional standard

- Correctly record displayed vs set current values

- Observe and record change when wind added

- Correctly explain why displayed current changed when wind was added

- Understand that DP current vector represents all unmodelled forces

### Observe and record

| Field | Type |
| --- | --- |
| Current set in simulation (kts) | number |
| Current set direction (degrees) | number |
| DP displayed current vector after 10 minutes (kts) | number |
| DP displayed current vector direction after 10 minutes (degrees) | number |
| Did displayed value match set value — if not why not? | text |
| Displayed current vector after adding wind (kts) | number |
| Displayed current vector direction after adding wind (degrees) | number |
| Your explanation of why the vector changed when wind was added | text |
| Notes | text |

### Debrief questions

1. Did the displayed current vector match what you set in the simulation? If not — why not and what does this tell you?

2. When you added wind, why did the current vector change even though the tidal current did not change?

3. What does the DP current vector actually represent — is it purely tidal current?

4. How does this understanding affect how you interpret the current vector during a real operation?

### Variants (1)

env_004a — Current Vector in Complex Environment

| Change | Value |
| --- | --- |
| Wind speed kts | 20 |
| Wave height m | 1.5 |

Add all three environmental forces and analyse the current vector. How much of what you see is tidal current vs wind vs wave force?

## Slot 4 · sen_002 — GPS Failure Response

| Domain | Sensor and Reference System Management |
| --- | --- |
| Competency | Reference management |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

5.1

5.2

7.9

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Two DGPS references active. Vessel stable on DP in moderate conditions. Note current position quality.

### Execute

Without warning your DPO will disable one GPS reference. Observe the alarm sequence. Record which alarms triggered and in what order. Assess your remaining reference configuration. Decide whether to continue or go off DP. Re-enable the failed GPS and confirm re-acceptance procedure.

### Professional standard

- Alarm acknowledged within 15 seconds

- Correct assessment of remaining reference configuration

- Correct go or no-go decision with reasoning

- Correct re-acceptance procedure for recovered reference

### Observe and record

| Field | Type |
| --- | --- |
| Alarm sequence — which alarms in what order? | text |
| Time to acknowledge first alarm (seconds) | number |
| Your assessment of remaining reference configuration | text |
| Decision — continue or go off DP? Reasoning? | text |
| Position quality with single reference | text |
| How did you re-accept the recovered reference? | text |
| Notes | text |

### Debrief questions

1. What is the minimum reference configuration for your vessel's typical operations and where is this defined?

2. Why does losing one GPS not automatically mean you must go off DP?

3. What changed about your position quality with only one reference active?

4. When re-enabling a recovered reference — why must you confirm it is healthy before accepting it back into the solution?

### Variants (2)

sen_002a — GPS Failure Near Structure

| Change | Value |
| --- | --- |
| Virtual structure active | yes |
| Standoff metres | 150 |

Same failure but you are 150m from a virtual structure. Does proximity change your minimum reference requirement? Does it change your decision?

sen_002b — Both GPS References Degrade Simultaneously

| Change | Value |
| --- | --- |
| Both references degrade | yes |

Harder scenario. Both references degrade at the same time. What is your immediate action? This is a genuine off-DP decision.

## Slot 5 · buffer_006 — buffer

Buffer slot · 40 min — Sensor observation — document all reference system details for this vessel

# Day 7 — Drift Assessment and Recovery

120 min across 5 slots · target 120 min

## Slot 1 · jc_001 — Pure Drift Stop

| Domain | Joystick Control and Vessel Handling |
| --- | --- |
| Competency | Stopping and speed management |
| Difficulty | 1 of 5 |
| Duration | 20 min |
| Vessel modes | DP → JSMH |

### NI task references

Reference

7.5

7.14

7.18

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Note your exact current position on the motion page. This is your datum. You will not be returning to it in this task — the only goal is stopping.

### Execute

Select JSMH and set all joystick inputs to zero. Let the vessel drift freely for exactly 4 minutes. Do not touch any controls. When 4 minutes expires, using joystick only, bring the vessel to a complete stop. Both velocity axes must read below 0.1kts. Do not return to datum — stopping is the entire task.

### Professional standard

- Stopped within 3 deliberate joystick inputs

- Residual velocity below 0.1kts on both axes

- Heading maintained throughout — no yaw during the stop

- No overcorrection after initial input

### Observe and record

| Field | Type |
| --- | --- |
| Drift direction (degrees) | number |
| Drift distance (metres) | number |
| Vessel speed at first input (kts) | number |
| Total joystick inputs | number |
| Residual velocity at stop (kts) | number |
| Heading change during stop (degrees) | number |
| Time from first input to stop (seconds) | number |
| Notes | text |

### Debrief questions

1. Which direction did the vessel drift and which environmental force dominated — wind, wave or current? How do you know?

2. At the speed your vessel was moving when you intervened, and knowing your stopping distance — at what range from a hard exclusion zone would you have needed to begin your deceleration to stop cleanly?

3. Why does stopping a vessel into the weather require different technique to stopping beam-on to it?

4. If wind increased from 15kts to 25kts mid-stop, what would change about your technique and why?

5. The DP mathematical model builds its understanding of environmental forces partly from thruster activity. After a large joystick transit, what does the model incorrectly learn — and why does selecting Monitor briefly before DP help the model reset more accurately than going straight from JSMH to DP?

### Variants (4)

jc_001a — Extended Drift Stop

| Change | Value |
| --- | --- |
| Drift minutes | 6 |

More speed to manage. Start your deceleration earlier than you think you need to.

jc_001b — Beam Drift Stop

| Change | Value |
| --- | --- |
| Environment — Wind dir deg | 90 |

The vessel will behave completely differently. Tunnel thruster geometry means beam stopping authority differs from head/stern stopping authority. Notice which thrusters the system uses.

jc_001c — Drift Stop with Position Constraint

| Change | Value |
| --- | --- |
| Position constraint metres | 50 |

Now you have two objectives. Stopping cleanly AND where you stop matters. This begins introducing position awareness into speed management.

jc_001d — Drift, Stop and Return

| Change | Value |
| --- | --- |
| Add return phase | yes |
| Return accuracy metres | 2 |
| Final mode | DP |

This is the complete manoeuvre. Three distinct phases. Do not start the return until velocity is below 0.1kts. Do not engage DP until you are within the DP operating radius and below 0.1kts.

## Slot 2 · jc_001 — Pure Drift Stop

| Domain | Joystick Control and Vessel Handling |
| --- | --- |
| Competency | Stopping and speed management |
| Difficulty | 1 of 5 |
| Duration | 20 min |
| Vessel modes | DP → JSMH |

### NI task references

Reference

7.5

7.14

7.18

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Note your exact current position on the motion page. This is your datum. You will not be returning to it in this task — the only goal is stopping.

### Execute

Select JSMH and set all joystick inputs to zero. Let the vessel drift freely for exactly 4 minutes. Do not touch any controls. When 4 minutes expires, using joystick only, bring the vessel to a complete stop. Both velocity axes must read below 0.1kts. Do not return to datum — stopping is the entire task.

### Professional standard

- Stopped within 3 deliberate joystick inputs

- Residual velocity below 0.1kts on both axes

- Heading maintained throughout — no yaw during the stop

- No overcorrection after initial input

### Observe and record

| Field | Type |
| --- | --- |
| Drift direction (degrees) | number |
| Drift distance (metres) | number |
| Vessel speed at first input (kts) | number |
| Total joystick inputs | number |
| Residual velocity at stop (kts) | number |
| Heading change during stop (degrees) | number |
| Time from first input to stop (seconds) | number |
| Notes | text |

### Debrief questions

1. Which direction did the vessel drift and which environmental force dominated — wind, wave or current? How do you know?

2. At the speed your vessel was moving when you intervened, and knowing your stopping distance — at what range from a hard exclusion zone would you have needed to begin your deceleration to stop cleanly?

3. Why does stopping a vessel into the weather require different technique to stopping beam-on to it?

4. If wind increased from 15kts to 25kts mid-stop, what would change about your technique and why?

5. The DP mathematical model builds its understanding of environmental forces partly from thruster activity. After a large joystick transit, what does the model incorrectly learn — and why does selecting Monitor briefly before DP help the model reset more accurately than going straight from JSMH to DP?

### Variants (4)

jc_001a — Extended Drift Stop

| Change | Value |
| --- | --- |
| Drift minutes | 6 |

More speed to manage. Start your deceleration earlier than you think you need to.

jc_001b — Beam Drift Stop

| Change | Value |
| --- | --- |
| Environment — Wind dir deg | 90 |

The vessel will behave completely differently. Tunnel thruster geometry means beam stopping authority differs from head/stern stopping authority. Notice which thrusters the system uses.

jc_001c — Drift Stop with Position Constraint

| Change | Value |
| --- | --- |
| Position constraint metres | 50 |

Now you have two objectives. Stopping cleanly AND where you stop matters. This begins introducing position awareness into speed management.

jc_001d — Drift, Stop and Return

| Change | Value |
| --- | --- |
| Add return phase | yes |
| Return accuracy metres | 2 |
| Final mode | DP |

This is the complete manoeuvre. Three distinct phases. Do not start the return until velocity is below 0.1kts. Do not engage DP until you are within the DP operating radius and below 0.1kts.

## Slot 3 · jc_001 — Pure Drift Stop

| Domain | Joystick Control and Vessel Handling |
| --- | --- |
| Competency | Stopping and speed management |
| Difficulty | 1 of 5 |
| Duration | 20 min |
| Vessel modes | DP → JSMH |

### NI task references

Reference

7.5

7.14

7.18

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Note your exact current position on the motion page. This is your datum. You will not be returning to it in this task — the only goal is stopping.

### Execute

Select JSMH and set all joystick inputs to zero. Let the vessel drift freely for exactly 4 minutes. Do not touch any controls. When 4 minutes expires, using joystick only, bring the vessel to a complete stop. Both velocity axes must read below 0.1kts. Do not return to datum — stopping is the entire task.

### Professional standard

- Stopped within 3 deliberate joystick inputs

- Residual velocity below 0.1kts on both axes

- Heading maintained throughout — no yaw during the stop

- No overcorrection after initial input

### Observe and record

| Field | Type |
| --- | --- |
| Drift direction (degrees) | number |
| Drift distance (metres) | number |
| Vessel speed at first input (kts) | number |
| Total joystick inputs | number |
| Residual velocity at stop (kts) | number |
| Heading change during stop (degrees) | number |
| Time from first input to stop (seconds) | number |
| Notes | text |

### Debrief questions

1. Which direction did the vessel drift and which environmental force dominated — wind, wave or current? How do you know?

2. At the speed your vessel was moving when you intervened, and knowing your stopping distance — at what range from a hard exclusion zone would you have needed to begin your deceleration to stop cleanly?

3. Why does stopping a vessel into the weather require different technique to stopping beam-on to it?

4. If wind increased from 15kts to 25kts mid-stop, what would change about your technique and why?

5. The DP mathematical model builds its understanding of environmental forces partly from thruster activity. After a large joystick transit, what does the model incorrectly learn — and why does selecting Monitor briefly before DP help the model reset more accurately than going straight from JSMH to DP?

### Variants (4)

jc_001a — Extended Drift Stop

| Change | Value |
| --- | --- |
| Drift minutes | 6 |

More speed to manage. Start your deceleration earlier than you think you need to.

jc_001b — Beam Drift Stop

| Change | Value |
| --- | --- |
| Environment — Wind dir deg | 90 |

The vessel will behave completely differently. Tunnel thruster geometry means beam stopping authority differs from head/stern stopping authority. Notice which thrusters the system uses.

jc_001c — Drift Stop with Position Constraint

| Change | Value |
| --- | --- |
| Position constraint metres | 50 |

Now you have two objectives. Stopping cleanly AND where you stop matters. This begins introducing position awareness into speed management.

jc_001d — Drift, Stop and Return

| Change | Value |
| --- | --- |
| Add return phase | yes |
| Return accuracy metres | 2 |
| Final mode | DP |

This is the complete manoeuvre. Three distinct phases. Do not start the return until velocity is below 0.1kts. Do not engage DP until you are within the DP operating radius and below 0.1kts.

## Slot 4 · sen_003 — Gyro Failure Response

| Domain | Sensor and Reference System Management |
| --- | --- |
| Competency | Sensor management |
| Difficulty | 3 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

6.1

7.8

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Both gyros active. Note heading agreement between gyro 1 and gyro 2 — should be within 0.5 degrees. Vessel stable on DP.

### Execute

Without warning your DPO will introduce a 5 degree heading error on one gyro. Observe how the DP responds. Does it auto-reject? Does heading control degrade? Record the heading excursion. Manually inhibit the faulty gyro. Observe recovery. Record time from fault introduction to stable heading.

### Professional standard

- Identify faulty gyro within 30 seconds

- Correct inhibit procedure executed

- Recovery confirmed before continuing

- Time from fault to stable heading recorded

### Observe and record

| Field | Type |
| --- | --- |
| Initial gyro 1 vs gyro 2 agreement (degrees difference) | number |
| Did the DP auto-reject the faulty gyro? Yes or no? | text |
| Maximum heading excursion during fault (degrees) | number |
| Which gyro was identified as faulty and how? | text |
| Describe the inhibit procedure used | text |
| Time from fault introduction to stable heading (seconds) | number |
| Notes | text |

### Debrief questions

1. How does the DP decide which gyro to trust when two gyros disagree — what is the voting logic?

2. What heading excursion is acceptable during a gyro failure response near a structure?

3. Why is gyro failure more immediately dangerous to vessel position than a GPS failure?

4. How does heading error propagate into position error — explain the mechanism?

### Variants (1)

sen_003a — Gyro Failure During Heading Change

| Change | Value |
| --- | --- |
| Failure during heading change | yes |

Same failure introduced while the vessel is mid-turn. The combination of active heading change and gyro fault is harder to manage. Prioritise stopping the turn first.

## Slot 5 · buffer_007 — buffer

Buffer slot · 40 min — Drift stop with position constraint — stop within 50m of datum

# Day 8 — First Approach to Structure

170 min across 6 slots · target 170 min

## Slot 1 · ops_002 — 500m Zone Entry Checklist Walk-Through

| Domain | DP Operations and Operational Decision Making |
| --- | --- |
| Competency | Operational planning and execution |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

7.20

7.24

7.25

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel at 500m standoff of virtual structure. Before approaching, complete the full zone entry checklist verbally with DPO. Every item, in order, with reasoning for why each item matters.

### Execute

Work through the 500m zone entry checklist item by item. For each item state what you are checking, confirm its status, and explain why it matters before entering the 500m zone. DPO challenges any item you cannot explain. After completing the checklist verbally, execute the approach.

### Professional standard

- All checklist items covered without prompting

- Correct status confirmed for each item

- Clear reasoning given for each item

- No items skipped when challenged

### Observe and record

| Field | Type |
| --- | --- |
| Items completed without prompting | number |
| Which items required prompting? | text |
| Which items did DPO challenge? | text |
| DPO assessment of reasoning quality | text |
| Working side correctly assessed — windward or leeward? | text |
| Abort heading confirmed before approach commenced? | text |
| Notes | text |

### Debrief questions

1. Which checklist items have the highest safety significance for this specific operation?

2. What happens operationally if you enter the 500m zone without completing this checklist?

3. How does the working side assessment change your risk picture and your approach plan?

4. What SIMOPS information do you need from the installation before entering the 500m zone?

### Variants (1)

ops_002a — Zone Entry Checklist From Memory

| Change | Value |
| --- | --- |
| Checklist from memory | yes |

Complete the zone entry checklist entirely from memory — no app, no reference. This is the standard a DPO would expect on a real vessel. 14 of 16 items minimum.

## Slot 2 · approach_001 — Leeward Approach to Structure

| Domain | Approach to Structure and Close Proximity Operations |
| --- | --- |
| Competency | Approach and proximity |
| Difficulty | 3 of 5 |
| Duration | 35 min |
| Vessel modes | DP → JSMH inside 500m, DP for final positioning |

### NI task references

Reference

8.1

8.2

8.3

8.4

8.5

8.6

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Place virtual structure 500m on 037 degrees True from your current position. You are on the leeward side — the dominant environmental force is pushing you away from the structure. Confirm your abort heading before starting. Write it down.

### Execute

Enter the 500m zone on a heading of approximately 037 degrees True. Reduce to maximum 0.5kts by the 500m boundary. Reduce to maximum 0.3kts by the 200m boundary. Reduce to maximum 0.1kts by the 100m boundary. Transition from JSMH to DP at 100m. Hold at the DP operating radius for 10 minutes. Record approach track, speed at each zone boundary, and DP capture quality.

### Professional standard

- No hard exclusion zone breach

- Speed below 0.5kts at 500m boundary

- Speed below 0.3kts at 200m boundary

- Speed below 0.1kts at 100m boundary

- DP capture within operating radius

- Abort heading confirmed before entry

### Observe and record

| Field | Type |
| --- | --- |
| Abort heading planned before entry (degrees) | number |
| Windward or leeward — state which and why | text |
| Speed at 500m zone entry (kts) | number |
| Speed at 200m boundary (kts) | number |
| Speed at 100m — transition point (kts) | number |
| Distance from structure at DP capture (metres) | number |
| DP capture quality — smooth or hunting? | text |
| Position footprint during 10 minute hold | text |
| Approximate thruster demand at hold position | text |
| Hard exclusion zone breaches (target zero) | number |
| Notes | text |

### Debrief questions

1. This was a leeward approach — what is the failure mode if DP fails while you are at the operating radius?

2. At what point during the approach was the environment helping your deceleration and at what point was it not?

3. What is your abort heading from the operating radius and how long would it take to clear the 500m zone?

4. How did thruster demand change as you closed distance to the structure during the hold?

### Variants (2)

approach_001a — Leeward Approach in Heavy Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 25 |
| Wave height m | 2 |

Same approach geometry but heavier conditions. Speed management becomes more critical. The environment assists braking more strongly — use it.

approach_001b — Leeward Approach with Degraded Reference

| Change | Value |
| --- | --- |
| One reference degraded | yes |

Your DPO will degrade one reference before you enter the 500m zone. Assess whether to proceed. If you continue, how does reduced reference quality affect your approach discipline?

## Slot 3 · approach_004 — Controlled Departure from Structure

| Domain | Approach to Structure and Close Proximity Operations |
| --- | --- |
| Competency | Approach and proximity |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP then JSMH |

### NI task references

Reference

10.1

10.2

10.3

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP at operating radius of virtual structure. Moderate conditions. This task follows an approach drill — do not reset between approach and departure.

### Execute

Plan and execute a controlled departure. Select JSMH and move to the 500m boundary at appropriate speed — no more than 0.5kts. Once clear of 500m zone, return to open water DP. Complete the departure checklist verbally with your DPO. Record the mode transition point and speed management throughout.

### Professional standard

- Speed below 0.5kts throughout departure

- Mode change to JSMH executed at appropriate time

- 500m zone cleared cleanly

- Departure checklist completed verbally

- Navigation equipment status confirmed on clearing zone

### Observe and record

| Field | Type |
| --- | --- |
| Distance from structure at mode transition to JSMH (metres) | number |
| Maximum speed during departure (kts) | number |
| Time from DP off to 500m zone clear (seconds) | number |
| Departure checklist completed — any items missed? | text |
| Was this a drift-on or drift-off departure situation? | text |
| Notes | text |

### Debrief questions

1. Why must you manoeuvre well clear of the structure before changing mode during departure?

2. The NI checklist states 1.5 to 2.5 ship lengths before mode change depending on drift-on or drift-off — which applied here and why?

3. How does the drift-on vs drift-off situation affect the urgency and direction of your departure?

4. What navigation equipment must be restored to correct status once clear of the 500m zone?

### Variants (1)

approach_004a — Emergency Departure

| Change | Value |
| --- | --- |
| Emergency trigger | yes |

Your DPO will trigger an emergency at the operating radius without warning — DP failure, thruster fault, or consequence alarm. You must execute an immediate departure. No planning time.

## Slot 4 · act_003 — Personal ASOG Quick-Reference Build

| Domain | ASOG, CAM & TAM Decision Support |
| --- | --- |
| Competency | Decision support application |
| Difficulty | 2 of 5 |
| Duration | 35 min |
| Task type | Applied exercise |
| Vessel modes | DP → DP — sustained hold while working scenarios |

### NI task references

Reference

7.20

7.21

7.24

### Setup

TDPO has access to the vessel's full ASOG document for the operational area currently relevant (e.g. 500m zone entry near a fixed structure).

### Execute

TDPO condenses the ASOG's Green/Blue/Yellow/Red criteria for the systems most relevant to the current activity into a single-page quick-reference card, in their own words. DPO or supervisor reviews it for accuracy and gaps.

### Professional standard

- Quick-reference is accurate against the source document, not just plausible-sounding

- Covers the specific systems relevant to the current operation, not a generic copy of the whole ASOG

- Correctly separates 'what triggers the status change' from 'what the response action is'

- TDPO can explain any abbreviation or threshold used on their own card without referring back to the source

### Observe and record

| Field | Type |
| --- | --- |
| Systems covered on the card | text |
| Number of corrections needed on review | number |
| Notes | text |

### Debrief questions

1. What did you leave off the card, and was that a deliberate scoping decision or an oversight?

2. If a new crew member picked up your card cold, would they be able to use it correctly, or does it rely on knowledge you already had in your head?

3. Which threshold on the card surprised you most once you looked at the actual numbers — thruster loading, heading excursion, position excursion?

## Slot 5 · er_003 — DP Event Investigation, Classification and Report

| Domain | Emergency Response and DP Event Reporting |
| --- | --- |
| Competency | Event classification and reporting |
| Difficulty | 2 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

1.3.5

1.3.3

11.4

### Environment

Wind 15 kt @ 090° · Sea 1 m @ 100° · Current 0.3 kt @ 045°

### Setup

Arrange with the supervising DPO for one degradation to be injected at some point during a 15-minute hold — a reference deselection, a sensor deviation, a thruster deselection — without telling you the time or the nature. If working alone, use the simulator's scenario or failure injection function on a delay. Have the three IMCA station keeping event category definitions written in front of you: DP incident (total loss of DP capability), DP undesired event (loss of redundancy and/or compromised capability), DP observation (neither, but worth sharing). Start a written timeline with the session start time as entry one.

### Execute

Hold position. When the event occurs, respond to it correctly and log your actions with times as you go. Once the vessel is stable, reconstruct the full event timeline from the DP alarm and event log, compare it against your own handwritten timeline, classify the event into one of the three IMCA categories, identify the initiating event as distinct from its consequences, and complete a draft station keeping event report containing: timeline, initiating event, consequences, response actions taken, and classification with one sentence of justification.

### Professional standard

- Handwritten timeline captures the event onset within 30 seconds of the system log time

- First correct response action taken before turning to documentation — vessel first, paperwork second

- Initiating event and consequences separated correctly in the report — the alarm you saw first is often not the initiating event

- Classification defensible against the written definitions, stated in one sentence

- Report complete enough that a DPO who was not present could reconstruct the event from it alone

### Observe and record

| Field | Type |
| --- | --- |
| Event onset per system log (time) | text |
| Event onset per your timeline (time) | text |
| Time from onset to first correct response action (seconds) | number |
| Timeline entries in your record (count) | number |
| Relevant entries in system log (count) | number |
| Classification (incident / undesired event / observation) | text |
| Initiating event identified | text |
| Notes | text |

### Debrief questions

1. Walk through why you chose the classification you did — and what single change to the scenario would have pushed it into the category above or below?

2. What is the difference between the initiating event and the first alarm you noticed, and why do reports that confuse the two mislead the industry?

3. Why does the IMCA scheme publish events anonymously, and what reporting behaviours does that anonymity exist to protect against?

4. How does an initiating event differ from a root cause, and which one does your report identify?

5. On your vessel, who does a station keeping event report go to, and what should happen to it after that?

### Variants (2)

er_003a — Compound Event Report

| Change | Value |
| --- | --- |
| Injected events | 2 |

Two degradations, close together. One timeline, one report, one classification for the combined event. Deciding whether it is one event or two is part of the task.

er_003b — Report a Clean Session

| Change | Value |
| --- | --- |
| Injected events | 0 |
| Report type | DP observation |

Nothing is injected, but something in any session is worth sharing — a reference wander, a load spike, a near-limit excursion. Find it, document it, and justify it as a DP observation. Learning to see reportable material in a normal watch is the point.

## Slot 6 · ds_003 — Capability Plot vs Reality Cross-Check

| Domain | DP System Behaviour, Modes and Performance Monitoring |
| --- | --- |
| Competency | Critical use of decision support tools |
| Difficulty | 2 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

7.21

7.17

9.15

### Setup

With the vessel settled on DP, open the live DP capability plot for the present conditions.

### Execute

Record what the plotted capability envelope shows for the current heading and wind. Separately, read the actual thrust percentage being used by each thruster from the propulsion readout. Compare the two.

### Professional standard

- Recorded actual per-thruster load, not just an overall average

- Correctly identified whether the plotted envelope and actual load were consistent

- Could explain what the capability plot does NOT account for

- Knew the difference between Live and Theoretical modes without being told

### Observe and record

| Field | Type |
| --- | --- |
| Headings shown as marginal/at-risk on the plot | text |
| Actual thrust % used, each thruster | text |
| Any generator or thruster offline? (Y/N, which) | text |
| Notes | text |

### Debrief questions

1. Capability plots always assume full power is available. If a generator were offline right now, would the plot in front of you reflect that? What could go wrong if a DPO trusted the plot without checking against actual thrust loads?

2. How would you independently verify whether today's capability plot is trustworthy, using only the real thrust readings on your desk?

3. The plot has Live and Theoretical modes. When would a DPO deliberately switch to Theoretical during passive time rather than just reading Live?

4. If your actual thrust usage on the critical heading is already close to the plotted limit in benign weather, what does that tell you about your reserve capacity if weather deteriorates?

# Day 9 — Windward Approach and Environmental Awareness

125 min across 4 slots · target 125 min

## Slot 1 · approach_002 — Windward Approach to Structure

| Domain | Approach to Structure and Close Proximity Operations |
| --- | --- |
| Competency | Approach and proximity |
| Difficulty | 4 of 5 |
| Duration | 35 min |
| Vessel modes | DP → JSMH inside 500m, DP for final positioning |

### NI task references

Reference

8.1

8.2

8.3

8.4

8.5

8.6

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Place virtual structure 500m on 217 degrees True from your current position. You are now on the windward side — the dominant environmental force is pushing you toward the structure. Confirm your abort heading before starting. This is more complex on the windward side — think carefully before writing it down.

### Execute

Same speed profile as approach_001. Enter at 217 degrees True, reduce speed at each zone boundary, transition to DP at 100m, hold for 10 minutes. Pay particular attention to how the environment affects your closure rate and thruster demand compared to the leeward approach.

### Professional standard

- No hard exclusion zone breach

- All speed limits observed at each boundary

- Abort heading correctly identified for windward side

- Recognition of increased risk compared to leeward approach

- Higher thruster demand noted and recorded

### Observe and record

| Field | Type |
| --- | --- |
| Abort heading planned before entry (degrees) | number |
| Why did you choose that abort heading on the windward side? | text |
| Speed at 500m zone entry (kts) | number |
| Speed at 200m boundary (kts) | number |
| Speed at 100m — transition point (kts) | number |
| How did thruster demand compare to leeward approach? | text |
| Was closure rate harder to control than leeward? Why? | text |
| Distance from structure at DP capture (metres) | number |
| Hard exclusion zone breaches (target zero) | number |
| Your risk assessment comparison — windward vs leeward | text |
| Notes | text |

### Debrief questions

1. What is the failure mode on the windward side compared to the leeward side — why is windward higher risk?

2. How much higher was thruster demand during the windward approach and hold compared to leeward?

3. Why is the abort heading more complex to plan on the windward side?

4. Under what operational circumstances would you accept a windward approach and what additional safeguards would you put in place?

### Variants (1)

approach_002a — Windward Approach in Heavy Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 25 |
| Wave height m | 2 |

Heavy conditions on the windward side. Closure rate will be hard to control. Start your deceleration significantly earlier than you think necessary.

## Slot 2 · env_005 — Model Corruption and Recovery

| Domain | Environmental Awareness and Model Behaviour |
| --- | --- |
| Competency | Model behaviour |
| Difficulty | 3 of 5 |
| Duration | 25 min |
| Vessel modes | DP → JSMH then DP |

### NI task references

Reference

7.16

7.17

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Moderate environment set and settled. Vessel on DP at datum. Note current vector value before starting.

### Execute

Run 1 — Corrupted transition: Make an aggressive 300m JSMH transit with large inputs and abrupt stops. Immediately select DP. Record thruster behaviour and current vector for 2 minutes. Note time to settle. Run 2 — Clean transition: Reset. Make the same 300m transit but slow to near zero before selecting Monitor briefly, then DP. Record settle time and capture quality. Compare both runs.

### Professional standard

- Observe measurable difference in capture quality between runs

- Correctly identify current vector corruption after aggressive transit

- Explain the mechanism of model corruption

- Record settle time difference between methods

### Observe and record

| Field | Type |
| --- | --- |
| Current vector before transit (kts and direction) | text |
| Run 1 — Current vector immediately after aggressive transit | text |
| Run 1 — Describe thruster behaviour first 30 seconds on DP | text |
| Run 1 — Time to stable position (seconds) | number |
| Run 2 — Current vector after clean transition | text |
| Run 2 — Describe thruster behaviour first 30 seconds on DP | text |
| Run 2 — Time to stable position (seconds) | number |
| Difference in settle time between runs (seconds) | number |
| Notes | text |

### Debrief questions

1. What did the current vector show immediately after the aggressive transit and what caused that reading?

2. Why does slowing to near zero before the Monitor transition improve DP capture quality?

3. What does the Kalman filter inherit from a large JSMH transit — and why is that a problem?

4. How long did the model take to settle in each method — and what explains the difference in time?

### Variants (1)

env_005a — Model Corruption — Large Transit

| Change | Value |
| --- | --- |
| Transit distance metres | 1000 |

A much larger transit produces much greater corruption. This simulates moving between rigs. Notice how much longer the model takes to settle.

## Slot 3 · watch_001 — Sustained Position Keeping Observation

| Domain | Watchkeeping and Bridge Team Procedures |
| --- | --- |
| Competency | Watchkeeping |
| Difficulty | 2 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP — observe only unless safety issue |

### NI task references

Reference

9.1

9.2

9.3

9.4

9.5

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

| Setting | Value |
| --- | --- |
| Ramp to — Wind speed kts | 25 |
| Ramp to — Ramp rate | gradual |

### Setup

Vessel on DP in moderate conditions. DPO will gradually increase environmental conditions during the observation period. Your role is monitor and assess — do not intervene unless a genuine safety issue develops.

### Execute

Monitor the DP system holding position for 30 minutes while conditions increase. Record position footprint, thruster demand, reference quality, and current vector at 5-minute intervals. Identify any trends. Assess at what point intervention would be required. At each 5-minute mark write your observation before looking at the previous entry — do not let previous readings anchor your assessment.

### Professional standard

- All parameters recorded at every 5-minute interval without prompting

- Trends correctly identified from data

- Correct intervention threshold identified with reasoning

- Assessment does not simply mirror the previous reading

### Observe and record

| Field | Type |
| --- | --- |
| 0 min — position footprint, thruster demand, reference quality, current vector | text |
| 5 min — position footprint, thruster demand, reference quality, current vector | text |
| 10 min — position footprint, thruster demand, reference quality, current vector | text |
| 15 min — position footprint, thruster demand, reference quality, current vector | text |
| 20 min — position footprint, thruster demand, reference quality, current vector | text |
| 25 min — position footprint, thruster demand, reference quality, current vector | text |
| 30 min — position footprint, thruster demand, reference quality, current vector | text |
| Trends identified across the 30 minute period | text |
| At what point would you have recommended intervention and why? | text |
| Notes | text |

### Debrief questions

1. What trends did you identify across the 30-minute period — were conditions genuinely deteriorating or just varying?

2. At what point would you have recommended intervention and what specific evidence supported that recommendation?

3. What is the difference between active watchkeeping and passive observation — are you doing one or both?

4. What DP log entries would you have made during this 30-minute period?

### Variants (1)

watch_001a — Sustained Observation Near Structure

| Change | Value |
| --- | --- |
| Virtual structure active | yes |
| Standoff metres | 100 |

Same observation but with a virtual structure at 100m. Degrading conditions now have a direct safety consequence. Your intervention threshold assessment is no longer theoretical.

## Slot 4 · drill_002 — Loss of Seawater Cooling Pump Redundancy

| Domain | DP Emergency Drills & Preparedness |
| --- | --- |
| Competency | Failure response procedural |
| Difficulty | 2 of 5 |
| Duration | 35 min |
| Task type | Live simulator drill |
| Vessel modes | DP → DP |

### NI task references

Reference

11.3

11.12

4.4

### Setup

Vessel in full auto DP, power plant configured per the vessel's DP operations manual for the current DP class. Simulated location and activity agreed and stated aloud before starting.

### Execute

Supervising DPO simulates loss of a duty seawater pump. TDPO observes the standby pump start and confirms alarms and equipment temperatures remain within normal limits. Test is then repeated with the standby pump isolated, so the TDPO can observe what happens when the redundancy itself is unavailable.

### Professional standard

- Correctly distinguishes the two scenarios: standby available vs standby isolated

- Notices and reports any DP equipment temperature trend, not just the immediate alarm

- Recognises which system components are on which side of the redundancy split

- Communicates the fault and status clearly rather than working silently through it

### Observe and record

| Field | Type |
| --- | --- |
| Standby pump was available? | boolean |
| Alarms generated | text |
| Equipment temperature trend observed | text |
| Time to recognise the fault (seconds) | number |
| Notes | text |

### Debrief questions

1. What risks arise from 'multi-tasking' during DP operations that could delay noticing a scenario like this — radio traffic, deck operations, routine paperwork?

2. Following this drill, would your response change if you faced the same fault for real, given what you now know about the vessel's reaction?

3. How is the seawater system's segregation related to the vessel's redundancy groups, and how would you check that on your own vessel's diagrams?

4. What would you look for on the operator station specifically to distinguish 'duty pump failed, standby running fine' from 'both pumps degraded'?

# Day 10 — Extended Hold and Alarm Introduction

160 min across 6 slots · target 160 min

## Slot 1 · approach_003 — Extended Hold at Operating Radius

| Domain | Approach to Structure and Close Proximity Operations |
| --- | --- |
| Competency | Approach and proximity |
| Difficulty | 3 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

8.5

8.6

9.1

9.2

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

| Setting | Value |
| --- | --- |
| Ramp to — Wind speed kts | 30 |
| Ramp to — Ramp rate | gradual |

### Setup

Position vessel at DP operating radius of virtual structure. Moderate conditions initially. DPO will ramp wind gradually during the hold.

### Execute

Hold position at the DP operating radius for 20 minutes. During the hold your DPO will gradually increase wind from 15kts to 30kts. Record position footprint at 15, 20, 25, and 30kts. Record thruster demand at each interval. Identify the point at which you would recommend moving further from structure or reviewing operations with the OIM.

### Professional standard

- Position maintained throughout hold

- Correct identification of degrading margin

- Reasoned recommendation point with evidence

- Correct communication chain identified

### Observe and record

| Field | Type |
| --- | --- |
| Position footprint at 15kts wind | text |
| Thruster demand at 15kts (percent) | number |
| Position footprint at 20kts wind | text |
| Thruster demand at 20kts (percent) | number |
| Position footprint at 25kts wind | text |
| Thruster demand at 25kts (percent) | number |
| Position footprint at 30kts wind | text |
| Thruster demand at 30kts (percent) | number |
| Wind speed at which you would recommend reviewing operations (kts) | number |
| Reasoning for your recommendation | text |
| Who would you inform and in what order? | text |
| Notes | text |

### Debrief questions

1. At what wind speed did your position footprint become unacceptable at this operating radius?

2. What is the ASOG concept and how would a properly defined ASOG have defined your action limit here?

3. Who do you inform and in what order when you identify a deteriorating situation during a hold?

4. What is the difference between moving further from the structure and going fully off DP — when would you choose each?

### Variants (1)

approach_003a — Hold with Thruster Failure During Ramp

| Change | Value |
| --- | --- |
| Thruster failure during hold | yes |

Your DPO will fail one thruster at an unannounced point during the environmental ramp. You now have two simultaneous pressures — increasing weather and reduced thruster complement. Assess and decide.

## Slot 2 · approach_004 — Controlled Departure from Structure

| Domain | Approach to Structure and Close Proximity Operations |
| --- | --- |
| Competency | Approach and proximity |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP then JSMH |

### NI task references

Reference

10.1

10.2

10.3

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP at operating radius of virtual structure. Moderate conditions. This task follows an approach drill — do not reset between approach and departure.

### Execute

Plan and execute a controlled departure. Select JSMH and move to the 500m boundary at appropriate speed — no more than 0.5kts. Once clear of 500m zone, return to open water DP. Complete the departure checklist verbally with your DPO. Record the mode transition point and speed management throughout.

### Professional standard

- Speed below 0.5kts throughout departure

- Mode change to JSMH executed at appropriate time

- 500m zone cleared cleanly

- Departure checklist completed verbally

- Navigation equipment status confirmed on clearing zone

### Observe and record

| Field | Type |
| --- | --- |
| Distance from structure at mode transition to JSMH (metres) | number |
| Maximum speed during departure (kts) | number |
| Time from DP off to 500m zone clear (seconds) | number |
| Departure checklist completed — any items missed? | text |
| Was this a drift-on or drift-off departure situation? | text |
| Notes | text |

### Debrief questions

1. Why must you manoeuvre well clear of the structure before changing mode during departure?

2. The NI checklist states 1.5 to 2.5 ship lengths before mode change depending on drift-on or drift-off — which applied here and why?

3. How does the drift-on vs drift-off situation affect the urgency and direction of your departure?

4. What navigation equipment must be restored to correct status once clear of the 500m zone?

### Variants (1)

approach_004a — Emergency Departure

| Change | Value |
| --- | --- |
| Emergency trigger | yes |

Your DPO will trigger an emergency at the operating radius without warning — DP failure, thruster fault, or consequence alarm. You must execute an immediate departure. No planning time.

## Slot 3 · alarm_001 — Position Deviation Alarm Response

| Domain | DP Alarms, Warnings and Emergency Response |
| --- | --- |
| Competency | Alarm recognition and response |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

11.1

11.2

11.4

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP in moderate conditions. DPO will trigger a position deviation alarm without warning at an unannounced time. Do not anticipate — watch the display normally.

### Execute

When the alarm sounds, respond correctly. Acknowledge the alarm. Identify the cause — is it environmental, reference, or thruster related? Take appropriate action. Record your response time and the steps you took. DPO repeats with a different cause. Minimum two runs with different causes.

### Professional standard

- Alarm acknowledged within 15 seconds

- Cause identified within 60 seconds

- Correct action taken for the identified cause

- Correct escalation if required

- Time from alarm to acknowledgement recorded honestly

### Observe and record

Run 1

| Field | Type |
| --- | --- |
| Time from alarm to acknowledgement (seconds) | number |
| Cause identified — environmental, reference, or thruster? | text |
| Time to identify cause (seconds) | number |
| Action taken | text |
| Was escalation required? Who was informed? | text |
| DPO assessment — was response correct? | text |
| Notes | text |

Run 2

| Field | Type |
| --- | --- |
| Time from alarm to acknowledgement (seconds) | number |
| Cause identified — environmental, reference, or thruster? | text |
| Time to identify cause (seconds) | number |
| Action taken | text |
| Was escalation required? Who was informed? | text |
| DPO assessment — was response correct? | text |
| Notes | text |

### Debrief questions

1. What is the first thing you check when a position deviation alarm sounds — before anything else?

2. How do you distinguish between an environmental cause and a reference cause from the DP display alone?

3. At what point does a position alert become a position action alarm and what does that change?

4. Who needs to be informed of a position action alarm and in what order?

### Variants (1)

alarm_001a — Position Deviation Near Structure

| Change | Value |
| --- | --- |
| Virtual structure active | yes |
| Standoff metres | 100 |

Same alarm but you are at the operating radius of a virtual structure. The urgency is higher. Your response time target is reduced to 10 seconds acknowledgement and 30 seconds cause identification.

## Slot 4 · er_004 — Emergency Procedure Touch Drill Circuit

| Domain | Emergency Response and DP Event Reporting |
| --- | --- |
| Competency | Touch drill procedure recall |
| Difficulty | 2 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

11.13

11.4

7.19

1.3.2

### Environment

Wind 15 kt @ 045° · Sea 1 m @ 050° · Current 0.3 kt @ 090°

### Setup

Vessel settled in DP at datum and holding throughout — the touch drill runs over a live station keeping picture, not a frozen screen. Select three emergency procedures from your vessel's DP operations manual (for example: loss of a position reference, loss of a thruster, initiation of the drive off response). For each, have the written procedure available but face down. A touch drill means physically touching or pointing to each control, page, or indicator in sequence — never actuating anything. The simulator state must be identical at the end to the start.

### Execute

Round one: for each of the three procedures in turn, execute the full response as a touch drill from memory — touch each control in order, say each step aloud, and time yourself. Then turn the written procedure over and mark every step you missed, added, or performed out of sequence. Round two: repeat all three drills. Record time and error count per drill per round. Confirm at the end that nothing was actuated and the DP picture is unchanged.

### Professional standard

- Every step physically located — touched or pointed to — not just named from the chair

- Nothing actuated at any point; vessel position, heading and system state identical before and after

- Round two error count lower than round one on every drill

- Round two times faster than round one without steps being skipped to achieve it

- Missed steps identified honestly against the written procedure, not rationalised away

### Observe and record

| Field | Type |
| --- | --- |
| The three procedures drilled | text |
| Round 1 time per drill (seconds, x3) | text |
| Round 1 errors per drill (missed/added/out of sequence, x3) | text |
| Round 2 time per drill (seconds, x3) | text |
| Round 2 errors per drill (x3) | text |
| Steps most commonly missed across all drills | text |
| Notes | text |

### Debrief questions

1. Which steps did you miss in round one, and what do they have in common — are they the steps you understand least, or the ones that feel too obvious to rehearse?

2. Why does physically touching each control build a different kind of recall than reading the procedure or reciting it?

3. In a real emergency, which of your three procedures gives you the least time to think, and does your round two time for it reflect that?

4. What is the risk of touch drilling a procedure incorrectly and never checking against the written version — and what does that say about how this task should always be run?

5. Which procedure on your vessel would you touch drill next, and why that one?

### Variants (2)

er_004a — Cold Touch Drill

| Change | Value |
| --- | --- |
| Trigger | Supervising DPO names one of the three procedures at an unannounced moment during a routine hold |

No warm-up, no choosing the order. One procedure, called cold, timed. This is closer to how the real thing arrives.

er_004b — Cross-Station Touch Drill

| Change | Value |
| --- | --- |
| Station | Perform the same three drills at the backup DP station |

Same procedures, different desk. Notice which parts of your recall were tied to muscle memory of one console layout rather than to the procedure itself. Pairs naturally with er_001.

## Slot 5 · dc_001 — Pre-Drill Briefing and Expected Behaviour

| Domain | Drill Conduct, Briefing and Debrief |
| --- | --- |
| Competency | Drill briefing and behaviour prediction |
| Difficulty | 2 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

1.3.4

2.3

7.25

11.9

### Environment

Wind 18 kt @ 120° · Sea 1.2 m @ 130° · Current 0.4 kt @ 090°

### Setup

Choose a single failure you have already practiced — one position reference loss is the baseline. Write a briefing covering, in order: drill type and boundaries; purpose and the failure mode under test; current system status; the scenario step by step, stating what is done for real and what is mimicked; hazards; roles, communications, and who may stop the drill; expected behaviour — the alarms you predict, what the DP system should do, and how long stabilisation should take; and a two-to-three point summary. Deliver the briefing aloud and time it. Target five to ten minutes.

### Execute

Run the failure exactly as briefed. Respond, recover, restore. Then score your briefing against reality: how many of your predicted alarms appeared, whether the system behaved as you said it would, and whether recovery took the time you estimated.

### Professional standard

- Briefing delivered in full before any action, inside five to ten minutes

- All eight briefing sections addressed

- Expected behaviour stated in specific, checkable terms before execution

- Every mismatch between briefed and actual behaviour noted in the record

### Observe and record

| Field | Type |
| --- | --- |
| Briefing duration (minutes) | number |
| Briefing sections covered (of 8) | number |
| Alarms predicted | number |
| Alarms observed | number |
| Predictions confirmed | number |
| Time to recovery (seconds) | number |
| Notes | text |

### Debrief questions

1. Why does a briefing spell out which actions are real and which are mimic-only, and what kind of incident does that distinction prevent?

2. What did predicting the alarms in advance force you to know that simply reacting to them would not?

3. Why must the authority to stop a drill be stated aloud before it starts?

4. Which of your predictions missed, and what does the miss tell you about your mental model of the system?

### Variants (2)

dc_001a — Thruster Deselection Briefing

| Change | Value |
| --- | --- |
| Failure mode | single thruster deselection |

Thrust reallocation is harder to predict than a reference failure. Your expected-behaviour section should name which thrusters pick up the load.

dc_001b — Brief Another Trainee

| Change | Value |
| --- | --- |
| Delivery | briefing delivered to a second person who then runs the drill while you observe |

If your briefing was clear, their response will show it. If it wasn't, that will show too. Requires a second person at the desk.

## Slot 6 · prs_002 — Working Near a Fixed Structure

| Domain | Position Reference Systems, Redundancy and Failure Awareness |
| --- | --- |
| Competency | Environmental and prs degradation awareness |
| Difficulty | 3 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

5.1

6.3

8.1

### Environment

Wind 18 kt @ 300° · Sea 1.2 m @ 300°

### Setup

Position the vessel at moderate range from a virtual platform structure, with DGPS enabled as one of the active PRS and the vessel positioned downwind of the structure.

### Execute

Close on the structure per the exercise plan while watching DGPS position quality/scatter. Note any degradation as range decreases, and compare your wind sensor reading against the wind you'd expect given the logged environment.

### Professional standard

- Detected DGPS degradation from the data, not just from an alarm

- Recognised the wind sensor reading as suspect before it caused a visible position problem

- Could state which PRS types would NOT be expected to degrade in this scenario

- Maintained a direct visual reference to the structure throughout, not just instrument watch

### Observe and record

| Field | Type |
| --- | --- |
| DGPS scatter, far from structure (m) | number |
| DGPS scatter, close to structure (m) | number |
| Wind sensor reading vs expected true wind | text |
| Notes | text |

### Debrief questions

1. Why does a large fixed structure nearby specifically degrade DGPS, while an acoustic or laser-based PRS might be unaffected by the same proximity?

2. Working close-in on the downwind side of a platform, your wind sensor may be actively wind-shadowed rather than simply unreliable. What's the practical difference between those two failure types for a DPO trying to trust the reading?

3. What visual cues should tell you a positioning problem is developing near a structure before any instrument alarm fires?

4. If your only degraded PRS near the structure is DGPS, and you still have two other independent references online, should that change how concerned you are? Why or why not?

# Day 11 — Sensor Failures and Watch Handover

120 min across 4 slots · target 120 min

## Slot 1 · sen_003 — Gyro Failure Response

| Domain | Sensor and Reference System Management |
| --- | --- |
| Competency | Sensor management |
| Difficulty | 3 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

6.1

7.8

11.9

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Both gyros active. Note heading agreement between gyro 1 and gyro 2 — should be within 0.5 degrees. Vessel stable on DP.

### Execute

Without warning your DPO will introduce a 5 degree heading error on one gyro. Observe how the DP responds. Does it auto-reject? Does heading control degrade? Record the heading excursion. Manually inhibit the faulty gyro. Observe recovery. Record time from fault introduction to stable heading.

### Professional standard

- Identify faulty gyro within 30 seconds

- Correct inhibit procedure executed

- Recovery confirmed before continuing

- Time from fault to stable heading recorded

### Observe and record

| Field | Type |
| --- | --- |
| Initial gyro 1 vs gyro 2 agreement (degrees difference) | number |
| Did the DP auto-reject the faulty gyro? Yes or no? | text |
| Maximum heading excursion during fault (degrees) | number |
| Which gyro was identified as faulty and how? | text |
| Describe the inhibit procedure used | text |
| Time from fault introduction to stable heading (seconds) | number |
| Notes | text |

### Debrief questions

1. How does the DP decide which gyro to trust when two gyros disagree — what is the voting logic?

2. What heading excursion is acceptable during a gyro failure response near a structure?

3. Why is gyro failure more immediately dangerous to vessel position than a GPS failure?

4. How does heading error propagate into position error — explain the mechanism?

### Variants (1)

sen_003a — Gyro Failure During Heading Change

| Change | Value |
| --- | --- |
| Failure during heading change | yes |

Same failure introduced while the vessel is mid-turn. The combination of active heading change and gyro fault is harder to manage. Prioritise stopping the turn first.

## Slot 2 · watch_001 — Sustained Position Keeping Observation

| Domain | Watchkeeping and Bridge Team Procedures |
| --- | --- |
| Competency | Watchkeeping |
| Difficulty | 2 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP — observe only unless safety issue |

### NI task references

Reference

9.1

9.2

9.7

9.8

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

| Setting | Value |
| --- | --- |
| Ramp to — Wind speed kts | 25 |
| Ramp to — Ramp rate | gradual |

### Setup

Vessel on DP in moderate conditions. DPO will gradually increase environmental conditions during the observation period. Your role is monitor and assess — do not intervene unless a genuine safety issue develops.

### Execute

Monitor the DP system holding position for 30 minutes while conditions increase. Record position footprint, thruster demand, reference quality, and current vector at 5-minute intervals. Identify any trends. Assess at what point intervention would be required. At each 5-minute mark write your observation before looking at the previous entry — do not let previous readings anchor your assessment.

### Professional standard

- All parameters recorded at every 5-minute interval without prompting

- Trends correctly identified from data

- Correct intervention threshold identified with reasoning

- Assessment does not simply mirror the previous reading

### Observe and record

| Field | Type |
| --- | --- |
| 0 min — position footprint, thruster demand, reference quality, current vector | text |
| 5 min — position footprint, thruster demand, reference quality, current vector | text |
| 10 min — position footprint, thruster demand, reference quality, current vector | text |
| 15 min — position footprint, thruster demand, reference quality, current vector | text |
| 20 min — position footprint, thruster demand, reference quality, current vector | text |
| 25 min — position footprint, thruster demand, reference quality, current vector | text |
| 30 min — position footprint, thruster demand, reference quality, current vector | text |
| Trends identified across the 30 minute period | text |
| At what point would you have recommended intervention and why? | text |
| Notes | text |

### Debrief questions

1. What trends did you identify across the 30-minute period — were conditions genuinely deteriorating or just varying?

2. At what point would you have recommended intervention and what specific evidence supported that recommendation?

3. What is the difference between active watchkeeping and passive observation — are you doing one or both?

4. What DP log entries would you have made during this 30-minute period?

### Variants (1)

watch_001a — Sustained Observation Near Structure

| Change | Value |
| --- | --- |
| Virtual structure active | yes |
| Standoff metres | 100 |

Same observation but with a virtual structure at 100m. Degrading conditions now have a direct safety consequence. Your intervention threshold assessment is no longer theoretical.

## Slot 3 · watch_002 — DP Watch Handover

| Domain | Watchkeeping and Bridge Team Procedures |
| --- | --- |
| Competency | Watch handover |
| Difficulty | 3 of 5 |
| Duration | 25 min |
| Vessel modes | DP → DP |

### NI task references

Reference

2.5

2.6

2.7

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP in moderate conditions with a virtual structure active at 100m. A realistic handover scenario — the incoming DPO needs everything.

### Execute

Part 1 — Give handover: You are the outgoing DPO. Brief your DPO on full system status covering every checklist item. Vessel position and heading, mode and setpoint, reference systems and quality, sensor status, thruster configuration, power configuration, environmental conditions, nearby structures and working side, alarms active or recently cleared, any system degradation, operational requirements for the next watch. Part 2 — Receive handover: Swap roles. Receive a handover from your DPO. Ask all questions you would ask as an incoming DPO. Identify any gaps in what you were told.

### Professional standard

- All handover items covered without prompting

- Minimum 5 questions asked as incoming DPO

- Gaps in received handover correctly identified

- Refusal to accept an incomplete handover demonstrated

### Observe and record

| Field | Type |
| --- | --- |
| Items covered in handover you gave — list them | text |
| Items DPO identified as missed in your handover | text |
| Questions you asked when receiving handover | text |
| Gaps identified in the handover you received | text |
| Would you have accepted the watch based on the handover you received? Why or why not? | text |
| Notes | text |

### Debrief questions

1. What items did you miss in the handover you gave — and are those items safety-critical?

2. Under what circumstances would you refuse to accept a watch handover?

3. What is the single most important piece of information in a DP watch handover?

4. How does a DP watch handover differ from a standard bridge watchkeeping handover?

### Variants (1)

watch_002a — Handover with Degraded System

| Change | Value |
| --- | --- |
| System degradation active | yes |

DPO will introduce a system degradation before the handover — one reference inhibited, one thruster on standby, an alarm cleared but not resolved. You must identify all degradations in the handover you receive and decide whether to accept.

## Slot 4 · buffer_011 — buffer

Buffer slot · 45 min — Repeat sen_002 near structure variant

# Day 12 — Alarms and Single Failures

165 min across 7 slots · target 165 min

## Slot 1 · alarm_002 — Reference System Rejection Response

| Domain | DP Alarms, Warnings and Emergency Response |
| --- | --- |
| Competency | Alarm recognition and response |
| Difficulty | 3 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

11.2

11.3

11.9

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Two DGPS references active. Vessel stable on DP. DPO will trigger reference rejection without warning.

### Execute

Run 1 — Single rejection: DPO triggers auto-rejection of one reference. Respond correctly. Assess remaining configuration. Make go or no-go decision. Run 2 — Compound: After restoring Run 1, DPO degrades a second reference shortly after restoring the first. Assess minimum reference threshold and make go or no-go decision with reasoning.

### Professional standard

- Alarm acknowledged within 15 seconds

- Correct minimum reference assessment

- Correct go or no-go decision with clear reasoning

- Correct re-acceptance procedure

- Appropriate escalation

### Observe and record

Run 1

| Field | Type |
| --- | --- |
| Time to acknowledge alarm (seconds) | number |
| Reference configuration after rejection | text |
| Is minimum reference configuration met? Yes or no and why? | text |
| Decision — continue or go off DP? | text |
| Reasoning for decision | text |
| Re-acceptance procedure used | text |
| Notes | text |

Run 2

| Field | Type |
| --- | --- |
| Time to acknowledge alarm (seconds) | number |
| Reference configuration after rejection | text |
| Is minimum reference configuration met? Yes or no and why? | text |
| Decision — continue or go off DP? | text |
| Reasoning for decision | text |
| Re-acceptance procedure used | text |
| Notes | text |

### Debrief questions

1. What is the difference between auto-rejection and manual inhibit — and does it matter which way a reference is removed from the solution?

2. What is the minimum reference configuration for your vessel's operations and where is this defined?

3. When re-enabling a recovered reference why must you verify it is healthy before accepting it — what is the risk of premature acceptance?

4. How does being near a structure change the minimum reference requirement compared to open water operations?

### Variants (1)

alarm_002a — Reference Rejection During Approach

| Change | Value |
| --- | --- |
| Failure during approach | yes |

Reference rejected while you are inside the 500m zone at 150m from structure. Do you hold position on remaining references or initiate departure? This is a genuine operational decision.

## Slot 2 · alarm_003 — Thruster Fault Response

| Domain | DP Alarms, Warnings and Emergency Response |
| --- | --- |
| Competency | Alarm recognition and response |
| Difficulty | 3 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

11.3

11.4

11.10

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

All thrusters active. Vessel on DP in moderate conditions. DPO will fault one thruster without warning.

### Execute

When thruster fault alarm sounds, identify which thruster has faulted. Assess your remaining thrust capability. Assess the new worst case failure given the reduced thruster complement. Inform the appropriate parties. DPO asks: can you continue operations? Give a reasoned answer. Repeat with a different thruster faulted.

### Professional standard

- Correct thruster identified within 30 seconds

- Correct capability assessment

- Correct new worst case failure identified

- Appropriate parties informed in correct order

- Reasoned continue or abort decision

### Observe and record

Run 1

| Field | Type |
| --- | --- |
| Which thruster faulted? | text |
| Time to identify faulted thruster (seconds) | number |
| Assessment of remaining thrust capability | text |
| New worst case failure with this thruster lost | text |
| Who was informed and in what order? | text |
| Decision — continue or abort? Reasoning? | text |
| Notes | text |

Run 2

| Field | Type |
| --- | --- |
| Which thruster faulted? | text |
| Time to identify faulted thruster (seconds) | number |
| Assessment of remaining thrust capability | text |
| New worst case failure with this thruster lost | text |
| Who was informed and in what order? | text |
| Decision — continue or abort? Reasoning? | text |
| Notes | text |

### Debrief questions

1. How does losing this specific thruster change your worst case failure scenario?

2. Which direction is position keeping most degraded with this thruster lost — and why?

3. At what point does a thruster fault require going off DP rather than continuing with reduced capability?

4. What is the correct communication chain for a thruster fault during DP operations near a structure?

### Variants (1)

alarm_003a — Thruster Fault Near Structure

| Change | Value |
| --- | --- |
| Virtual structure active | yes |
| Standoff metres | 80 |

Thruster fault triggered at 80m from structure. Urgency is high. Assess quickly — hold or depart? Your decision window is shorter than you think.

## Slot 3 · watch_003 — DP Log Entry Discipline

| Domain | Watchkeeping and Bridge Team Procedures |
| --- | --- |
| Competency | Watchkeeping |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

9.12

9.13

9.14

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP. Paper and pen ready for log entries. DPO will trigger events during a 20-minute period without announcing what or when. Your job is to log everything correctly.

### Execute

During the 20-minute period your DPO will trigger several events — a reference quality degradation, a wind increase, a thruster loading change, and a position alert. For each event make a DP log entry in the correct format: time, event description, action taken, system status after action. DPO assesses completeness and accuracy of all entries at the end.

### Professional standard

- Every event logged within 2 minutes of occurrence

- Timestamp accurate to within 1 minute

- All required fields completed for each entry

- Action taken clearly stated for each entry

- No events missed

### Observe and record

| Field | Type |
| --- | --- |
| Log entry for Event 1 — include time, event, action, status | text |
| Log entry for Event 2 — include time, event, action, status | text |
| Log entry for Event 3 — include time, event, action, status | text |
| Log entry for Event 4 — include time, event, action, status | text |
| DPO assessment — entries complete and accurate? | text |
| Any events missed or logged late? | text |
| Notes | text |

### Debrief questions

1. What information is mandatory in every DP log entry — what are the minimum required fields?

2. Why is timestamp accuracy critical in a DP log — what happens to your log if a DP incident occurs?

3. What constitutes an event worthy of a log entry versus routine monitoring that does not need logging?

4. Who reads your DP log entries after an incident — and how should that knowledge affect how you write them?

### Variants (1)

watch_003a — Log Entries Under Pressure

| Change | Value |
| --- | --- |
| Events compressed timeframe | yes |

Same exercise but DPO will trigger multiple events in quick succession. Logging discipline under time pressure is harder than when events are spaced out. Prioritise — what gets logged first?

## Slot 4 · fail_001 — Single Thruster Failure Near Structure

| Domain | Failure Scenarios and Degraded Operations |
| --- | --- |
| Competency | Failure management |
| Difficulty | 3 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

11.3

11.4

11.10

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP at operating radius of virtual structure. Moderate conditions settled. DPO will trigger a single thruster failure at an unannounced time during the hold.

### Execute

When the thruster failure occurs, hold position with remaining thrusters. Identify the faulted thruster. Assess new worst case failure. Assess remaining capability for current conditions. Decide whether to maintain position or increase standoff distance. Communicate your decision and reasoning to DPO as if communicating to OIM.

### Professional standard

- Position maintained within double the normal footprint

- Faulted thruster identified within 30 seconds

- Correct new worst case failure identified

- Correct capability assessment for current conditions

- Clear reasoned decision with communication

### Observe and record

| Field | Type |
| --- | --- |
| Which thruster faulted? | text |
| Maximum position excursion during fault (metres) | number |
| Time to stable position after fault (seconds) | number |
| New worst case failure with this thruster lost | text |
| Can you maintain position in current conditions with reduced complement? | text |
| Decision — hold position or increase standoff? Reasoning? | text |
| What would you say to the OIM? | text |
| Notes | text |

### Debrief questions

1. How did your worst case failure change when one thruster was lost — and why does that matter?

2. Was your decision to hold position or increase standoff correct given remaining capability and conditions?

3. What would the ASOG say about continuing operations with this degraded thruster complement?

4. What is your new abort threshold with reduced thrusters — at what environmental condition would you now go off DP?

### Variants (2)

fail_001a — Single Thruster Failure in Heavy Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 25 |
| Wave height m | 2 |

Same failure in heavier conditions. The margin between capable and incapable is much narrower. Your decision to hold or depart will be harder to justify either way.

fail_001b — Critical Thruster Failure

| Change | Value |
| --- | --- |
| Dpo selects critical thruster | yes |

Your DPO will select the most operationally critical thruster to fail for your vessel configuration. This may mean an immediate departure decision is the only safe choice.

## Slot 5 · act_004 — Consequence Analysis Alarm Response

| Domain | ASOG, CAM & TAM Decision Support |
| --- | --- |
| Competency | Decision support application |
| Difficulty | 3 of 5 |
| Duration | 25 min |
| Task type | Applied exercise |
| Vessel modes | DP → DP — sustained hold while working scenarios |

### NI task references

Reference

11.5

11.4

9.11

### Setup

Applicable where the vessel's DP system has consequence analysis fitted and enabled (Class 2/3). Vessel settled on DP holding position; the hold is sustained while the scenario is worked. DPO presents a scenario where a consequence-analysis drift-off warning has been triggered.

### Execute

TDPO must state: what the warning actually means (predicted loss of position/heading if the next failure occurred now, not an actual failure), what status it should trigger per the ASOG, and what the correct operator response is — including whether it is appropriate to continue working under the alarm. While answering, the TDPO references the live thruster loading, position footprint and environmental readings to ground each judgement in the vessel's present state.

### Professional standard

- Does not confuse a consequence-analysis warning with an actual equipment failure

- Correctly states that continuing to work while the alarm is active is a Yellow-status decision requiring justification, not a default

- Can describe what would be happening in the ECR (power plant under stress) that corroborates the alarm

- States the correct action if a further failure occurred while the alarm was already active

### Observe and record

| Field | Type |
| --- | --- |
| Explained warning meaning correctly? | boolean |
| Status stated | text |
| Continue or suspend decision | text |
| Notes | text |

### Debrief questions

1. Is it ever appropriate to keep working while a consequence-analysis drift-off warning is active? What would need to be true for that to be acceptable?

2. Why does the DP system give priority to heading over position when it starts shedding capability — and what does that mean practically for you as the watchkeeper?

3. How would you tell the difference between the DP system being genuinely at its limit versus a false or over-sensitive alarm?

## Slot 6 · er_001 — Backup DP Station Changeover

| Domain | Emergency Response and DP Event Reporting |
| --- | --- |
| Competency | Control station transfer |
| Difficulty | 3 of 5 |
| Duration | 25 min |
| Vessel modes | DP → DP |

### NI task references

Reference

4.2

3.1

11.10

7.19

### Environment

Wind 20 kt @ 315° · Sea 1.5 m @ 315° · Current 0.5 kt @ 270°

### Setup

Vessel settled in DP at datum with the model built. Confirm the backup DP station is powered, available, and showing valid data. Locate your vessel's control transfer procedure and read it once before starting. Note which station currently holds command and record your settled position and heading.

### Execute

Hold position for 5 minutes and record your maximum excursion as a baseline. Then treat the primary station as unavailable: transfer command to the backup DP station following the vessel procedure step by step. Continue holding position from the backup station for 10 minutes. Record the time from decision to completed transfer, the position and heading excursion during the transfer, and every alarm generated.

### Professional standard

- Transfer completed as a continuous deliberate sequence — no backtracking through the procedure

- Command confirmed at the backup station and stated out loud (or logged) before releasing attention from the primary

- Position excursion during transfer no greater than the baseline excursion recorded in the first 5 minutes

- Heading held within 2 degrees throughout

- All alarms generated by the transfer acknowledged and understood, not just silenced

### Observe and record

| Field | Type |
| --- | --- |
| Baseline max excursion, first 5 min (metres) | number |
| Time from decision to completed transfer (seconds) | number |
| Max position excursion during transfer (metres) | number |
| Max heading deviation during transfer (degrees) | number |
| Alarms generated during transfer (count) | number |
| Steps in vessel transfer procedure (count) | number |
| Notes | text |

### Debrief questions

1. What checks would you make at the backup station before accepting command, and what could go wrong if you skipped them?

2. Which failure modes could take out the primary station while leaving the backup healthy? Which failures would take out both?

3. What happens to the vessel's station keeping in the seconds between the primary station being lost and command being established at the backup?

4. How would working at 50m from a structure change your decision about whether to transfer command or take the vessel on joystick first?

5. Why does DP class notation require an independent backup control station at all — what redundancy principle is it protecting?

### Variants (3)

er_001a — Unplanned Changeover

| Change | Value |
| --- | --- |
| Trigger | Supervising DPO calls the transfer at an unannounced moment during a 15-minute hold |

Same procedure, but you no longer choose the moment. Notice how much of your first attempt's smoothness came from being mentally ready.

er_001b — Changeover in Weather

| Change | Value |
| --- | --- |
| Environment — Wind speed kts | 30 |
| Environment — Wave height m | 2.5 |

Bigger excursions, less time to think. The procedure does not change — your margin does.

er_001c — Transfer and Return

| Change | Value |
| --- | --- |
| Add return transfer | yes |

After 10 minutes at the backup station, transfer command back to the primary. Two transfers, both to standard, in one session.

## Slot 7 · prs_003 — Equipment Class and Worst-Case Failure Identification

| Domain | Position Reference Systems, Redundancy and Failure Awareness |
| --- | --- |
| Competency | Redundancy concept application |
| Difficulty | 3 of 5 |
| Duration | 25 min |
| Vessel modes | DP → DP |

### NI task references

Reference

11.3

11.12

3.1

### Setup

Using the vessel's own FMEA or class documentation where available (or the exercise's provided vessel specification), identify the assigned equipment class and the worst-case failure it's designed to survive without losing position.

### Execute

With the vessel settled on DP, talk through what would actually happen to position-keeping if that specific worst-case failure occurred right now. Use the consequence analysis display if the system is fitted with one.

### Professional standard

- Correctly stated the vessel's equipment class from documentation, not assumption

- Could describe the worst-case failure in specific terms (which thruster group, which bus section), not just 'a failure'

- Correctly interpreted whether consequence analysis was currently showing margin or risk

- Distinguished single-fault criteria from a total, unrealistic failure of everything at once

### Observe and record

| Field | Type |
| --- | --- |
| Assigned equipment class | text |
| Defined worst-case failure | text |
| Consequence analysis status/alarm | text |
| Notes | text |

### Debrief questions

1. In your own words: what actually separates equipment class 1, 2 and 3, in terms of the kind of failure the vessel must survive without losing position?

2. Consequence analysis continuously checks that the vessel could survive its worst-case failure right now, under present conditions. What's the practical difference in a real emergency between a vessel that has this running and one that doesn't?

3. A single inadvertent act by someone on board counts as a single fault under class 2/3 criteria. Can you think of a plausible inadvertent act on this vessel that could, on its own, cause the worst-case failure?

4. If the consequence analysis showed marginal capability on your present heading, what are your options, and which would you reach for first?

### Variants (1)

prs_003a — Worst-Case Failure Under Degraded Power

| Change | Value |
| --- | --- |
| One generator offline | yes |

Repeat with one generator already offline before you start. Notice how much the margin in the consequence analysis has already been eaten into before any new failure occurs.

# Day 13 — Failure Scenarios

120 min across 4 slots · target 120 min

## Slot 1 · fail_001 — Single Thruster Failure Near Structure

| Domain | Failure Scenarios and Degraded Operations |
| --- | --- |
| Competency | Failure management |
| Difficulty | 3 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

11.3

11.10

11.12

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP at operating radius of virtual structure. Moderate conditions settled. DPO will trigger a single thruster failure at an unannounced time during the hold.

### Execute

When the thruster failure occurs, hold position with remaining thrusters. Identify the faulted thruster. Assess new worst case failure. Assess remaining capability for current conditions. Decide whether to maintain position or increase standoff distance. Communicate your decision and reasoning to DPO as if communicating to OIM.

### Professional standard

- Position maintained within double the normal footprint

- Faulted thruster identified within 30 seconds

- Correct new worst case failure identified

- Correct capability assessment for current conditions

- Clear reasoned decision with communication

### Observe and record

| Field | Type |
| --- | --- |
| Which thruster faulted? | text |
| Maximum position excursion during fault (metres) | number |
| Time to stable position after fault (seconds) | number |
| New worst case failure with this thruster lost | text |
| Can you maintain position in current conditions with reduced complement? | text |
| Decision — hold position or increase standoff? Reasoning? | text |
| What would you say to the OIM? | text |
| Notes | text |

### Debrief questions

1. How did your worst case failure change when one thruster was lost — and why does that matter?

2. Was your decision to hold position or increase standoff correct given remaining capability and conditions?

3. What would the ASOG say about continuing operations with this degraded thruster complement?

4. What is your new abort threshold with reduced thrusters — at what environmental condition would you now go off DP?

### Variants (2)

fail_001a — Single Thruster Failure in Heavy Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 25 |
| Wave height m | 2 |

Same failure in heavier conditions. The margin between capable and incapable is much narrower. Your decision to hold or depart will be harder to justify either way.

fail_001b — Critical Thruster Failure

| Change | Value |
| --- | --- |
| Dpo selects critical thruster | yes |

Your DPO will select the most operationally critical thruster to fail for your vessel configuration. This may mean an immediate departure decision is the only safe choice.

## Slot 2 · fail_002 — Power Failure Response

| Domain | Failure Scenarios and Degraded Operations |
| --- | --- |
| Competency | Failure management |
| Difficulty | 4 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

11.3

11.11

11.12

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP. DPO will trigger a generator dropout on one bus without warning. Know your vessel power configuration — which thrusters are on which bus — before this drill.

### Execute

When the generator dropout occurs, observe which thrusters lose power. Observe auto load shedding. Assess remaining propulsion capability. Prevent blackout. Either hold position or initiate controlled departure. Communicate to DPO as OIM.

### Professional standard

- Correct identification of thrusters lost on faulted bus

- Auto load shedding observed and understood

- Blackout prevention demonstrated

- Correct hold or depart decision

- Clear communication of situation

### Observe and record

| Field | Type |
| --- | --- |
| Which bus faulted? | text |
| Which thrusters lost power on that bus? | text |
| Did auto load shedding occur? What shed? | text |
| Position excursion during fault (metres) | number |
| Was blackout prevented? How? | text |
| Decision — hold or depart? Reasoning? | text |
| Is this event the vessels defined WCF or worse? | text |
| Notes | text |

### Debrief questions

1. Which thrusters lost power and does the bus configuration of your vessel explain why those ones specifically?

2. What is the role of blackout prevention in this scenario — what would happen without it?

3. Why does the bus tie position matter so much in a power failure scenario?

4. What is the worst case failure for this vessel and how does this event relate to it — was this the WCF or something different?

### Variants (1)

fail_002a — Power Failure Near Structure

| Change | Value |
| --- | --- |
| Virtual structure active | yes |
| Standoff metres | 90 |

Same power failure at 90m from structure. Your decision window is shorter and the consequences of the wrong decision are higher. Decide fast and communicate clearly.

## Slot 3 · drill_003 — Loss of Main DP System UPS

| Domain | DP Emergency Drills & Preparedness |
| --- | --- |
| Competency | Failure response procedural |
| Difficulty | 3 of 5 |
| Duration | 35 min |
| Task type | Live simulator drill |
| Vessel modes | DP → DP |

### NI task references

Reference

3.2

4.8

11.3

### Setup

Vessel in full auto DP, power plant and equipment configured per the vessel's DP operations manual and applicable checklists.

### Execute

Supervising DPO simulates loss of a main DP system UPS by isolating its outputs. TDPO observes and reports the vessel's ability to maintain position using remaining online equipment, and identifies what specifically was lost.

### Professional standard

- Correctly identifies which DP-critical equipment was fed from the affected UPS

- Does not assume the DP system is unaffected just because position is being maintained — checks what redundancy remains

- Recognises that an internal UPS fault can sometimes prevent its own bypass from working, and knows to check for this

- Reports the fault and its implications clearly and promptly

### Observe and record

| Field | Type |
| --- | --- |
| DP equipment affected | text |
| Position maintained on remaining equipment? | boolean |
| Redundancy remaining after the fault | text |
| Notes | text |

### Debrief questions

1. What DP-specific equipment on your vessel is fed by each main UPS, and could you locate that on a single-line diagram if asked?

2. Why might an internal UPS failure prevent a bypass supply from working, even though the bypass circuit itself is intact?

3. What alarms or events specifically indicate a UPS failure rather than a downstream equipment failure?

4. How would you methodically fault-find this rather than just reacting to the first alarm you see?

## Slot 4 · buffer_013 — buffer

Buffer slot · 25 min — Power failure near structure variant plus debrief

# Day 14 — Consequence Alarms and ASOG

120 min across 5 slots · target 120 min

## Slot 1 · alarm_004 — Consequence Alarm Recognition

| Domain | DP Alarms, Warnings and Emergency Response |
| --- | --- |
| Competency | Alarm recognition and response |
| Difficulty | 4 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

11.4

11.5

11.6

### Environment

Wind 20 kt @ 037° · Sea 1.5 m @ 048° · Current 0.5 kt @ 090°

### Setup

Vessel on DP near virtual structure. Moderate to heavy conditions. DPO will configure conditions to trigger consequence alarm.

### Execute

When consequence alarm sounds, respond immediately. Identify the alarm type. Understand its significance. Take appropriate immediate actions. Escalate to OIM immediately. Assess abort criteria. Record everything.

### Professional standard

- Immediate recognition as highest priority alarm

- OIM informed within 60 seconds

- Abort criteria assessed immediately

- Correct understanding of what the alarm predicts

- No hesitation in escalation

### Observe and record

| Field | Type |
| --- | --- |
| Time to acknowledge alarm (seconds) | number |
| Alarm type identified correctly? | text |
| Immediate actions taken — in order | text |
| Time to OIM notification (seconds) | number |
| Abort criteria assessment — continue or abort? | text |
| In your own words — what does a consequence alarm predict? | text |
| Notes | text |

### Debrief questions

1. What does a consequence alarm predict — and why is this alarm categorically different from all other DP alarms?

2. Why is OIM notification the immediate first action rather than trying to resolve the situation yourself?

3. What immediate actions follow a consequence alarm before you have spoken to the OIM?

4. Under what circumstances if any would you accept a consequence alarm and continue operations?

### Variants (1)

alarm_004a — Consequence Alarm During ROV in Water

| Change | Value |
| --- | --- |
| Rov in water | yes |

Same alarm but ROV is deployed. The consequence of position excursion is now significantly higher. How does this change your abort threshold and the urgency of your response?

## Slot 2 · ops_003 — ASOG Application

| Domain | DP Operations and Operational Decision Making |
| --- | --- |
| Competency | Operational decision making |
| Difficulty | 3 of 5 |
| Duration | 25 min |
| Vessel modes | DP → DP |

### NI task references

Reference

7.21

7.22

7.23

7.24

### Setup

DPO will present three operational scenarios. For each scenario you must state the ASOG status, the action required, and who must be informed. No reference material — knowledge only.

### Execute

Scenario 1 — Green status: Conditions and system configuration within all ASOG limits. State status and confirm operation proceeds. Scenario 2 — Yellow status: One parameter approaching its ASOG limit. State status, required action, and communication chain. Scenario 3 — Red or abort criteria: A parameter has exceeded its ASOG limit during an active operation. State status, immediate actions, and who decides whether to continue.

### Professional standard

- Correct ASOG status assessment all three scenarios

- Correct action identified for each status

- Correct communication chain for each scenario

- Correct identification of who has authority to override

### Observe and record

| Field | Type |
| --- | --- |
| Scenario 1 — ASOG status and action | text |
| Scenario 1 — Communication chain | text |
| Scenario 2 — ASOG status and action | text |
| Scenario 2 — Communication chain | text |
| Scenario 3 — ASOG status and action | text |
| Scenario 3 — Communication chain | text |
| Who has authority to continue operations outside ASOG limits? | text |
| Notes | text |

### Debrief questions

1. What is the ASOG and who produces it for this vessel and operation?

2. What is the difference between a yellow and red ASOG status in terms of required actions?

3. Who has authority to continue operations outside ASOG limits and what documentation is required?

4. How does the ASOG relate to the vessel FMEA — are they the same document or different?

### Variants (1)

ops_003a — ASOG Assessment During Live Operation

| Change | Value |
| --- | --- |
| Assessment during hold | yes |

DPO will change conditions during an active hold to create a live ASOG transition — green to yellow to red. You must identify each transition in real time and respond correctly.

## Slot 3 · approach_003 — Extended Hold at Operating Radius

| Domain | Approach to Structure and Close Proximity Operations |
| --- | --- |
| Competency | Approach and proximity |
| Difficulty | 3 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

8.5

8.6

9.1

9.2

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

| Setting | Value |
| --- | --- |
| Ramp to — Wind speed kts | 30 |
| Ramp to — Ramp rate | gradual |

### Setup

Position vessel at DP operating radius of virtual structure. Moderate conditions initially. DPO will ramp wind gradually during the hold.

### Execute

Hold position at the DP operating radius for 20 minutes. During the hold your DPO will gradually increase wind from 15kts to 30kts. Record position footprint at 15, 20, 25, and 30kts. Record thruster demand at each interval. Identify the point at which you would recommend moving further from structure or reviewing operations with the OIM.

### Professional standard

- Position maintained throughout hold

- Correct identification of degrading margin

- Reasoned recommendation point with evidence

- Correct communication chain identified

### Observe and record

| Field | Type |
| --- | --- |
| Position footprint at 15kts wind | text |
| Thruster demand at 15kts (percent) | number |
| Position footprint at 20kts wind | text |
| Thruster demand at 20kts (percent) | number |
| Position footprint at 25kts wind | text |
| Thruster demand at 25kts (percent) | number |
| Position footprint at 30kts wind | text |
| Thruster demand at 30kts (percent) | number |
| Wind speed at which you would recommend reviewing operations (kts) | number |
| Reasoning for your recommendation | text |
| Who would you inform and in what order? | text |
| Notes | text |

### Debrief questions

1. At what wind speed did your position footprint become unacceptable at this operating radius?

2. What is the ASOG concept and how would a properly defined ASOG have defined your action limit here?

3. Who do you inform and in what order when you identify a deteriorating situation during a hold?

4. What is the difference between moving further from the structure and going fully off DP — when would you choose each?

### Variants (1)

approach_003a — Hold with Thruster Failure During Ramp

| Change | Value |
| --- | --- |
| Thruster failure during hold | yes |

Your DPO will fail one thruster at an unannounced point during the environmental ramp. You now have two simultaneous pressures — increasing weather and reduced thruster complement. Assess and decide.

## Slot 4 · er_002 — Collision Threat Escape Move

| Domain | Emergency Response and DP Event Reporting |
| --- | --- |
| Competency | Time critical escape |
| Difficulty | 3 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP position move or JSMH — your choice, justified in debrief |

### NI task references

Reference

8.6

7.13

11.12

### Environment

Wind 18 kt @ 200° · Sea 1.2 m @ 210° · Current 0.4 kt @ 180°

### Setup

Vessel settled in DP at datum. Before anything else, plan your escape: assess the environment and decide, out loud or on paper, the direction you would move the vessel if a drifting vessel or object threatened your position, and whether you would use a DP position move or take the vessel on joystick. Then set a random trigger — either the supervising DPO calls it, or use a timer set between 3 and 10 minutes that you cannot see.

### Execute

Hold position normally. On the trigger, move the vessel 100m in your pre-planned escape direction as quickly as you can do it safely and in control. Record the time from trigger to first commanded input, time to reach 50m and 100m from datum, maximum speed achieved, and maximum heading deviation. Settle the vessel at the new position with residual velocity below 0.1kts.

### Professional standard

- First commanded input within 15 seconds of the trigger

- Escape direction matches the plan made in setup — no improvising a new direction under pressure

- Heading either held throughout or altered deliberately as part of the plan, not lost

- Speed built decisively but deceleration planned — vessel settled at 100m without a large overshoot

- Residual velocity below 0.1kts at the settled position

### Observe and record

| Field | Type |
| --- | --- |
| Time from trigger to first commanded input (seconds) | number |
| Time to 50m from datum (seconds) | number |
| Time to 100m from datum (seconds) | number |
| Maximum speed achieved (kts) | number |
| Max heading deviation (degrees) | number |
| Overshoot past 100m at settle (metres) | number |
| Control method used (DP move / joystick) and why | text |
| Notes | text |

### Debrief questions

1. Why did you choose the escape direction you planned — which environmental force were you working with, and which were you accepting working against?

2. DP position move versus joystick takeover for an escape: what does each give you, what does each cost you, and what would tip your choice one way or the other?

3. At the maximum speed you reached, roughly what stopping distance did you need — and how does that number change what '100m clear' actually buys you?

4. How is this manoeuvre different from a planned departure from a worksite, in both technique and mindset?

5. If the threat direction had been the same as your planned escape direction, what would your alternative have been?

### Variants (2)

er_002a — Escape With Heading Change

| Change | Value |
| --- | --- |
| Require heading change degrees | 45 |

Escape and rotate 45 degrees during the move — as you might to present the stern or reduce beam exposure. Two control problems at once.

er_002b — Escape From Degraded Start

| Change | Value |
| --- | --- |
| Start condition | One thruster deselected before the hold begins |

Reduced authority changes your acceleration, your stopping distance, and possibly your best direction. Re-plan the escape in setup accordingly.

## Slot 5 · dc_002 — Observer Discipline

| Domain | Drill Conduct, Briefing and Debrief |
| --- | --- |
| Competency | Drill observation and timeline recording |
| Difficulty | 3 of 5 |
| Duration | 25 min |
| Vessel modes | DP → DP |

### NI task references

Reference

2.2

2.3

11.3

11.12

### Environment

Wind 20 kt @ 045° · Sea 1.4 m @ 050° · Current 0.5 kt @ 020°

### Setup

This task is best run with a second person — a supervising DPO or another trainee — inducing the failure while you observe. Working alone, induce a familiar single failure yourself and switch immediately into the observer role. Prepare a blank timeline sheet before starting: one column for elapsed time, one for what happened, one for what the operator did.

### Execute

From the moment the failure is applied, keep a continuous timestamped log: first alarm, alarm count, first operator action, every subsequent input, and the moment the vessel is stable again. Nothing is reconstructed afterwards — if you missed an entry, log that you missed it. Finish with maximum excursion and total recovery time.

### Professional standard

- Timeline continuous and written in real time

- Every operator action timestamped

- Missed observations admitted in the record, not backfilled

- Log legible to a third party without verbal explanation

### Observe and record

| Field | Type |
| --- | --- |
| Failure to first alarm (seconds) | number |
| First alarm to first action (seconds) | number |
| Operator actions logged | number |
| Time to stable (seconds) | number |
| Max excursion (metres) | number |
| Entries admitted as missed | number |
| Notes | text |

### Debrief questions

1. Why do properly run drills assign an observer who is not the operator?

2. What does the interval between first alarm and first action tell you, and what in training or ergonomics would shorten it?

3. Which of your observations would justify a change to a checklist or a familiarisation programme?

4. Why is an honest record with gaps worth more than a tidy record written afterwards?

# Day 15 — Mid Programme Review and Advanced Joystick

120 min across 5 slots · target 120 min

## Slot 1 · jc_007 — Blind Stop Challenge

| Domain | Joystick Control and Vessel Handling |
| --- | --- |
| Competency | Stopping and speed management |
| Difficulty | 4 of 5 |
| Duration | 20 min |
| Vessel modes | JSMH |

### NI task references

Reference

7.5

7.14

7.18

### Environment

| Setting | Value |
| --- | --- |
| Value | Set by DPO — unknown to trainee |

### Setup

DPO sets a random drift duration between 3 and 8 minutes and an environment the trainee has not seen. Trainee does not know duration or conditions in advance.

### Execute

DPO starts the drift without telling the trainee how long it will run. Trainee observes the drift. When DPO gives the instruction to stop — stop the vessel. No preparation time. Residual velocity and time to stop are your score.

### Professional standard

- Residual velocity below 0.2kts

- Vessel stopped within 60 seconds of instruction

- Correct assessment of dominant environmental force stated before acting

- Heading maintained during stop

### Observe and record

| Field | Type |
| --- | --- |
| Drift direction assessed before acting | text |
| Dominant force assessed — wind, wave or current? | text |
| Speed at intervention (kts) | number |
| Residual velocity at stop (kts) | number |
| Time from instruction to stopped (seconds) | number |
| Total joystick inputs | number |
| Notes | text |

### Debrief questions

1. How did unknown conditions change your approach compared to prepared drills?

2. What information did you gather in the first 30 seconds of observation — and was it enough?

3. How does this drill simulate real operational conditions when DP fails unexpectedly?

4. What is the difference in your mental process between a prepared stop and a blind stop?

### Variants (1)

jc_007a — Blind Stop with Position Constraint

| Change | Value |
| --- | --- |
| Virtual structure active | yes |

Same blind stop but a virtual structure is active. You must stop before reaching the alert zone. Stakes are now real.

## Slot 2 · alarm_005 — Drive Off Recognition and Response

| Domain | DP Alarms, Warnings and Emergency Response |
| --- | --- |
| Competency | Alarm recognition and response |
| Difficulty | 4 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP then immediate JSMH or manual |

### NI task references

Reference

11.6

11.7

11.8

11.10

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP near virtual structure. DPO will simulate a drive off without warning at an unannounced time.

### Execute

When drive off begins — vessel accelerating away from setpoint with thrusters at maximum in the wrong direction — recognise it, select joystick or manual immediately, and arrest the vessel movement. Record time from drive off initiation to your recognition. Record distance travelled before arrested.

### Professional standard

- Drive off recognised within 30 seconds

- Correct immediate mode selection — joystick or manual

- Vessel movement arrested

- No collision with virtual structure

- Post drive off checks completed

### Observe and record

| Field | Type |
| --- | --- |
| Time from drive off initiation to recognition (seconds) | number |
| What visual signatures indicated a drive off? | text |
| Mode selected on recognition — joystick or manual? | text |
| Distance travelled before vessel arrested (metres) | number |
| Direction of drive off — toward or away from structure? | text |
| Post drive off checks completed before returning to DP? | text |
| Notes | text |

### Debrief questions

1. What are the visual signatures of a drive off on the DP display that distinguish it from a normal position excursion?

2. Why does a drive off require immediate manual intervention rather than waiting for the DP to self-correct?

3. What is the difference between a drive off and a position excursion in terms of cause and response?

4. What checks must be completed after a drive off before you return the vessel to DP control?

### Variants (1)

alarm_005a — Drive Off Toward Structure

| Change | Value |
| --- | --- |
| Drive off direction | toward_structure |

The drive off is toward the structure not away from it. Time to collision is very short. Recognition and action within 15 seconds is the target. This is the worst case drive off scenario.

## Slot 3 · review_002 — Self-Assessment — Mid Programme

| Domain | Self-Assessment and Progress Review |
| --- | --- |
| Competency | Self assessment |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP — sustained hold while conducting review |

### NI task references

None recorded.

### Setup

Vessel on DP in moderate conditions. Run a sustained hold while you reflect.

### Execute

Hold position for 20 minutes. While monitoring the system, answer the following. Has your stopping distance improved since day 1? Has your approach discipline improved? Can you set up the simulator without reference in under 15 minutes? What can you do on day 15 that you could not do on day 1? Discuss with DPO.

### Professional standard

- Measurable comparisons to day 1 performance

- Honest gap assessment relative to NI Simulator Course standard

- Specific focus areas for days 16-25 agreed

- No generic statements — specific evidence only

### Observe and record

| Field | Type |
| --- | --- |
| Stopping distance day 1 vs day 15 — has it improved? By how much? | text |
| Approach discipline improvement — specific evidence | text |
| Setup time now vs day 1 | text |
| What can you do on day 15 that you could not do on day 1? | text |
| Your honest assessment of gap to NI Simulator Course standard | text |
| Agreed focus areas for days 16-25 — agreed with DPO | text |
| Notes | text |

### Debrief questions

1. Is the improvement you identified real and measurable or do you just feel more comfortable?

2. What is the single biggest gap between your current standard and the NI Simulator Course standard?

3. If days 16 to 25 go well what will you be able to do on day 25 that you cannot do today?

4. What does your DPO think your biggest weakness is — and do you agree?

## Slot 4 · ta_001 — Trials Test Sheet Discipline

| Domain | DP Trials and Assurance Practice |
| --- | --- |
| Competency | Test documentation and witnessing |
| Difficulty | 3 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

1.1

1.1.4

6.1

11.9

### Environment

Wind 20 kt @ 210° · Sea 1.5 m @ 220° · Current 0.4 kt @ 180°

### Setup

Choose a single sensor failure test — one gyro is the baseline. Before touching anything, write a formal test sheet: test objective, method steps in order, the alarms you expect, the system response you expect, and the position and heading effect you expect. The sheet is complete before the first action. This is how a trials practitioner works: expected results are committed to paper before the failure is applied, never after.

### Execute

With the model settled, execute your test sheet exactly as written. Deselect the gyro. Record every alarm as it appears, the actual system response, and the actual position and heading excursion. Against each expected item mark confirmed, not confirmed, or unexpected. Reinstate the gyro, verify the system is fully restored, and close the sheet with date, session identifier and a signature line.

### Professional standard

- Test sheet complete before execution begins

- Every expected item has an actual result recorded against it

- Deviations and unexpected events documented, not omitted

- System restored and restoration verified before the sheet is closed

### Observe and record

| Field | Type |
| --- | --- |
| Expected items written before test | number |
| Expected items confirmed | number |
| Expected items not confirmed | number |
| Unexpected events | number |
| Max position excursion (metres) | number |
| Max heading excursion (degrees) | number |
| Time to restore and verify (seconds) | number |
| Notes | text |

### Debrief questions

1. Why must expected results be written before the test rather than reconstructed afterwards?

2. If an actual result did not match your expectation, what does that gap mean for the vessel's assurance documentation, and who would need to know on a real vessel?

3. In formal trials, the person witnessing a test cannot only be the author of the test programme. What failure of assurance does that rule protect against?

4. What is the difference between this exercise run as a trial and the same failure run as a drill — what is each one for?

5. Your test produced a measurable heading excursion. How would you decide whether that excursion is an acceptable result or a finding?

### Variants (2)

ta_001a — Wind Sensor Test Sheet

| Change | Value |
| --- | --- |
| Test subject | single wind sensor deselection |
| Ni task ref 1 | 1.1 |
| Ni task ref 2 | 1.1.4 |
| Ni task ref 3 | 6.3 |
| Ni task ref 4 | 11.9 |

The wind feed drives feedforward thrust. Predict what the thrust picture does the moment the sensor drops — this one is harder to predict correctly than the gyro.

ta_001b — Single PRS Test Sheet

| Change | Value |
| --- | --- |
| Test subject | single position reference deselection |
| Ni task ref 1 | 1.1 |
| Ni task ref 2 | 1.1.4 |
| Ni task ref 3 | 7.9 |
| Ni task ref 4 | 11.9 |

Predict the reference weighting shift, not just the alarm. Your expected-results column should name which surviving reference takes the load.

## Slot 5 · buffer_015 — buffer

Buffer slot · 30 min — Repeat weakest skill identified in review_002

# Day 16 — Compound Failures and Risk Assessment

165 min across 5 slots · target 165 min

## Slot 1 · fail_003 — Compound Failure — Reference Loss During Approach

| Domain | Failure Scenarios and Degraded Operations |
| --- | --- |
| Competency | Failure management |
| Difficulty | 4 of 5 |
| Duration | 35 min |
| Vessel modes | DP → JSMH approach then DP |

### NI task references

Reference

11.3

11.9

11.12

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Begin an active approach to virtual structure from 500m. DPO will trigger reference loss at an unannounced point during the approach — inside the 500m zone.

### Execute

Continue the approach normally. When reference loss occurs inside the 500m zone, make an immediate decision — continue approach on remaining references or abort. If continuing, hold position and reassess. If aborting, execute a controlled departure. There is no correct answer predetermined — your decision must be reasoned and clearly communicated.

### Professional standard

- Immediate recognition of reference loss

- Correct minimum reference assessment at moment of failure

- Clear reasoned decision within 30 seconds

- Correct execution of chosen action

- Clear communication throughout

### Observe and record

| Field | Type |
| --- | --- |
| Distance from structure when reference failed (metres) | number |
| Reference configuration at moment of failure | text |
| Was minimum reference configuration still met? | text |
| Decision — continue or abort? Made within how many seconds? | text |
| Reasoning for decision | text |
| How was the decision executed — was it clean? | text |
| Outcome — vessel safe, zone maintained? | text |
| Notes | text |

### Debrief questions

1. What was your reference configuration at the moment of failure and was minimum configuration still met?

2. Was your decision to continue or abort correct given your proximity to structure at the moment of failure?

3. How does being inside the 500m zone change the minimum reference requirement compared to open water?

4. What would the ASOG say about this scenario — does it define a minimum reference requirement for inside 500m?

### Variants (1)

fail_003a — Compound Failure — Reference Loss Plus Environmental Increase

| Change | Value |
| --- | --- |
| Wind increase simultaneous | yes |
| Wind increase to kts | 25 |

Reference loss and wind increase occur simultaneously during the approach. Two problems at once. Prioritise the higher risk issue first — which is it?

## Slot 2 · ops_004 — Risk Assessment and Toolbox Talk

| Domain | DP Operations and Operational Decision Making |
| --- | --- |
| Competency | Operational planning and execution |
| Difficulty | 3 of 5 |
| Duration | 25 min |
| Vessel modes | DP → DP |

### NI task references

Reference

7.24

7.25

9.11

9.13

### Setup

DPO describes a notional DP operation: vessel holding position near a fixed platform, ROV deployed to 50m depth, a second vessel on standby 300m away, wind 20kts and increasing. You must conduct a risk assessment and toolbox talk before the operation proceeds.

### Execute

Conduct a verbal risk assessment covering all major risk categories: environmental conditions and forecast, vessel capability and ASOG status, reference configuration and quality, SIMOPS considerations, escape route and abort criteria, communication plan, personnel briefing requirements. Then conduct a one-minute toolbox talk covering the top three risks and the actions planned for each.

### Professional standard

- All major risk categories addressed

- Specific risks identified not just categories

- Toolbox talk clear and under 2 minutes

- Top three risks correctly prioritised

- Action plan stated for each top risk

### Observe and record

| Field | Type |
| --- | --- |
| Risk categories covered in assessment | text |
| Risk categories missed — DPO assessment | text |
| Top 3 risks you identified | text |
| Toolbox talk duration (seconds) | number |
| DPO assessment of toolbox talk quality | text |
| Action plan stated for each top risk? | text |
| Notes | text |

### Debrief questions

1. What risk categories did you miss and why are they significant for this type of operation?

2. What makes a toolbox talk effective versus one that people switch off during?

3. How does the risk assessment change if wind increases by 10kts during the operation?

4. Who must be present for the toolbox talk and what happens if a key person is absent?

### Variants (1)

ops_004a — Risk Assessment with Changing Scenario

| Change | Value |
| --- | --- |
| Scenario changes mid assessment | yes |

DPO will change one element of the scenario mid-assessment — a second vessel moves closer, weather forecast worsens, ROV has a technical issue. How does your risk assessment change? Can you adapt in real time?

## Slot 3 · drill_004 — Loss of All Position References

| Domain | DP Emergency Drills & Preparedness |
| --- | --- |
| Competency | Failure response procedural |
| Difficulty | 3 of 5 |
| Duration | 35 min |
| Task type | Live simulator drill |
| Vessel modes | DP → DP |

### NI task references

Reference

11.6

11.9

5.1

### Setup

Vessel on full auto DP, mathematical model allowed to build and stabilise for at least 30 minutes before the drill begins. Environmental conditions noted beforehand.

### Execute

Supervising DPO deselects all Position Reference Systems from DP control. TDPO monitors the vessel's dead-reckoning behaviour using an independent position measuring method (waypoint range/bearing, or UTM northings/eastings) at one-minute intervals for 5 minutes, and reports the drift observed.

### Professional standard

- Correctly sets up an independent monitoring method before the PRSs are deselected, not after

- Recognises the DP system is now on dead-reckoning, not "no control"

- Position change of no more than roughly 20m over 5 minutes is treated as the expected benchmark, and deviation from that prompts further investigation

- Can discuss what should happen next given the vessel's Time to Safely Terminate for the current activity

### Observe and record

| Field | Type |
| --- | --- |
| Starting position (northings/eastings or bearing/range) | text |
| Position at +5 minutes | text |
| Drift distance over 5 minutes (metres) | number |
| Heading change over 5 minutes (degrees) | number |
| Notes | text |

### Debrief questions

1. This drill connects directly to your earlier drift-and-recovery drill design — how does dead-reckoning behaviour here compare to what you saw during a genuine drift-off?

2. Given your vessel's Time to Safely Terminate, at what point after losing all references would you need to have already started terminating the operation?

3. How would this scenario play out differently in benign versus deteriorating weather?

4. What's the very first practical action you'd take, before touching any controls, on realising all references were lost?

### Variants (1)

drill_004a — Combined with Manual/IJS Handover

Once dead-reckoning drift is confirmed, transition to Independent Joystick or manual thruster control and demonstrate a controlled recovery, rather than just observing the drift passively.

## Slot 4 · dc_003 — Structured Debrief and Drill Record

| Domain | Drill Conduct, Briefing and Debrief |
| --- | --- |
| Competency | Post drill debrief and record keeping |
| Difficulty | 3 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

1.3.3

1.3.4

1.3.5

7.24

### Environment

Wind 16 kt @ 200° · Sea 1 m @ 210° · Current 0.3 kt @ 180°

### Setup

Run any familiar single-failure drill on the simulator. The drill itself is not the task — the fifteen minutes after it is. Before starting, prepare a debrief sheet with six headings: factual review; comparison to expected behaviour; communication and decisions; human factors; procedural or training needs; actions and follow-up.

### Execute

Execute the drill and recover. Immediately after recovery, work the debrief sheet aloud, heading by heading. Then complete a formal drill record: type, date, scenario, participants, timings, equipment behaviour, deviations, lessons, and action points — every action point with an owner and a close-out date. The record is finished before the session ends, not later.

### Professional standard

- Debrief conducted immediately after the drill, not at end of session

- At least one honest item recorded under what did not go well

- Every action point has an owner and a close-out date

- Record filed within the same session

### Observe and record

| Field | Type |
| --- | --- |
| Drill duration (minutes) | number |
| Debrief duration (minutes) | number |
| Lessons identified | number |
| Of which: did not go well | number |
| Action points raised | number |
| Action points with owner and date | number |
| Notes | text |

### Debrief questions

1. Why is the value of a drill said to sit in the debrief rather than in the simulation itself?

2. What happens to the honesty of debriefs the first time one is used to assign blame?

3. Pick one lesson from today: which document on a real vessel would it change — a checklist, the familiarisation programme, or a decision support tool — and how?

4. What does an action point without an owner and a date actually achieve?

## Slot 5 · tg_001 — Build and Validate a TAGOS Row

| Domain | Thruster and Generator Operating Strategy |
| --- | --- |
| Competency | Post wcf limit calculation and validation |
| Difficulty | 4 of 5 |
| Duration | 40 min |
| Vessel modes | DP → DP |

### NI task references

Reference

4.4

4.6

7.6

7.17

### Environment

Wind 20 kt @ 250° · Sea 1.5 m @ 260° · Current 0.5 kt @ 230°

### Setup

For your simulator vessel's configuration, write one TAGOS-style row for normal DP operation: thrusters online, generators online, bus-tie state, and — the core of the exercise — the maximum permitted pre-failure thrust limit. Compute it from the governing principle: the units surviving the worst case failure must replace the lost capacity. If the worst case failure removes two of four thrusters, the survivors are half the fleet, so thrust must be held low enough beforehand that the remaining half can carry the total demand. Write the number down before executing anything.

### Execute

Set the configuration and settle in DP. Confirm pre-failure thrust sits inside your computed limit. Then apply the worst case failure: remove the entire redundancy group at once. Hold position for 5 minutes. Record peak survivor thrust, peak generator load, and maximum position excursion. Reinstate and stabilise.

### Professional standard

- Thrust limit computed and written before execution

- Pre-failure thrust confirmed inside the computed limit before the failure is applied

- Position held throughout the post-failure period

- Any exceedance of the computed limits explained in the record, not ignored

### Observe and record

| Field | Type |
| --- | --- |
| Computed pre-failure thrust limit (%) | number |
| Peak pre-failure thrust (%) | number |
| Peak post-failure survivor thrust (%) | number |
| Peak post-failure generator load (%) | number |
| Max post-failure excursion (metres) | number |
| Notes | text |

### Debrief questions

1. Why are thrust limits set against post-failure capacity rather than installed capacity?

2. Your post-failure survivor thrust peaked at a specific number. At roughly what environmental increase would this operation have become unsafe to start?

3. Why does generator step-load capability belong in the same calculation as thruster limits?

4. If a vessel routinely operates above its strategy limits in benign weather without incident, what has actually been lost?

### Variants (1)

tg_001a — TAGOS Validation Beam-On

| Change | Value |
| --- | --- |
| Vessel heading | beam-on to the environment |

Same computed limit, different demand picture. Notice how heading choice consumes or preserves your post-failure margin.

# Day 17 — Heavy Weather Approaches

120 min across 3 slots · target 120 min

## Slot 1 · approach_001 — Leeward Approach to Structure

| Domain | Approach to Structure and Close Proximity Operations |
| --- | --- |
| Competency | Approach and proximity |
| Difficulty | 3 of 5 |
| Duration | 35 min |
| Vessel modes | DP → JSMH inside 500m, DP for final positioning |

### NI task references

Reference

8.1

8.2

8.3

8.4

8.5

8.6

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Place virtual structure 500m on 037 degrees True from your current position. You are on the leeward side — the dominant environmental force is pushing you away from the structure. Confirm your abort heading before starting. Write it down.

### Execute

Enter the 500m zone on a heading of approximately 037 degrees True. Reduce to maximum 0.5kts by the 500m boundary. Reduce to maximum 0.3kts by the 200m boundary. Reduce to maximum 0.1kts by the 100m boundary. Transition from JSMH to DP at 100m. Hold at the DP operating radius for 10 minutes. Record approach track, speed at each zone boundary, and DP capture quality.

### Professional standard

- No hard exclusion zone breach

- Speed below 0.5kts at 500m boundary

- Speed below 0.3kts at 200m boundary

- Speed below 0.1kts at 100m boundary

- DP capture within operating radius

- Abort heading confirmed before entry

### Observe and record

| Field | Type |
| --- | --- |
| Abort heading planned before entry (degrees) | number |
| Windward or leeward — state which and why | text |
| Speed at 500m zone entry (kts) | number |
| Speed at 200m boundary (kts) | number |
| Speed at 100m — transition point (kts) | number |
| Distance from structure at DP capture (metres) | number |
| DP capture quality — smooth or hunting? | text |
| Position footprint during 10 minute hold | text |
| Approximate thruster demand at hold position | text |
| Hard exclusion zone breaches (target zero) | number |
| Notes | text |

### Debrief questions

1. This was a leeward approach — what is the failure mode if DP fails while you are at the operating radius?

2. At what point during the approach was the environment helping your deceleration and at what point was it not?

3. What is your abort heading from the operating radius and how long would it take to clear the 500m zone?

4. How did thruster demand change as you closed distance to the structure during the hold?

### Variants (2)

approach_001a — Leeward Approach in Heavy Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 25 |
| Wave height m | 2 |

Same approach geometry but heavier conditions. Speed management becomes more critical. The environment assists braking more strongly — use it.

approach_001b — Leeward Approach with Degraded Reference

| Change | Value |
| --- | --- |
| One reference degraded | yes |

Your DPO will degrade one reference before you enter the 500m zone. Assess whether to proceed. If you continue, how does reduced reference quality affect your approach discipline?

## Slot 2 · approach_002 — Windward Approach to Structure

| Domain | Approach to Structure and Close Proximity Operations |
| --- | --- |
| Competency | Approach and proximity |
| Difficulty | 4 of 5 |
| Duration | 35 min |
| Vessel modes | DP → JSMH inside 500m, DP for final positioning |

### NI task references

Reference

8.1

8.2

8.3

8.4

8.5

8.6

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Place virtual structure 500m on 217 degrees True from your current position. You are now on the windward side — the dominant environmental force is pushing you toward the structure. Confirm your abort heading before starting. This is more complex on the windward side — think carefully before writing it down.

### Execute

Same speed profile as approach_001. Enter at 217 degrees True, reduce speed at each zone boundary, transition to DP at 100m, hold for 10 minutes. Pay particular attention to how the environment affects your closure rate and thruster demand compared to the leeward approach.

### Professional standard

- No hard exclusion zone breach

- All speed limits observed at each boundary

- Abort heading correctly identified for windward side

- Recognition of increased risk compared to leeward approach

- Higher thruster demand noted and recorded

### Observe and record

| Field | Type |
| --- | --- |
| Abort heading planned before entry (degrees) | number |
| Why did you choose that abort heading on the windward side? | text |
| Speed at 500m zone entry (kts) | number |
| Speed at 200m boundary (kts) | number |
| Speed at 100m — transition point (kts) | number |
| How did thruster demand compare to leeward approach? | text |
| Was closure rate harder to control than leeward? Why? | text |
| Distance from structure at DP capture (metres) | number |
| Hard exclusion zone breaches (target zero) | number |
| Your risk assessment comparison — windward vs leeward | text |
| Notes | text |

### Debrief questions

1. What is the failure mode on the windward side compared to the leeward side — why is windward higher risk?

2. How much higher was thruster demand during the windward approach and hold compared to leeward?

3. Why is the abort heading more complex to plan on the windward side?

4. Under what operational circumstances would you accept a windward approach and what additional safeguards would you put in place?

### Variants (1)

approach_002a — Windward Approach in Heavy Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 25 |
| Wave height m | 2 |

Heavy conditions on the windward side. Closure rate will be hard to control. Start your deceleration significantly earlier than you think necessary.

## Slot 3 · buffer_017 — buffer

Buffer slot · 50 min — Emergency departure drill — alarm triggered at operating radius

# Day 18 — Worst Case Failure

130 min across 4 slots · target 130 min

## Slot 1 · fail_004 — Worst Case Failure Exercise

| Domain | Failure Scenarios and Degraded Operations |
| --- | --- |
| Competency | Failure management |
| Difficulty | 5 of 5 |
| Duration | 40 min |
| Vessel modes | DP → DP |

### NI task references

Reference

11.3

11.4

11.10

11.11

11.12

### Environment

Wind 20 kt @ 037° · Sea 1.5 m @ 048° · Current 0.5 kt @ 090°

### Setup

Vessel on DP at operating radius of virtual structure. Moderate to heavy conditions. Before starting, state to your DPO what you understand to be the vessels defined worst case failure. DPO will then trigger it without further warning during the hold.

### Execute

When the WCF is triggered, observe the full system response — position excursion, alarm sequence, thruster redistribution. Record the excursion distance. Assess whether the excursion would be within safe limits for a notional subsea operation with divers at 50m depth. Make a safety assessment and communicate it clearly.

### Professional standard

- Correct pre-statement of vessel WCF

- Full alarm sequence observed and recorded

- Position excursion measured accurately

- Correct safety assessment for subsea operation context

- Clear communication of assessment

### Observe and record

| Field | Type |
| --- | --- |
| WCF stated before drill — what did you say? | text |
| What was actually triggered by DPO? | text |
| Alarm sequence — which alarms in what order? | text |
| Maximum position excursion (metres) | number |
| Direction of excursion | text |
| Time from failure to maximum excursion (seconds) | number |
| Did vessel recover or continue to excurse? | text |
| Would this excursion be safe for diving or ROV operations? Reasoning? | text |
| Notes | text |

### Debrief questions

1. What is the defined WCF for this vessel — where is it documented and how was it determined?

2. Was the excursion within acceptable limits for a notional subsea operation and what is that limit based on?

3. What is the FMEA and how does it define the WCF for this vessel class?

4. Under what environmental conditions does the WCF excursion become unacceptable for the operations this vessel typically supports?

### Variants (1)

fail_004a — WCF in Heavy Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 30 |
| Wave height m | 2.5 |

Same WCF in significantly heavier conditions. The excursion will be larger and recovery may not occur. Observe the full consequence and assess whether operations should ever be conducted in these conditions on this vessel.

## Slot 2 · ops_003 — ASOG Application

| Domain | DP Operations and Operational Decision Making |
| --- | --- |
| Competency | Operational decision making |
| Difficulty | 3 of 5 |
| Duration | 25 min |
| Vessel modes | DP → DP |

### NI task references

Reference

7.21

7.22

7.23

7.24

### Setup

DPO will present three operational scenarios. For each scenario you must state the ASOG status, the action required, and who must be informed. No reference material — knowledge only.

### Execute

Scenario 1 — Green status: Conditions and system configuration within all ASOG limits. State status and confirm operation proceeds. Scenario 2 — Yellow status: One parameter approaching its ASOG limit. State status, required action, and communication chain. Scenario 3 — Red or abort criteria: A parameter has exceeded its ASOG limit during an active operation. State status, immediate actions, and who decides whether to continue.

### Professional standard

- Correct ASOG status assessment all three scenarios

- Correct action identified for each status

- Correct communication chain for each scenario

- Correct identification of who has authority to override

### Observe and record

| Field | Type |
| --- | --- |
| Scenario 1 — ASOG status and action | text |
| Scenario 1 — Communication chain | text |
| Scenario 2 — ASOG status and action | text |
| Scenario 2 — Communication chain | text |
| Scenario 3 — ASOG status and action | text |
| Scenario 3 — Communication chain | text |
| Who has authority to continue operations outside ASOG limits? | text |
| Notes | text |

### Debrief questions

1. What is the ASOG and who produces it for this vessel and operation?

2. What is the difference between a yellow and red ASOG status in terms of required actions?

3. Who has authority to continue operations outside ASOG limits and what documentation is required?

4. How does the ASOG relate to the vessel FMEA — are they the same document or different?

### Variants (1)

ops_003a — ASOG Assessment During Live Operation

| Change | Value |
| --- | --- |
| Assessment during hold | yes |

DPO will change conditions during an active hold to create a live ASOG transition — green to yellow to red. You must identify each transition in real time and respond correctly.

## Slot 3 · ta_002 — FMEA Prediction and Verification

| Domain | DP Trials and Assurance Practice |
| --- | --- |
| Competency | Fmea verification and gap analysis |
| Difficulty | 4 of 5 |
| Duration | 35 min |
| Vessel modes | DP → DP |

### NI task references

Reference

1.2

4.6

7.6

11.4

### Environment

Wind 22 kt @ 315° · Sea 1.8 m @ 320° · Current 0.6 kt @ 270°

### Setup

From your knowledge of the vessel's DP layout, write down the redundancy groups as you understand them. For the loss of each complete group, predict in writing: which thrusters and generators remain, the peak thrust you expect on the survivors, and whether the current environment sits inside the post-failure capability. All predictions are on paper before anything is executed.

### Execute

Allow the model to settle. Remove every element of the first redundancy group from DP. Hold position for 5 minutes. Record peak thruster load on the survivors, maximum position and heading excursion, and alarm count. Reinstate, stabilise fully, then repeat for the second group. Finish by writing one line per group: prediction confirmed, or gap found — and exactly what the gap was.

### Professional standard

- Predictions written in full before execution

- Both groups tested, with full stabilisation between tests

- Every gap between predicted and observed behaviour explicitly documented

- Environment assessed against post-failure capability before each removal

### Observe and record

| Field | Type |
| --- | --- |
| Group 1 — predicted surviving units | text |
| Group 1 — peak survivor thruster load (%) | number |
| Group 1 — max excursion (metres) | number |
| Group 1 — alarm count | number |
| Group 2 — predicted surviving units | text |
| Group 2 — peak survivor thruster load (%) | number |
| Group 2 — max excursion (metres) | number |
| Group 2 — alarm count | number |
| Prediction gaps found | number |
| Notes | text |

### Debrief questions

1. What is an FMEA gap analysis, and why is a difference between documented failure effects and observed behaviour treated as a serious finding rather than a curiosity?

2. Why do formal trials require the witnessing practitioner to be independent of the vessel?

3. Which group loss produced the worse station-keeping outcome, and does that ranking match your pre-test capability assessment?

4. How would rotating the environment 90 degrees change which group loss is worst?

5. Why must both the prediction and the result survive in the record even when the prediction was wrong?

### Variants (1)

ta_002a — FMEA Verification in Degraded Weather

| Change | Value |
| --- | --- |
| Environment — Wind speed kts | 30 |
| Environment — Wind dir deg | 315 |

Same predictions, harder environment. If your capability assessment says one group removal is no longer acceptable, that written no-go is a valid result — record it and test the other group only.

## Slot 4 · tg_003 — Force Off Recognition

| Domain | Thruster and Generator Operating Strategy |
| --- | --- |
| Competency | Environmental exceedance recognition |
| Difficulty | 3 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

7.17

7.21

9.2

9.12

### Environment

| Setting | Value |
| --- | --- |
| Starting conditions — Wind speed kts | 15 |
| Starting conditions — Wind dir deg | 270 |
| Starting conditions — Wave height m | 1 |
| Starting conditions — Wave dir deg | 280 |
| Starting conditions — Current speed kts | 0.4 |
| Starting conditions — Current dir deg | 250 |
| Escalation | wind increased in 5 kt steps during execution |

### Setup

Settle in DP in the starting environment and log baseline thrust and generator load. Force off is being pushed outside the capability envelope by environment alone, with nothing failed — the loss-of-position mode this task teaches you to see coming. Before the first wind step, decide and write down the mean thrust percentages at which you would declare an advisory status and a degraded status. Note: the power and thrust monitoring habits this task builds map to NI logbook Section 9 references, which can only be signed off during active DP sea time — here you are building the habit, not the sign-off.

### Execute

Increase simulated wind in 5 knot steps, allowing two minutes of settling at each step. At each step record wind speed, mean thrust percentage, and generator load. Declare your pre-decided status changes aloud as their thresholds are crossed. Stop the escalation once mean thrust exceeds 75 percent or position becomes unstable, then step the environment back down in a controlled de-escalation and record the recovery.

### Professional standard

- Status thresholds written before the first wind step

- Every step settled and recorded before the next increase

- Declarations made when thresholds are crossed, not retrospectively

- Controlled de-escalation completed with the vessel stable at every step down

### Observe and record

| Field | Type |
| --- | --- |
| Baseline mean thrust (%) | number |
| Wind at advisory declaration (kts) | number |
| Wind at degraded declaration (kts) | number |
| Wind at 75% mean thrust (kts) | number |
| Max excursion during escalation (metres) | number |
| Generator load at peak (%) | number |
| Notes | text |

### Debrief questions

1. Define drift off, drive off and force off in one sentence each. Which of the three gives the least warning per unit of attention, and why?

2. Why do operating guidelines put status triggers on thrust and generator load rather than on absolute wind speeds?

3. How does heading relative to the environment move the wind speed at which force off occurs?

4. At your advisory declaration point, what should a DPO on a working vessel actually be doing — list the concrete actions, not the sentiment?

### Variants (1)

tg_003a — Force Off with Current Dominant

| Change | Value |
| --- | --- |
| Escalation | current increased in 0.2 kt steps instead of wind |

Current is not measured by any sensor — the DP model infers it. Watch how much later the picture becomes obvious compared to a wind escalation.

# Day 19 — Watchkeeping Standards

120 min across 4 slots · target 120 min

## Slot 1 · watch_001 — Sustained Position Keeping Observation

| Domain | Watchkeeping and Bridge Team Procedures |
| --- | --- |
| Competency | Watchkeeping |
| Difficulty | 2 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP — observe only unless safety issue |

### NI task references

Reference

9.1

9.2

9.3

9.4

9.5

9.7

9.8

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

| Setting | Value |
| --- | --- |
| Ramp to — Wind speed kts | 25 |
| Ramp to — Ramp rate | gradual |

### Setup

Vessel on DP in moderate conditions. DPO will gradually increase environmental conditions during the observation period. Your role is monitor and assess — do not intervene unless a genuine safety issue develops.

### Execute

Monitor the DP system holding position for 30 minutes while conditions increase. Record position footprint, thruster demand, reference quality, and current vector at 5-minute intervals. Identify any trends. Assess at what point intervention would be required. At each 5-minute mark write your observation before looking at the previous entry — do not let previous readings anchor your assessment.

### Professional standard

- All parameters recorded at every 5-minute interval without prompting

- Trends correctly identified from data

- Correct intervention threshold identified with reasoning

- Assessment does not simply mirror the previous reading

### Observe and record

| Field | Type |
| --- | --- |
| 0 min — position footprint, thruster demand, reference quality, current vector | text |
| 5 min — position footprint, thruster demand, reference quality, current vector | text |
| 10 min — position footprint, thruster demand, reference quality, current vector | text |
| 15 min — position footprint, thruster demand, reference quality, current vector | text |
| 20 min — position footprint, thruster demand, reference quality, current vector | text |
| 25 min — position footprint, thruster demand, reference quality, current vector | text |
| 30 min — position footprint, thruster demand, reference quality, current vector | text |
| Trends identified across the 30 minute period | text |
| At what point would you have recommended intervention and why? | text |
| Notes | text |

### Debrief questions

1. What trends did you identify across the 30-minute period — were conditions genuinely deteriorating or just varying?

2. At what point would you have recommended intervention and what specific evidence supported that recommendation?

3. What is the difference between active watchkeeping and passive observation — are you doing one or both?

4. What DP log entries would you have made during this 30-minute period?

### Variants (1)

watch_001a — Sustained Observation Near Structure

| Change | Value |
| --- | --- |
| Virtual structure active | yes |
| Standoff metres | 100 |

Same observation but with a virtual structure at 100m. Degrading conditions now have a direct safety consequence. Your intervention threshold assessment is no longer theoretical.

## Slot 2 · watch_002 — DP Watch Handover

| Domain | Watchkeeping and Bridge Team Procedures |
| --- | --- |
| Competency | Watch handover |
| Difficulty | 3 of 5 |
| Duration | 25 min |
| Vessel modes | DP → DP |

### NI task references

Reference

2.5

2.6

2.7

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP in moderate conditions with a virtual structure active at 100m. A realistic handover scenario — the incoming DPO needs everything.

### Execute

Part 1 — Give handover: You are the outgoing DPO. Brief your DPO on full system status covering every checklist item. Vessel position and heading, mode and setpoint, reference systems and quality, sensor status, thruster configuration, power configuration, environmental conditions, nearby structures and working side, alarms active or recently cleared, any system degradation, operational requirements for the next watch. Part 2 — Receive handover: Swap roles. Receive a handover from your DPO. Ask all questions you would ask as an incoming DPO. Identify any gaps in what you were told.

### Professional standard

- All handover items covered without prompting

- Minimum 5 questions asked as incoming DPO

- Gaps in received handover correctly identified

- Refusal to accept an incomplete handover demonstrated

### Observe and record

| Field | Type |
| --- | --- |
| Items covered in handover you gave — list them | text |
| Items DPO identified as missed in your handover | text |
| Questions you asked when receiving handover | text |
| Gaps identified in the handover you received | text |
| Would you have accepted the watch based on the handover you received? Why or why not? | text |
| Notes | text |

### Debrief questions

1. What items did you miss in the handover you gave — and are those items safety-critical?

2. Under what circumstances would you refuse to accept a watch handover?

3. What is the single most important piece of information in a DP watch handover?

4. How does a DP watch handover differ from a standard bridge watchkeeping handover?

### Variants (1)

watch_002a — Handover with Degraded System

| Change | Value |
| --- | --- |
| System degradation active | yes |

DPO will introduce a system degradation before the handover — one reference inhibited, one thruster on standby, an alarm cleared but not resolved. You must identify all degradations in the handover you receive and decide whether to accept.

## Slot 3 · watch_003 — DP Log Entry Discipline

| Domain | Watchkeeping and Bridge Team Procedures |
| --- | --- |
| Competency | Watchkeeping |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

9.12

9.13

9.14

9.15

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP. Paper and pen ready for log entries. DPO will trigger events during a 20-minute period without announcing what or when. Your job is to log everything correctly.

### Execute

During the 20-minute period your DPO will trigger several events — a reference quality degradation, a wind increase, a thruster loading change, and a position alert. For each event make a DP log entry in the correct format: time, event description, action taken, system status after action. DPO assesses completeness and accuracy of all entries at the end.

### Professional standard

- Every event logged within 2 minutes of occurrence

- Timestamp accurate to within 1 minute

- All required fields completed for each entry

- Action taken clearly stated for each entry

- No events missed

### Observe and record

| Field | Type |
| --- | --- |
| Log entry for Event 1 — include time, event, action, status | text |
| Log entry for Event 2 — include time, event, action, status | text |
| Log entry for Event 3 — include time, event, action, status | text |
| Log entry for Event 4 — include time, event, action, status | text |
| DPO assessment — entries complete and accurate? | text |
| Any events missed or logged late? | text |
| Notes | text |

### Debrief questions

1. What information is mandatory in every DP log entry — what are the minimum required fields?

2. Why is timestamp accuracy critical in a DP log — what happens to your log if a DP incident occurs?

3. What constitutes an event worthy of a log entry versus routine monitoring that does not need logging?

4. Who reads your DP log entries after an incident — and how should that knowledge affect how you write them?

### Variants (1)

watch_003a — Log Entries Under Pressure

| Change | Value |
| --- | --- |
| Events compressed timeframe | yes |

Same exercise but DPO will trigger multiple events in quick succession. Logging discipline under time pressure is harder than when events are spaced out. Prioritise — what gets logged first?

## Slot 4 · buffer_019 — buffer

Buffer slot · 45 min — Sustained observation near structure — watch_001a variant

# Day 20 — First Full Operation

200 min across 3 slots · target 200 min

## Slot 1 · ops_001 — Full DP Operation — End to End

| Domain | DP Operations and Operational Decision Making |
| --- | --- |
| Competency | Operational planning and execution |
| Difficulty | 5 of 5 |
| Duration | 120 min |
| Vessel modes | Off → All modes as required |

### NI task references

Reference

7.1

7.4

7.5

7.20

7.24

7.25

8.1

8.2

8.5

8.6

10.1

10.2

2.5

2.6

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Cold screen. No pre-configuration. This is the full operation with no prompting from DPO unless a genuine safety issue develops.

### Execute

Complete the following sequence without prompting. Phase 1 — Setup: Full system setup from scratch. All checklists completed. Phase 2 — Transit: Move vessel from open water to 500m standoff of virtual structure. Phase 3 — Zone entry: Complete 500m zone entry checklist verbally. Approach to operating radius on JSMH then DP. Phase 4 — Hold: Hold at operating radius for 20 minutes. DPO will trigger one alarm during hold. Phase 5 — Handover: Conduct a full watch handover to DPO. Phase 6 — Departure: Receive the watch back. Execute controlled departure. Return to open water. Save session record.

### Professional standard

- All phases completed in correct sequence without prompting

- No zone breaches during approach or hold

- Alarm during hold responded to correctly

- Handover covers all required items

- Departure executed cleanly

- Session record saved with all observations completed

### Observe and record

| Field | Type |
| --- | --- |
| Setup completion time (minutes) | number |
| 500m zone entry checklist completed without prompting? | text |
| Speed limits observed at each zone boundary? | text |
| DP capture quality at operating radius | text |
| Position footprint during 20 minute hold | text |
| Alarm during hold — identified and responded to correctly? | text |
| DPO assessment of handover quality | text |
| Departure execution — clean and controlled? | text |
| Total hard exclusion zone breaches (target zero) | number |
| Which phases if any required DPO prompting? | text |
| Notes | text |

### Debrief questions

1. Which phase was hardest and what specifically caused the difficulty?

2. What would you do differently on the next full operation?

3. How did the alarm during hold affect your situational awareness for the remainder of the operation?

4. Were your log entries and session record complete enough to reconstruct the operation from the record alone?

### Variants (2)

ops_001a — Full Operation in Heavy Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 25 |
| Wave height m | 2 |

Same full operation in heavier conditions. Every phase becomes harder. Setup and checklist discipline is even more important when the operation itself is more demanding.

ops_001b — Full Operation with Compound Failure

| Change | Value |
| --- | --- |
| Compound failure during hold | yes |

DPO will trigger a compound failure during the hold phase rather than a simple alarm. No warning. This is the assessment-level scenario — Day 29 standard.

## Slot 2 · tg_002 — Maintenance Configuration and Go/No-Go

| Domain | Thruster and Generator Operating Strategy |
| --- | --- |
| Competency | Degraded configuration planning |
| Difficulty | 4 of 5 |
| Duration | 35 min |
| Vessel modes | DP → DP |

### NI task references

Reference

4.5

4.6

7.6

11.3

### Environment

Wind 22 kt @ 190° · Sea 1.6 m @ 200° · Current 0.5 kt @ 160°

### Setup

Take one thruster out of service — deselected and treated as under maintenance for the whole task. Recompute the operating strategy row for this degraded configuration: which units now survive the worst case failure, and what the new pre-failure thrust limit must be. Then make a written go or no-go call: is the current environment still acceptable for DP operations in this state? Write the reasoning, not just the answer.

### Execute

If your call is go: settle in the degraded configuration, confirm thrust inside the recomputed limit, apply the worst case failure to the remaining worst group, and hold for 5 minutes. If your call is no-go: reduce the simulated environment step by step until the answer becomes go, record that threshold, then execute the failure at the threshold. Either way, record survivor thrust, maximum excursion, and heading deviation.

### Professional standard

- Recomputed limit and go/no-go decision written before operating

- Threshold reasoning recorded, not just the number

- Post-failure control assessed honestly against the recomputed limits

- Heading held throughout the post-failure period

### Observe and record

| Field | Type |
| --- | --- |
| Recomputed thrust limit (%) | number |
| Go/no-go wind threshold (kts) | number |
| Peak post-failure survivor thrust (%) | number |
| Max excursion (metres) | number |
| Max heading deviation (degrees) | number |
| Notes | text |

### Debrief questions

1. Operating with a thruster offline is a textbook task appropriate mode decision. What must exist on paper, and who must sign it, before a real vessel operates that way?

2. Who is notified, and what operating-guideline status change occurs, when DP equipment comes out of service mid-operation?

3. Why is reinstating equipment done outside the work area wherever possible?

4. Your go/no-go threshold was a single number. Describe what sits on each side of it.

## Slot 3 · dv_001 — The Paperwork Holding You in Position

| Domain | DP Trials and Assurance Practice |
| --- | --- |
| Competency | Dp documentation and verification |
| Difficulty | 2 of 5 |
| Duration | 45 min |
| Vessel modes | DP → DP — sustained hold throughout documentation audit |

### NI task references

Reference

1.1

1.1.5

1.2

1.3.1

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Establish a stable DP hold at your datum with the model fully built. The vessel does nothing but hold position for this entire task. That is the point: while she holds, you audit the documentation that certifies she can. Have a notebook ready — you will be moving around the bridge, not sitting at the desk.

### Execute

While the vessel holds position, locate aboard — or, where the simulator vessel's set differs, name and state the correct location of — each of the following: the DPVAD; the FMEA and its proving trials results; the DP operations manual; annual survey and periodic (5-year) test records; DP capability plots (fully operational AND post-worst-case-failure); the location checklist; the watchkeeping checklist; the blackout recovery procedure; the list of critical components; and the ASOG if the vessel carries one. For each, record where it lives, its date or revision, and one sentence on what it proves about the system holding position in front of you right now. Then make three live cross-checks: (1) confirm the equipment class stated on the DPVAD matches the redundancy actually set up on the desk — bus configuration, thruster split across power systems, number and diversity of position references; (2) plot the current wind and sea state against the post-worst-case-failure capability plot and state whether you are inside the envelope, and by how much; (3) confirm the watchkeeping checklist in use at the desk is the version referenced in the operations manual.

### Professional standard

- Every listed document located, or its absence positively identified and recorded — no line left blank

- DPVAD equipment class correctly matched against the live desk configuration, with the specific evidence stated (bus state, thruster allocation, PRS count and diversity)

- Current conditions correctly placed inside or outside the post-worst-case-failure capability envelope, with the margin stated

- Currency correctly assessed for each record: annual survey within three months either side of the DPVAD anniversary date, periodic testing within five years, FMEA revision reflecting the vessel as she currently is

### Observe and record

| Field | Type |
| --- | --- |
| Equipment class stated on DPVAD | text |
| DPVAD valid until (date) | text |
| Last annual survey date | text |
| FMEA revision date | text |
| Documents located (of 10) | number |
| Documents missing or out of date | text |
| Post-WCF capability check — inside/outside envelope and margin | text |
| DPVAD class vs live desk configuration — match or discrepancy | text |
| Notes | text |

### Debrief questions

1. The annual survey does not just confirm the equipment works — it tests the vessel's ability to keep position after single failures for her assigned equipment class, and validates the FMEA and operations manual. Why is that distinction the entire point of the survey, and what would a survey that only function-tested equipment fail to prove?

2. Name three things that cause a DPVAD to cease to be valid, and explain for each one exactly why it breaks the chain of assurance between the paperwork and the vessel holding position right now.

3. Your online consequence analysis and the post-worst-case-failure capability plot answer the same question — 'will she hold if the worst single failure happens now?' — in two different ways. What does each rely on, what does each assume, and why does the guidance require both rather than either one alone?

4. The FMEA must be kept updated so that it remains current, and no significant change may be made to the DP system after survey without sanction, except direct like-for-like replacement for repair. A yard fitted a different-model thruster drive last refit. What has to happen to the FMEA, the proving trials and the DPVAD before this vessel legitimately works a Class 2 operation again?

5. You join this vessel cold tomorrow as an incoming DPO. Of everything you handled in this task, which single document do you read first, and why does it beat the others?

### Variants (1)

dv_001a — The Gap

| Change | Value |
| --- | --- |
| Inject discrepancy | yes |

Have your supervising DPO secretly designate one document as missing, expired, or mismatched against the desk — or set the trap for yourself before you start. When you find it, state the operational consequence in one sentence, whether DP operations may continue, and who gets told before anything else happens. Finding the gap is easy. Knowing what it costs is the skill.

# Day 21 — Advanced Sensor Management

120 min across 4 slots · target 120 min

## Slot 1 · sen_001 — Reference System Weighting

| Domain | Sensor and Reference System Management |
| --- | --- |
| Competency | Reference management |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

5.1

5.2

7.9

7.10

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Two DGPS references active. Vessel stable on DP. Open the Sensors tab and observe the weighting assigned to each reference before doing anything.

### Execute

Record initial weights for both references. Ask your DPO to introduce simulated HDOP degradation on one reference. Observe how the weighting changes. Record the new weights. Note how position quality changes. Assess at what point you would manually inhibit the degraded reference rather than waiting for auto-rejection. Re-enable and confirm both references return to healthy status.

### Professional standard

- Correctly record initial reference weights

- Observe and record weight change after degradation

- Correct threshold assessment for manual inhibit

- Correct procedure for re-enabling recovered reference

### Observe and record

| Field | Type |
| --- | --- |
| Reference 1 initial weight | text |
| Reference 2 initial weight | text |
| Reference 1 weight after degradation introduced | text |
| Reference 2 weight after degradation introduced | text |
| How did position quality change after degradation? | text |
| At what point would you manually inhibit — and why that point? | text |
| Did both references return to healthy status on re-enable? | text |
| Notes | text |

### Debrief questions

1. How does the DP Kalman filter decide how much weight to assign each reference — what factors influence this?

2. What HDOP value would concern you and at what value would you act?

3. At what point would you manually inhibit a reference rather than waiting for auto-rejection — and what is the risk of waiting?

4. What is the difference between a reference being auto-rejected and manually inhibited — and does it matter which way it happens?

### Variants (1)

sen_001a — Reference Weighting with Three References

| Change | Value |
| --- | --- |
| Reference count | 3 |

Add a third reference and observe how the weighting distributes across three. Then degrade one — how do the remaining two respond?

## Slot 2 · sen_002 — GPS Failure Response

| Domain | Sensor and Reference System Management |
| --- | --- |
| Competency | Reference management |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

5.1

5.2

7.9

11.9

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Two DGPS references active. Vessel stable on DP in moderate conditions. Note current position quality.

### Execute

Without warning your DPO will disable one GPS reference. Observe the alarm sequence. Record which alarms triggered and in what order. Assess your remaining reference configuration. Decide whether to continue or go off DP. Re-enable the failed GPS and confirm re-acceptance procedure.

### Professional standard

- Alarm acknowledged within 15 seconds

- Correct assessment of remaining reference configuration

- Correct go or no-go decision with reasoning

- Correct re-acceptance procedure for recovered reference

### Observe and record

| Field | Type |
| --- | --- |
| Alarm sequence — which alarms in what order? | text |
| Time to acknowledge first alarm (seconds) | number |
| Your assessment of remaining reference configuration | text |
| Decision — continue or go off DP? Reasoning? | text |
| Position quality with single reference | text |
| How did you re-accept the recovered reference? | text |
| Notes | text |

### Debrief questions

1. What is the minimum reference configuration for your vessel's typical operations and where is this defined?

2. Why does losing one GPS not automatically mean you must go off DP?

3. What changed about your position quality with only one reference active?

4. When re-enabling a recovered reference — why must you confirm it is healthy before accepting it back into the solution?

### Variants (2)

sen_002a — GPS Failure Near Structure

| Change | Value |
| --- | --- |
| Virtual structure active | yes |
| Standoff metres | 150 |

Same failure but you are 150m from a virtual structure. Does proximity change your minimum reference requirement? Does it change your decision?

sen_002b — Both GPS References Degrade Simultaneously

| Change | Value |
| --- | --- |
| Both references degrade | yes |

Harder scenario. Both references degrade at the same time. What is your immediate action? This is a genuine off-DP decision.

## Slot 3 · sen_003 — Gyro Failure Response

| Domain | Sensor and Reference System Management |
| --- | --- |
| Competency | Sensor management |
| Difficulty | 3 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

6.1

7.8

11.9

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Both gyros active. Note heading agreement between gyro 1 and gyro 2 — should be within 0.5 degrees. Vessel stable on DP.

### Execute

Without warning your DPO will introduce a 5 degree heading error on one gyro. Observe how the DP responds. Does it auto-reject? Does heading control degrade? Record the heading excursion. Manually inhibit the faulty gyro. Observe recovery. Record time from fault introduction to stable heading.

### Professional standard

- Identify faulty gyro within 30 seconds

- Correct inhibit procedure executed

- Recovery confirmed before continuing

- Time from fault to stable heading recorded

### Observe and record

| Field | Type |
| --- | --- |
| Initial gyro 1 vs gyro 2 agreement (degrees difference) | number |
| Did the DP auto-reject the faulty gyro? Yes or no? | text |
| Maximum heading excursion during fault (degrees) | number |
| Which gyro was identified as faulty and how? | text |
| Describe the inhibit procedure used | text |
| Time from fault introduction to stable heading (seconds) | number |
| Notes | text |

### Debrief questions

1. How does the DP decide which gyro to trust when two gyros disagree — what is the voting logic?

2. What heading excursion is acceptable during a gyro failure response near a structure?

3. Why is gyro failure more immediately dangerous to vessel position than a GPS failure?

4. How does heading error propagate into position error — explain the mechanism?

### Variants (1)

sen_003a — Gyro Failure During Heading Change

| Change | Value |
| --- | --- |
| Failure during heading change | yes |

Same failure introduced while the vessel is mid-turn. The combination of active heading change and gyro fault is harder to manage. Prioritise stopping the turn first.

## Slot 4 · buffer_021 — buffer

Buffer slot · 60 min — Combined sensor degradation scenario — reference plus gyro

# Day 22 — Compound Failure Scenarios

120 min across 4 slots · target 120 min

## Slot 1 · fail_001 — Single Thruster Failure Near Structure

| Domain | Failure Scenarios and Degraded Operations |
| --- | --- |
| Competency | Failure management |
| Difficulty | 3 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

11.3

11.10

11.12

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP at operating radius of virtual structure. Moderate conditions settled. DPO will trigger a single thruster failure at an unannounced time during the hold.

### Execute

When the thruster failure occurs, hold position with remaining thrusters. Identify the faulted thruster. Assess new worst case failure. Assess remaining capability for current conditions. Decide whether to maintain position or increase standoff distance. Communicate your decision and reasoning to DPO as if communicating to OIM.

### Professional standard

- Position maintained within double the normal footprint

- Faulted thruster identified within 30 seconds

- Correct new worst case failure identified

- Correct capability assessment for current conditions

- Clear reasoned decision with communication

### Observe and record

| Field | Type |
| --- | --- |
| Which thruster faulted? | text |
| Maximum position excursion during fault (metres) | number |
| Time to stable position after fault (seconds) | number |
| New worst case failure with this thruster lost | text |
| Can you maintain position in current conditions with reduced complement? | text |
| Decision — hold position or increase standoff? Reasoning? | text |
| What would you say to the OIM? | text |
| Notes | text |

### Debrief questions

1. How did your worst case failure change when one thruster was lost — and why does that matter?

2. Was your decision to hold position or increase standoff correct given remaining capability and conditions?

3. What would the ASOG say about continuing operations with this degraded thruster complement?

4. What is your new abort threshold with reduced thrusters — at what environmental condition would you now go off DP?

### Variants (2)

fail_001a — Single Thruster Failure in Heavy Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 25 |
| Wave height m | 2 |

Same failure in heavier conditions. The margin between capable and incapable is much narrower. Your decision to hold or depart will be harder to justify either way.

fail_001b — Critical Thruster Failure

| Change | Value |
| --- | --- |
| Dpo selects critical thruster | yes |

Your DPO will select the most operationally critical thruster to fail for your vessel configuration. This may mean an immediate departure decision is the only safe choice.

## Slot 2 · fail_002 — Power Failure Response

| Domain | Failure Scenarios and Degraded Operations |
| --- | --- |
| Competency | Failure management |
| Difficulty | 4 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

11.3

11.11

11.12

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP. DPO will trigger a generator dropout on one bus without warning. Know your vessel power configuration — which thrusters are on which bus — before this drill.

### Execute

When the generator dropout occurs, observe which thrusters lose power. Observe auto load shedding. Assess remaining propulsion capability. Prevent blackout. Either hold position or initiate controlled departure. Communicate to DPO as OIM.

### Professional standard

- Correct identification of thrusters lost on faulted bus

- Auto load shedding observed and understood

- Blackout prevention demonstrated

- Correct hold or depart decision

- Clear communication of situation

### Observe and record

| Field | Type |
| --- | --- |
| Which bus faulted? | text |
| Which thrusters lost power on that bus? | text |
| Did auto load shedding occur? What shed? | text |
| Position excursion during fault (metres) | number |
| Was blackout prevented? How? | text |
| Decision — hold or depart? Reasoning? | text |
| Is this event the vessels defined WCF or worse? | text |
| Notes | text |

### Debrief questions

1. Which thrusters lost power and does the bus configuration of your vessel explain why those ones specifically?

2. What is the role of blackout prevention in this scenario — what would happen without it?

3. Why does the bus tie position matter so much in a power failure scenario?

4. What is the worst case failure for this vessel and how does this event relate to it — was this the WCF or something different?

### Variants (1)

fail_002a — Power Failure Near Structure

| Change | Value |
| --- | --- |
| Virtual structure active | yes |
| Standoff metres | 90 |

Same power failure at 90m from structure. Your decision window is shorter and the consequences of the wrong decision are higher. Decide fast and communicate clearly.

## Slot 3 · ta_003 — Mini Trials Programme

| Domain | DP Trials and Assurance Practice |
| --- | --- |
| Competency | Trials programme design and record |
| Difficulty | 4 of 5 |
| Duration | 45 min |
| Vessel modes | DP → DP |

### NI task references

Reference

1.1

1.1.4

1.3.3

7.20

### Environment

Wind 18 kt @ 150° · Sea 1.2 m @ 160° · Current 0.4 kt @ 120°

### Setup

Design a three-test programme covering one power system test, one thruster or propulsion test, and one reference or sensor test. Write a full test sheet for each, then sequence them deliberately — decide which runs first and why. Prepare a single trials record header: date, session, environment, vessel configuration.

### Execute

Run the programme in your planned sequence. Between tests: reinstate, verify restoration, and allow full stabilisation before the next test begins. Log actual start and finish times per test, results against expected, and any deviation from your written method the moment it happens. Close out the record with a summary of findings and any items you would carry forward to a future session.

### Professional standard

- Programme fully written and sequenced before the first test

- Restoration verified — not assumed — before each subsequent test

- Deviations from written method logged at the time, not afterwards

- Record complete enough that another TDPO could repeat the programme without asking a single question

### Observe and record

| Field | Type |
| --- | --- |
| Tests completed | number |
| Deviations from written method | number |
| Total programme time (minutes) | number |
| Longest stabilisation wait between tests (seconds) | number |
| Findings carried forward | number |
| Notes | text |

### Debrief questions

1. What logic did you use to sequence the three tests, and what would go wrong if you ran them in reverse order?

2. Why must restoration be verified rather than assumed before the next test starts?

3. What makes a trials record repeatable, and why does repeatability matter more than neatness?

4. In real annual trials, the trials document exists in a pre-trials revision written before anyone steps aboard. Why is the authorship done in advance?

5. Which of your three tests would you promote into a recurring personal drill, and why that one?

## Slot 4 · buffer_022 — buffer

Buffer slot · 15 min — Compound failure — thruster loss plus reference degradation simultaneously

# Day 23 — Heavy Weather Approaches and Compound Failures

120 min across 3 slots · target 120 min

## Slot 1 · approach_003 — Extended Hold at Operating Radius

| Domain | Approach to Structure and Close Proximity Operations |
| --- | --- |
| Competency | Approach and proximity |
| Difficulty | 3 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP |

### NI task references

Reference

8.5

8.6

9.1

9.2

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

| Setting | Value |
| --- | --- |
| Ramp to — Wind speed kts | 30 |
| Ramp to — Ramp rate | gradual |

### Setup

Position vessel at DP operating radius of virtual structure. Moderate conditions initially. DPO will ramp wind gradually during the hold.

### Execute

Hold position at the DP operating radius for 20 minutes. During the hold your DPO will gradually increase wind from 15kts to 30kts. Record position footprint at 15, 20, 25, and 30kts. Record thruster demand at each interval. Identify the point at which you would recommend moving further from structure or reviewing operations with the OIM.

### Professional standard

- Position maintained throughout hold

- Correct identification of degrading margin

- Reasoned recommendation point with evidence

- Correct communication chain identified

### Observe and record

| Field | Type |
| --- | --- |
| Position footprint at 15kts wind | text |
| Thruster demand at 15kts (percent) | number |
| Position footprint at 20kts wind | text |
| Thruster demand at 20kts (percent) | number |
| Position footprint at 25kts wind | text |
| Thruster demand at 25kts (percent) | number |
| Position footprint at 30kts wind | text |
| Thruster demand at 30kts (percent) | number |
| Wind speed at which you would recommend reviewing operations (kts) | number |
| Reasoning for your recommendation | text |
| Who would you inform and in what order? | text |
| Notes | text |

### Debrief questions

1. At what wind speed did your position footprint become unacceptable at this operating radius?

2. What is the ASOG concept and how would a properly defined ASOG have defined your action limit here?

3. Who do you inform and in what order when you identify a deteriorating situation during a hold?

4. What is the difference between moving further from the structure and going fully off DP — when would you choose each?

### Variants (1)

approach_003a — Hold with Thruster Failure During Ramp

| Change | Value |
| --- | --- |
| Thruster failure during hold | yes |

Your DPO will fail one thruster at an unannounced point during the environmental ramp. You now have two simultaneous pressures — increasing weather and reduced thruster complement. Assess and decide.

## Slot 2 · fail_003 — Compound Failure — Reference Loss During Approach

| Domain | Failure Scenarios and Degraded Operations |
| --- | --- |
| Competency | Failure management |
| Difficulty | 4 of 5 |
| Duration | 35 min |
| Vessel modes | DP → JSMH approach then DP |

### NI task references

Reference

11.3

11.9

11.12

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Begin an active approach to virtual structure from 500m. DPO will trigger reference loss at an unannounced point during the approach — inside the 500m zone.

### Execute

Continue the approach normally. When reference loss occurs inside the 500m zone, make an immediate decision — continue approach on remaining references or abort. If continuing, hold position and reassess. If aborting, execute a controlled departure. There is no correct answer predetermined — your decision must be reasoned and clearly communicated.

### Professional standard

- Immediate recognition of reference loss

- Correct minimum reference assessment at moment of failure

- Clear reasoned decision within 30 seconds

- Correct execution of chosen action

- Clear communication throughout

### Observe and record

| Field | Type |
| --- | --- |
| Distance from structure when reference failed (metres) | number |
| Reference configuration at moment of failure | text |
| Was minimum reference configuration still met? | text |
| Decision — continue or abort? Made within how many seconds? | text |
| Reasoning for decision | text |
| How was the decision executed — was it clean? | text |
| Outcome — vessel safe, zone maintained? | text |
| Notes | text |

### Debrief questions

1. What was your reference configuration at the moment of failure and was minimum configuration still met?

2. Was your decision to continue or abort correct given your proximity to structure at the moment of failure?

3. How does being inside the 500m zone change the minimum reference requirement compared to open water?

4. What would the ASOG say about this scenario — does it define a minimum reference requirement for inside 500m?

### Variants (1)

fail_003a — Compound Failure — Reference Loss Plus Environmental Increase

| Change | Value |
| --- | --- |
| Wind increase simultaneous | yes |
| Wind increase to kts | 25 |

Reference loss and wind increase occur simultaneously during the approach. Two problems at once. Prioritise the higher risk issue first — which is it?

## Slot 3 · buffer_023 — buffer

Buffer slot · 55 min — Full scenario — approach in heavy weather with unannounced failure at operating radius

# Day 24 — Watchkeeping and Operational Decision Making

120 min across 4 slots · target 120 min

## Slot 1 · watch_003 — DP Log Entry Discipline

| Domain | Watchkeeping and Bridge Team Procedures |
| --- | --- |
| Competency | Watchkeeping |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

9.12

9.13

9.14

9.15

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP. Paper and pen ready for log entries. DPO will trigger events during a 20-minute period without announcing what or when. Your job is to log everything correctly.

### Execute

During the 20-minute period your DPO will trigger several events — a reference quality degradation, a wind increase, a thruster loading change, and a position alert. For each event make a DP log entry in the correct format: time, event description, action taken, system status after action. DPO assesses completeness and accuracy of all entries at the end.

### Professional standard

- Every event logged within 2 minutes of occurrence

- Timestamp accurate to within 1 minute

- All required fields completed for each entry

- Action taken clearly stated for each entry

- No events missed

### Observe and record

| Field | Type |
| --- | --- |
| Log entry for Event 1 — include time, event, action, status | text |
| Log entry for Event 2 — include time, event, action, status | text |
| Log entry for Event 3 — include time, event, action, status | text |
| Log entry for Event 4 — include time, event, action, status | text |
| DPO assessment — entries complete and accurate? | text |
| Any events missed or logged late? | text |
| Notes | text |

### Debrief questions

1. What information is mandatory in every DP log entry — what are the minimum required fields?

2. Why is timestamp accuracy critical in a DP log — what happens to your log if a DP incident occurs?

3. What constitutes an event worthy of a log entry versus routine monitoring that does not need logging?

4. Who reads your DP log entries after an incident — and how should that knowledge affect how you write them?

### Variants (1)

watch_003a — Log Entries Under Pressure

| Change | Value |
| --- | --- |
| Events compressed timeframe | yes |

Same exercise but DPO will trigger multiple events in quick succession. Logging discipline under time pressure is harder than when events are spaced out. Prioritise — what gets logged first?

## Slot 2 · ops_004 — Risk Assessment and Toolbox Talk

| Domain | DP Operations and Operational Decision Making |
| --- | --- |
| Competency | Operational planning and execution |
| Difficulty | 3 of 5 |
| Duration | 25 min |
| Vessel modes | DP → DP |

### NI task references

Reference

7.24

7.25

9.11

9.13

### Setup

DPO describes a notional DP operation: vessel holding position near a fixed platform, ROV deployed to 50m depth, a second vessel on standby 300m away, wind 20kts and increasing. You must conduct a risk assessment and toolbox talk before the operation proceeds.

### Execute

Conduct a verbal risk assessment covering all major risk categories: environmental conditions and forecast, vessel capability and ASOG status, reference configuration and quality, SIMOPS considerations, escape route and abort criteria, communication plan, personnel briefing requirements. Then conduct a one-minute toolbox talk covering the top three risks and the actions planned for each.

### Professional standard

- All major risk categories addressed

- Specific risks identified not just categories

- Toolbox talk clear and under 2 minutes

- Top three risks correctly prioritised

- Action plan stated for each top risk

### Observe and record

| Field | Type |
| --- | --- |
| Risk categories covered in assessment | text |
| Risk categories missed — DPO assessment | text |
| Top 3 risks you identified | text |
| Toolbox talk duration (seconds) | number |
| DPO assessment of toolbox talk quality | text |
| Action plan stated for each top risk? | text |
| Notes | text |

### Debrief questions

1. What risk categories did you miss and why are they significant for this type of operation?

2. What makes a toolbox talk effective versus one that people switch off during?

3. How does the risk assessment change if wind increases by 10kts during the operation?

4. Who must be present for the toolbox talk and what happens if a key person is absent?

### Variants (1)

ops_004a — Risk Assessment with Changing Scenario

| Change | Value |
| --- | --- |
| Scenario changes mid assessment | yes |

DPO will change one element of the scenario mid-assessment — a second vessel moves closer, weather forecast worsens, ROV has a technical issue. How does your risk assessment change? Can you adapt in real time?

## Slot 3 · watch_002 — DP Watch Handover

| Domain | Watchkeeping and Bridge Team Procedures |
| --- | --- |
| Competency | Watch handover |
| Difficulty | 3 of 5 |
| Duration | 25 min |
| Vessel modes | DP → DP |

### NI task references

Reference

2.5

2.6

2.7

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP in moderate conditions with a virtual structure active at 100m. A realistic handover scenario — the incoming DPO needs everything.

### Execute

Part 1 — Give handover: You are the outgoing DPO. Brief your DPO on full system status covering every checklist item. Vessel position and heading, mode and setpoint, reference systems and quality, sensor status, thruster configuration, power configuration, environmental conditions, nearby structures and working side, alarms active or recently cleared, any system degradation, operational requirements for the next watch. Part 2 — Receive handover: Swap roles. Receive a handover from your DPO. Ask all questions you would ask as an incoming DPO. Identify any gaps in what you were told.

### Professional standard

- All handover items covered without prompting

- Minimum 5 questions asked as incoming DPO

- Gaps in received handover correctly identified

- Refusal to accept an incomplete handover demonstrated

### Observe and record

| Field | Type |
| --- | --- |
| Items covered in handover you gave — list them | text |
| Items DPO identified as missed in your handover | text |
| Questions you asked when receiving handover | text |
| Gaps identified in the handover you received | text |
| Would you have accepted the watch based on the handover you received? Why or why not? | text |
| Notes | text |

### Debrief questions

1. What items did you miss in the handover you gave — and are those items safety-critical?

2. Under what circumstances would you refuse to accept a watch handover?

3. What is the single most important piece of information in a DP watch handover?

4. How does a DP watch handover differ from a standard bridge watchkeeping handover?

### Variants (1)

watch_002a — Handover with Degraded System

| Change | Value |
| --- | --- |
| System degradation active | yes |

DPO will introduce a system degradation before the handover — one reference inhibited, one thruster on standby, an alarm cleared but not resolved. You must identify all degradations in the handover you receive and decide whether to accept.

## Slot 4 · buffer_024 — buffer

Buffer slot · 50 min — Full watch cycle — hold, log entries, alarm, handover

# Day 25 — Pre-Completion Review and Open Water Drills

120 min across 4 slots · target 120 min

## Slot 1 · review_003 — Self-Assessment — Pre-Completion Review

| Domain | Self-Assessment and Progress Review |
| --- | --- |
| Competency | Self assessment |
| Difficulty | 3 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP — sustained hold while conducting review |

### NI task references

None recorded.

### Setup

Vessel on DP. Run the system while you conduct this assessment.

### Execute

Hold position and reflect honestly. Are you ready for the NI Simulator Course — yes or no, with evidence. What is the one skill you are least confident about? What scenario would most expose that weakness? What is the one thing you most wish you had more time to practise? Discuss with DPO.

### Professional standard

- Honest yes or no to readiness question with evidence

- One specific skill identified as weakest

- Specific scenario identified as most exposing

- Final five sessions have a concrete plan

### Observe and record

| Field | Type |
| --- | --- |
| Are you ready for the NI Simulator Course? Yes or no and why? | text |
| The one skill you are least confident about | text |
| The scenario that would most expose your weakness | text |
| What needs to happen in days 26-30 to make you confident? | text |
| Your DPOs assessment of your readiness — do they agree with yours? | text |
| Agreed focus for days 26-30 | text |
| Notes | text |

### Debrief questions

1. Is your readiness assessment consistent with what your session records show — or are you being optimistic?

2. Does your DPO agree with your self-assessment — and if not who is more likely to be right?

3. What is the single most important thing you can do in days 26 to 30 to close the biggest gap?

4. If the NI Simulator Course started tomorrow what would be the first thing that tripped you up?

## Slot 2 · ops_002 — 500m Zone Entry Checklist Walk-Through

| Domain | DP Operations and Operational Decision Making |
| --- | --- |
| Competency | Operational planning and execution |
| Difficulty | 2 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP |

### NI task references

Reference

7.20

7.24

7.25

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel at 500m standoff of virtual structure. Before approaching, complete the full zone entry checklist verbally with DPO. Every item, in order, with reasoning for why each item matters.

### Execute

Work through the 500m zone entry checklist item by item. For each item state what you are checking, confirm its status, and explain why it matters before entering the 500m zone. DPO challenges any item you cannot explain. After completing the checklist verbally, execute the approach.

### Professional standard

- All checklist items covered without prompting

- Correct status confirmed for each item

- Clear reasoning given for each item

- No items skipped when challenged

### Observe and record

| Field | Type |
| --- | --- |
| Items completed without prompting | number |
| Which items required prompting? | text |
| Which items did DPO challenge? | text |
| DPO assessment of reasoning quality | text |
| Working side correctly assessed — windward or leeward? | text |
| Abort heading confirmed before approach commenced? | text |
| Notes | text |

### Debrief questions

1. Which checklist items have the highest safety significance for this specific operation?

2. What happens operationally if you enter the 500m zone without completing this checklist?

3. How does the working side assessment change your risk picture and your approach plan?

4. What SIMOPS information do you need from the installation before entering the 500m zone?

### Variants (1)

ops_002a — Zone Entry Checklist From Memory

| Change | Value |
| --- | --- |
| Checklist from memory | yes |

Complete the zone entry checklist entirely from memory — no app, no reference. This is the standard a DPO would expect on a real vessel. 14 of 16 items minimum.

## Slot 3 · jc_007 — Blind Stop Challenge

| Domain | Joystick Control and Vessel Handling |
| --- | --- |
| Competency | Stopping and speed management |
| Difficulty | 4 of 5 |
| Duration | 20 min |
| Vessel modes | JSMH |

### NI task references

Reference

7.5

7.14

7.18

### Environment

| Setting | Value |
| --- | --- |
| Value | Set by DPO — unknown to trainee |

### Setup

DPO sets a random drift duration between 3 and 8 minutes and an environment the trainee has not seen. Trainee does not know duration or conditions in advance.

### Execute

DPO starts the drift without telling the trainee how long it will run. Trainee observes the drift. When DPO gives the instruction to stop — stop the vessel. No preparation time. Residual velocity and time to stop are your score.

### Professional standard

- Residual velocity below 0.2kts

- Vessel stopped within 60 seconds of instruction

- Correct assessment of dominant environmental force stated before acting

- Heading maintained during stop

### Observe and record

| Field | Type |
| --- | --- |
| Drift direction assessed before acting | text |
| Dominant force assessed — wind, wave or current? | text |
| Speed at intervention (kts) | number |
| Residual velocity at stop (kts) | number |
| Time from instruction to stopped (seconds) | number |
| Total joystick inputs | number |
| Notes | text |

### Debrief questions

1. How did unknown conditions change your approach compared to prepared drills?

2. What information did you gather in the first 30 seconds of observation — and was it enough?

3. How does this drill simulate real operational conditions when DP fails unexpectedly?

4. What is the difference in your mental process between a prepared stop and a blind stop?

### Variants (1)

jc_007a — Blind Stop with Position Constraint

| Change | Value |
| --- | --- |
| Virtual structure active | yes |

Same blind stop but a virtual structure is active. You must stop before reaching the alert zone. Stakes are now real.

## Slot 4 · buffer_025 — buffer

Buffer slot · 60 min — Repeat weakest identified skill from review_003

# Day 26 — Full Operation Without Prompting

120 min across 1 slot · target 120 min

## Slot 1 · ops_001 — Full DP Operation — End to End

| Domain | DP Operations and Operational Decision Making |
| --- | --- |
| Competency | Operational planning and execution |
| Difficulty | 5 of 5 |
| Duration | 120 min |
| Vessel modes | Off → All modes as required |

### NI task references

Reference

7.1

7.4

7.5

7.20

7.24

7.25

8.1

8.2

8.5

8.6

10.1

10.2

2.5

2.6

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Cold screen. No pre-configuration. This is the full operation with no prompting from DPO unless a genuine safety issue develops.

### Execute

Complete the following sequence without prompting. Phase 1 — Setup: Full system setup from scratch. All checklists completed. Phase 2 — Transit: Move vessel from open water to 500m standoff of virtual structure. Phase 3 — Zone entry: Complete 500m zone entry checklist verbally. Approach to operating radius on JSMH then DP. Phase 4 — Hold: Hold at operating radius for 20 minutes. DPO will trigger one alarm during hold. Phase 5 — Handover: Conduct a full watch handover to DPO. Phase 6 — Departure: Receive the watch back. Execute controlled departure. Return to open water. Save session record.

### Professional standard

- All phases completed in correct sequence without prompting

- No zone breaches during approach or hold

- Alarm during hold responded to correctly

- Handover covers all required items

- Departure executed cleanly

- Session record saved with all observations completed

### Observe and record

| Field | Type |
| --- | --- |
| Setup completion time (minutes) | number |
| 500m zone entry checklist completed without prompting? | text |
| Speed limits observed at each zone boundary? | text |
| DP capture quality at operating radius | text |
| Position footprint during 20 minute hold | text |
| Alarm during hold — identified and responded to correctly? | text |
| DPO assessment of handover quality | text |
| Departure execution — clean and controlled? | text |
| Total hard exclusion zone breaches (target zero) | number |
| Which phases if any required DPO prompting? | text |
| Notes | text |

### Debrief questions

1. Which phase was hardest and what specifically caused the difficulty?

2. What would you do differently on the next full operation?

3. How did the alarm during hold affect your situational awareness for the remainder of the operation?

4. Were your log entries and session record complete enough to reconstruct the operation from the record alone?

### Variants (2)

ops_001a — Full Operation in Heavy Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 25 |
| Wave height m | 2 |

Same full operation in heavier conditions. Every phase becomes harder. Setup and checklist discipline is even more important when the operation itself is more demanding.

ops_001b — Full Operation with Compound Failure

| Change | Value |
| --- | --- |
| Compound failure during hold | yes |

DPO will trigger a compound failure during the hold phase rather than a simple alarm. No warning. This is the assessment-level scenario — Day 29 standard.

# Day 27 — Worst Case Failure Near Structure

120 min across 3 slots · target 120 min

## Slot 1 · fail_004 — Worst Case Failure Exercise

| Domain | Failure Scenarios and Degraded Operations |
| --- | --- |
| Competency | Failure management |
| Difficulty | 5 of 5 |
| Duration | 40 min |
| Vessel modes | DP → DP |

### NI task references

Reference

11.3

11.4

11.10

11.11

11.12

### Environment

Wind 20 kt @ 037° · Sea 1.5 m @ 048° · Current 0.5 kt @ 090°

### Setup

Vessel on DP at operating radius of virtual structure. Moderate to heavy conditions. Before starting, state to your DPO what you understand to be the vessels defined worst case failure. DPO will then trigger it without further warning during the hold.

### Execute

When the WCF is triggered, observe the full system response — position excursion, alarm sequence, thruster redistribution. Record the excursion distance. Assess whether the excursion would be within safe limits for a notional subsea operation with divers at 50m depth. Make a safety assessment and communicate it clearly.

### Professional standard

- Correct pre-statement of vessel WCF

- Full alarm sequence observed and recorded

- Position excursion measured accurately

- Correct safety assessment for subsea operation context

- Clear communication of assessment

### Observe and record

| Field | Type |
| --- | --- |
| WCF stated before drill — what did you say? | text |
| What was actually triggered by DPO? | text |
| Alarm sequence — which alarms in what order? | text |
| Maximum position excursion (metres) | number |
| Direction of excursion | text |
| Time from failure to maximum excursion (seconds) | number |
| Did vessel recover or continue to excurse? | text |
| Would this excursion be safe for diving or ROV operations? Reasoning? | text |
| Notes | text |

### Debrief questions

1. What is the defined WCF for this vessel — where is it documented and how was it determined?

2. Was the excursion within acceptable limits for a notional subsea operation and what is that limit based on?

3. What is the FMEA and how does it define the WCF for this vessel class?

4. Under what environmental conditions does the WCF excursion become unacceptable for the operations this vessel typically supports?

### Variants (1)

fail_004a — WCF in Heavy Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 30 |
| Wave height m | 2.5 |

Same WCF in significantly heavier conditions. The excursion will be larger and recovery may not occur. Observe the full consequence and assess whether operations should ever be conducted in these conditions on this vessel.

## Slot 2 · alarm_005 — Drive Off Recognition and Response

| Domain | DP Alarms, Warnings and Emergency Response |
| --- | --- |
| Competency | Alarm recognition and response |
| Difficulty | 4 of 5 |
| Duration | 20 min |
| Vessel modes | DP → DP then immediate JSMH or manual |

### NI task references

Reference

11.6

11.7

11.8

11.10

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Vessel on DP near virtual structure. DPO will simulate a drive off without warning at an unannounced time.

### Execute

When drive off begins — vessel accelerating away from setpoint with thrusters at maximum in the wrong direction — recognise it, select joystick or manual immediately, and arrest the vessel movement. Record time from drive off initiation to your recognition. Record distance travelled before arrested.

### Professional standard

- Drive off recognised within 30 seconds

- Correct immediate mode selection — joystick or manual

- Vessel movement arrested

- No collision with virtual structure

- Post drive off checks completed

### Observe and record

| Field | Type |
| --- | --- |
| Time from drive off initiation to recognition (seconds) | number |
| What visual signatures indicated a drive off? | text |
| Mode selected on recognition — joystick or manual? | text |
| Distance travelled before vessel arrested (metres) | number |
| Direction of drive off — toward or away from structure? | text |
| Post drive off checks completed before returning to DP? | text |
| Notes | text |

### Debrief questions

1. What are the visual signatures of a drive off on the DP display that distinguish it from a normal position excursion?

2. Why does a drive off require immediate manual intervention rather than waiting for the DP to self-correct?

3. What is the difference between a drive off and a position excursion in terms of cause and response?

4. What checks must be completed after a drive off before you return the vessel to DP control?

### Variants (1)

alarm_005a — Drive Off Toward Structure

| Change | Value |
| --- | --- |
| Drive off direction | toward_structure |

The drive off is toward the structure not away from it. Time to collision is very short. Recognition and action within 15 seconds is the target. This is the worst case drive off scenario.

## Slot 3 · buffer_027 — buffer

Buffer slot · 60 min — Full scenario — approach plus WCF triggered at operating radius plus departure

# Day 28 — Mock Oral Examination

120 min across 3 slots · target 120 min

## Slot 1 · review_004 — Pre-Course Mock Oral Examination

| Domain | Self-Assessment and Progress Review |
| --- | --- |
| Competency | Self assessment |
| Difficulty | 4 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP — sustained hold while conducting review |

### NI task references

None recorded.

### Setup

Vessel on DP in moderate conditions. DPO conducts a verbal knowledge check while the vessel holds position.

### Execute

Hold position on DP while your DPO asks a minimum of 12 questions across system knowledge, failure responses, operational decisions, ASOG application, and watchkeeping. Answer without notes. The NI Simulator Course is a practical skills course not an examination — this is preparation for a productive learning experience, not an exam drill.

### Professional standard

- Correct answer to minimum 10 of 12 questions

- No single topic area completely blank

- Answers specific not vague

- Confidence maintained throughout even when uncertain

### Observe and record

| Field | Type |
| --- | --- |
| Questions asked by DPO — list all 12 minimum | text |
| Number of questions answered correctly | number |
| Weakest topic area identified by DPO | text |
| Strongest topic area | text |
| DPO overall assessment — ready for Simulator Course? | text |
| Topics to revise before days 29-30 | text |
| Notes | text |

### Debrief questions

1. Which topic area was weakest and what is your plan to address it before the real course?

2. Were there any questions where you knew the answer but could not articulate it clearly?

3. How did your confidence hold up under examination conditions — did uncertainty show?

4. Based on this mock examination — are you ready for the NI Simulator Course?

## Slot 2 · buffer_028a — buffer

Buffer slot · 45 min — Revise weakest topic areas from review_004

## Slot 3 · buffer_028b — buffer

Buffer slot · 45 min — Practical drill on weakest practical skill from review_004

# Day 29 — Full Assessment Operation

120 min across 1 slot · target 120 min

## Slot 1 · ops_001 — Full DP Operation — End to End

| Domain | DP Operations and Operational Decision Making |
| --- | --- |
| Competency | Operational planning and execution |
| Difficulty | 5 of 5 |
| Duration | 120 min |
| Vessel modes | Off → All modes as required |

### NI task references

Reference

7.1

7.4

7.5

7.20

7.24

7.25

8.1

8.2

8.5

8.6

10.1

10.2

2.5

2.6

### Environment

Wind 15 kt @ 037° · Sea 1 m @ 048° · Current 0.3 kt @ 090°

### Setup

Cold screen. No pre-configuration. This is the full operation with no prompting from DPO unless a genuine safety issue develops.

### Execute

Complete the following sequence without prompting. Phase 1 — Setup: Full system setup from scratch. All checklists completed. Phase 2 — Transit: Move vessel from open water to 500m standoff of virtual structure. Phase 3 — Zone entry: Complete 500m zone entry checklist verbally. Approach to operating radius on JSMH then DP. Phase 4 — Hold: Hold at operating radius for 20 minutes. DPO will trigger one alarm during hold. Phase 5 — Handover: Conduct a full watch handover to DPO. Phase 6 — Departure: Receive the watch back. Execute controlled departure. Return to open water. Save session record.

### Professional standard

- All phases completed in correct sequence without prompting

- No zone breaches during approach or hold

- Alarm during hold responded to correctly

- Handover covers all required items

- Departure executed cleanly

- Session record saved with all observations completed

### Observe and record

| Field | Type |
| --- | --- |
| Setup completion time (minutes) | number |
| 500m zone entry checklist completed without prompting? | text |
| Speed limits observed at each zone boundary? | text |
| DP capture quality at operating radius | text |
| Position footprint during 20 minute hold | text |
| Alarm during hold — identified and responded to correctly? | text |
| DPO assessment of handover quality | text |
| Departure execution — clean and controlled? | text |
| Total hard exclusion zone breaches (target zero) | number |
| Which phases if any required DPO prompting? | text |
| Notes | text |

### Debrief questions

1. Which phase was hardest and what specifically caused the difficulty?

2. What would you do differently on the next full operation?

3. How did the alarm during hold affect your situational awareness for the remainder of the operation?

4. Were your log entries and session record complete enough to reconstruct the operation from the record alone?

### Variants (2)

ops_001a — Full Operation in Heavy Weather

| Change | Value |
| --- | --- |
| Wind speed kts | 25 |
| Wave height m | 2 |

Same full operation in heavier conditions. Every phase becomes harder. Setup and checklist discipline is even more important when the operation itself is more demanding.

ops_001b — Full Operation with Compound Failure

| Change | Value |
| --- | --- |
| Compound failure during hold | yes |

DPO will trigger a compound failure during the hold phase rather than a simple alarm. No warning. This is the assessment-level scenario — Day 29 standard.

# Day 30 — DPO Sign-Off and Logbook Review

120 min across 2 slots · target 120 min

## Slot 1 · review_004 — Pre-Course Mock Oral Examination

| Domain | Self-Assessment and Progress Review |
| --- | --- |
| Competency | Self assessment |
| Difficulty | 4 of 5 |
| Duration | 30 min |
| Vessel modes | DP → DP — sustained hold while conducting review |

### NI task references

None recorded.

### Setup

Vessel on DP in moderate conditions. DPO conducts a verbal knowledge check while the vessel holds position.

### Execute

Hold position on DP while your DPO asks a minimum of 12 questions across system knowledge, failure responses, operational decisions, ASOG application, and watchkeeping. Answer without notes. The NI Simulator Course is a practical skills course not an examination — this is preparation for a productive learning experience, not an exam drill.

### Professional standard

- Correct answer to minimum 10 of 12 questions

- No single topic area completely blank

- Answers specific not vague

- Confidence maintained throughout even when uncertain

### Observe and record

| Field | Type |
| --- | --- |
| Questions asked by DPO — list all 12 minimum | text |
| Number of questions answered correctly | number |
| Weakest topic area identified by DPO | text |
| Strongest topic area | text |
| DPO overall assessment — ready for Simulator Course? | text |
| Topics to revise before days 29-30 | text |
| Notes | text |

### Debrief questions

1. Which topic area was weakest and what is your plan to address it before the real course?

2. Were there any questions where you knew the answer but could not articulate it clearly?

3. How did your confidence hold up under examination conditions — did uncertainty show?

4. Based on this mock examination — are you ready for the NI Simulator Course?

## Slot 2 · buffer_030 — buffer

Buffer slot · 90 min — Logbook review — all NI task sections reviewed with DPO. Confirm all passive-eligible sections addressed. Identify remaining active requirements.