# DPTrainer — Curriculum Review

DPTrainer is a structured session companion for Trainee Dynamic Positioning Operators undertaking Nautical Institute passive simulator days. It gives a TDPO a task to work through, a standard to meet, fields to record against, and questions to answer, for each of the 30 days the scheme allows. It is not a training provider. It does not instruct, assess, certify or sign off. The supervising DPO does all of that; DPTrainer only structures the time.

## Source documents behind the curriculum

- The Nautical Institute — DP Operator’s Certification Scheme (New Offshore Scheme), logbook task sections 1–12

- DP Operator’s Handbook, 3rd Edition — Bray, Daniels, Fiander, Foster (The Nautical Institute)

- IMCA M117 Rev. 3.3 — The Training and Experience of Key DP Personnel

- IMCA M220 Rev. 3.0 (Jan 2025) — Recommended Practice on Operational Activity Planning (ASOG, CAM/TAM, TAGOS)

- IMCA M249 Rev. 1 — DP Practitioner Accreditation Scheme Handbook

- IMCA M273 (Jan 2026) — A Guide to Conducting DP Drills and Ensuring Preparedness for DP Failures

- IMO MSC.1/Circ.1580 (Jun 2017) — Guidelines for vessels and units with dynamic positioning (DP) systems

## Scope

19 domains · 72 tasks · 30 training days

This copy covers training days 21–30 of 30.

## Reviewer instruction

Please mark anything unrealistic, missing, or wrongly placed within the 30 days. Marginal notes are more useful than a summary.

## Reviewer

| Name |  |
| --- | --- |
| Position |  |
| Organisation |  |
| Date |  |

Generated 10 September 2026

# Day 21 — Advanced Sensor Management

120 min across 4 slots · target 120 min

## Slot 1 · sen_001 — Reference System Weighting

Reference management · 20 min

### NI task references

Reference

5.1

5.2

7.9

7.10

### Setup

Two DGPS references active. Vessel stable on DP. Open the Sensors tab and observe the weighting assigned to each reference before doing anything.

### Execute

Record initial weights for both references. Ask your DPO to introduce simulated HDOP degradation on one reference. Observe how the weighting changes. Record the new weights. Note how position quality changes. Assess at what point you would manually inhibit the degraded reference rather than waiting for auto-rejection. Re-enable and confirm both references return to healthy status.

### Professional standard

- Correctly record initial reference weights

- Observe and record weight change after degradation

- Correct threshold assessment for manual inhibit

- Correct procedure for re-enabling recovered reference

### Debrief questions

1. How does the DP Kalman filter decide how much weight to assign each reference — what factors influence this?

2. What HDOP value would concern you and at what value would you act?

3. At what point would you manually inhibit a reference rather than waiting for auto-rejection — and what is the risk of waiting?

4. What is the difference between a reference being auto-rejected and manually inhibited — and does it matter which way it happens?

[1 variant omitted from this edition]

## Slot 2 · sen_002 — GPS Failure Response

Reference management · 20 min

### NI task references

Reference

5.1

5.2

7.9

11.9

### Setup

Two DGPS references active. Vessel stable on DP in moderate conditions. Note current position quality.

### Execute

Without warning your DPO will disable one GPS reference. Observe the alarm sequence. Record which alarms triggered and in what order. Assess your remaining reference configuration. Decide whether to continue or go off DP. Re-enable the failed GPS and confirm re-acceptance procedure.

### Professional standard

- Alarm acknowledged within 15 seconds

- Correct assessment of remaining reference configuration

- Correct go or no-go decision with reasoning

- Correct re-acceptance procedure for recovered reference

### Debrief questions

1. What is the minimum reference configuration for your vessel's typical operations and where is this defined?

2. Why does losing one GPS not automatically mean you must go off DP?

3. What changed about your position quality with only one reference active?

4. When re-enabling a recovered reference — why must you confirm it is healthy before accepting it back into the solution?

[2 variants omitted from this edition]

## Slot 3 · sen_003 — Gyro Failure Response

Sensor management · 20 min

### NI task references

Reference

6.1

7.8

11.9

### Setup

Both gyros active. Note heading agreement between gyro 1 and gyro 2 — should be within 0.5 degrees. Vessel stable on DP.

### Execute

Without warning your DPO will introduce a 5 degree heading error on one gyro. Observe how the DP responds. Does it auto-reject? Does heading control degrade? Record the heading excursion. Manually inhibit the faulty gyro. Observe recovery. Record time from fault introduction to stable heading.

### Professional standard

- Identify faulty gyro within 30 seconds

- Correct inhibit procedure executed

- Recovery confirmed before continuing

- Time from fault to stable heading recorded

### Debrief questions

1. How does the DP decide which gyro to trust when two gyros disagree — what is the voting logic?

2. What heading excursion is acceptable during a gyro failure response near a structure?

3. Why is gyro failure more immediately dangerous to vessel position than a GPS failure?

4. How does heading error propagate into position error — explain the mechanism?

[1 variant omitted from this edition]

## Slot 4 · buffer_021 — buffer

Buffer slot · 60 min — Combined sensor degradation scenario — reference plus gyro

# Day 22 — Compound Failure Scenarios

120 min across 4 slots · target 120 min

## Slot 1 · fail_001 — Single Thruster Failure Near Structure

Failure management · 30 min

### NI task references

Reference

11.3

11.10

11.12

### Setup

Vessel on DP at operating radius of virtual structure. Moderate conditions settled. DPO will trigger a single thruster failure at an unannounced time during the hold.

### Execute

When the thruster failure occurs, hold position with remaining thrusters. Identify the faulted thruster. Assess new worst case failure. Assess remaining capability for current conditions. Decide whether to maintain position or increase standoff distance. Communicate your decision and reasoning to DPO as if communicating to OIM.

### Professional standard

- Position maintained within double the normal footprint

- Faulted thruster identified within 30 seconds

- Correct new worst case failure identified

- Correct capability assessment for current conditions

- Clear reasoned decision with communication

### Debrief questions

1. How did your worst case failure change when one thruster was lost — and why does that matter?

2. Was your decision to hold position or increase standoff correct given remaining capability and conditions?

3. What would the ASOG say about continuing operations with this degraded thruster complement?

4. What is your new abort threshold with reduced thrusters — at what environmental condition would you now go off DP?

[2 variants omitted from this edition]

## Slot 2 · fail_002 — Power Failure Response

Failure management · 30 min

### NI task references

Reference

11.3

11.11

11.12

### Setup

Vessel on DP. DPO will trigger a generator dropout on one bus without warning. Know your vessel power configuration — which thrusters are on which bus — before this drill.

### Execute

When the generator dropout occurs, observe which thrusters lose power. Observe auto load shedding. Assess remaining propulsion capability. Prevent blackout. Either hold position or initiate controlled departure. Communicate to DPO as OIM.

### Professional standard

- Correct identification of thrusters lost on faulted bus

- Auto load shedding observed and understood

- Blackout prevention demonstrated

- Correct hold or depart decision

- Clear communication of situation

### Debrief questions

1. Which thrusters lost power and does the bus configuration of your vessel explain why those ones specifically?

2. What is the role of blackout prevention in this scenario — what would happen without it?

3. Why does the bus tie position matter so much in a power failure scenario?

4. What is the worst case failure for this vessel and how does this event relate to it — was this the WCF or something different?

[1 variant omitted from this edition]

## Slot 3 · ta_003 — Mini Trials Programme

Trials programme design and record · 45 min

### NI task references

Reference

1.1

1.1.4

1.3.3

7.20

### Setup

Design a three-test programme covering one power system test, one thruster or propulsion test, and one reference or sensor test. Write a full test sheet for each, then sequence them deliberately — decide which runs first and why. Prepare a single trials record header: date, session, environment, vessel configuration.

### Execute

Run the programme in your planned sequence. Between tests: reinstate, verify restoration, and allow full stabilisation before the next test begins. Log actual start and finish times per test, results against expected, and any deviation from your written method the moment it happens. Close out the record with a summary of findings and any items you would carry forward to a future session.

### Professional standard

- Programme fully written and sequenced before the first test

- Restoration verified — not assumed — before each subsequent test

- Deviations from written method logged at the time, not afterwards

- Record complete enough that another TDPO could repeat the programme without asking a single question

### Debrief questions

1. What logic did you use to sequence the three tests, and what would go wrong if you ran them in reverse order?

2. Why must restoration be verified rather than assumed before the next test starts?

3. What makes a trials record repeatable, and why does repeatability matter more than neatness?

4. In real annual trials, the trials document exists in a pre-trials revision written before anyone steps aboard. Why is the authorship done in advance?

5. Which of your three tests would you promote into a recurring personal drill, and why that one?

## Slot 4 · buffer_022 — buffer

Buffer slot · 15 min — Compound failure — thruster loss plus reference degradation simultaneously

# Day 23 — Heavy Weather Approaches and Compound Failures

120 min across 3 slots · target 120 min

## Slot 1 · approach_003 — Extended Hold at Operating Radius

Approach and proximity · 30 min

### NI task references

Reference

8.5

8.6

9.1

9.2

### Setup

Position vessel at DP operating radius of virtual structure. Moderate conditions initially. DPO will ramp wind gradually during the hold.

### Execute

Hold position at the DP operating radius for 20 minutes. During the hold your DPO will gradually increase wind from 15kts to 30kts. Record position footprint at 15, 20, 25, and 30kts. Record thruster demand at each interval. Identify the point at which you would recommend moving further from structure or reviewing operations with the OIM.

### Professional standard

- Position maintained throughout hold

- Correct identification of degrading margin

- Reasoned recommendation point with evidence

- Correct communication chain identified

### Debrief questions

1. At what wind speed did your position footprint become unacceptable at this operating radius?

2. What is the ASOG concept and how would a properly defined ASOG have defined your action limit here?

3. Who do you inform and in what order when you identify a deteriorating situation during a hold?

4. What is the difference between moving further from the structure and going fully off DP — when would you choose each?

[1 variant omitted from this edition]

## Slot 2 · fail_003 — Compound Failure — Reference Loss During Approach

Failure management · 35 min

### NI task references

Reference

11.3

11.9

11.12

### Setup

Begin an active approach to virtual structure from 500m. DPO will trigger reference loss at an unannounced point during the approach — inside the 500m zone.

### Execute

Continue the approach normally. When reference loss occurs inside the 500m zone, make an immediate decision — continue approach on remaining references or abort. If continuing, hold position and reassess. If aborting, execute a controlled departure. There is no correct answer predetermined — your decision must be reasoned and clearly communicated.

### Professional standard

- Immediate recognition of reference loss

- Correct minimum reference assessment at moment of failure

- Clear reasoned decision within 30 seconds

- Correct execution of chosen action

- Clear communication throughout

### Debrief questions

1. What was your reference configuration at the moment of failure and was minimum configuration still met?

2. Was your decision to continue or abort correct given your proximity to structure at the moment of failure?

3. How does being inside the 500m zone change the minimum reference requirement compared to open water?

4. What would the ASOG say about this scenario — does it define a minimum reference requirement for inside 500m?

[1 variant omitted from this edition]

## Slot 3 · buffer_023 — buffer

Buffer slot · 55 min — Full scenario — approach in heavy weather with unannounced failure at operating radius

# Day 24 — Watchkeeping and Operational Decision Making

120 min across 4 slots · target 120 min

## Slot 1 · watch_003 — DP Log Entry Discipline

Watchkeeping · 20 min

### NI task references

Reference

9.12

9.13

9.14

9.15

### Setup

Vessel on DP. Paper and pen ready for log entries. DPO will trigger events during a 20-minute period without announcing what or when. Your job is to log everything correctly.

### Execute

During the 20-minute period your DPO will trigger several events — a reference quality degradation, a wind increase, a thruster loading change, and a position alert. For each event make a DP log entry in the correct format: time, event description, action taken, system status after action. DPO assesses completeness and accuracy of all entries at the end.

### Professional standard

- Every event logged within 2 minutes of occurrence

- Timestamp accurate to within 1 minute

- All required fields completed for each entry

- Action taken clearly stated for each entry

- No events missed

### Debrief questions

1. What information is mandatory in every DP log entry — what are the minimum required fields?

2. Why is timestamp accuracy critical in a DP log — what happens to your log if a DP incident occurs?

3. What constitutes an event worthy of a log entry versus routine monitoring that does not need logging?

4. Who reads your DP log entries after an incident — and how should that knowledge affect how you write them?

[1 variant omitted from this edition]

## Slot 2 · ops_004 — Risk Assessment and Toolbox Talk

Operational planning and execution · 25 min

### NI task references

Reference

7.24

7.25

9.11

9.13

### Setup

DPO describes a notional DP operation: vessel holding position near a fixed platform, ROV deployed to 50m depth, a second vessel on standby 300m away, wind 20kts and increasing. You must conduct a risk assessment and toolbox talk before the operation proceeds.

### Execute

Conduct a verbal risk assessment covering all major risk categories: environmental conditions and forecast, vessel capability and ASOG status, reference configuration and quality, SIMOPS considerations, escape route and abort criteria, communication plan, personnel briefing requirements. Then conduct a one-minute toolbox talk covering the top three risks and the actions planned for each.

### Professional standard

- All major risk categories addressed

- Specific risks identified not just categories

- Toolbox talk clear and under 2 minutes

- Top three risks correctly prioritised

- Action plan stated for each top risk

### Debrief questions

1. What risk categories did you miss and why are they significant for this type of operation?

2. What makes a toolbox talk effective versus one that people switch off during?

3. How does the risk assessment change if wind increases by 10kts during the operation?

4. Who must be present for the toolbox talk and what happens if a key person is absent?

[1 variant omitted from this edition]

## Slot 3 · watch_002 — DP Watch Handover

Watch handover · 25 min

### NI task references

Reference

2.5

2.6

2.7

### Setup

Vessel on DP in moderate conditions with a virtual structure active at 100m. A realistic handover scenario — the incoming DPO needs everything.

### Execute

Part 1 — Give handover: You are the outgoing DPO. Brief your DPO on full system status covering every checklist item. Vessel position and heading, mode and setpoint, reference systems and quality, sensor status, thruster configuration, power configuration, environmental conditions, nearby structures and working side, alarms active or recently cleared, any system degradation, operational requirements for the next watch. Part 2 — Receive handover: Swap roles. Receive a handover from your DPO. Ask all questions you would ask as an incoming DPO. Identify any gaps in what you were told.

### Professional standard

- All handover items covered without prompting

- Minimum 5 questions asked as incoming DPO

- Gaps in received handover correctly identified

- Refusal to accept an incomplete handover demonstrated

### Debrief questions

1. What items did you miss in the handover you gave — and are those items safety-critical?

2. Under what circumstances would you refuse to accept a watch handover?

3. What is the single most important piece of information in a DP watch handover?

4. How does a DP watch handover differ from a standard bridge watchkeeping handover?

[1 variant omitted from this edition]

## Slot 4 · buffer_024 — buffer

Buffer slot · 50 min — Full watch cycle — hold, log entries, alarm, handover

# Day 25 — Pre-Completion Review and Open Water Drills

120 min across 4 slots · target 120 min

## Slot 1 · review_003 — Self-Assessment — Pre-Completion Review

Self assessment · 20 min

### NI task references

None recorded.

### Setup

Vessel on DP. Run the system while you conduct this assessment.

### Execute

Hold position and reflect honestly. Are you ready for the NI Simulator Course — yes or no, with evidence. What is the one skill you are least confident about? What scenario would most expose that weakness? What is the one thing you most wish you had more time to practise? Discuss with DPO.

### Professional standard

- Honest yes or no to readiness question with evidence

- One specific skill identified as weakest

- Specific scenario identified as most exposing

- Final five sessions have a concrete plan

### Debrief questions

1. Is your readiness assessment consistent with what your session records show — or are you being optimistic?

2. Does your DPO agree with your self-assessment — and if not who is more likely to be right?

3. What is the single most important thing you can do in days 26 to 30 to close the biggest gap?

4. If the NI Simulator Course started tomorrow what would be the first thing that tripped you up?

## Slot 2 · ops_002 — 500m Zone Entry Checklist Walk-Through

Operational planning and execution · 20 min

### NI task references

Reference

7.20

7.24

7.25

### Setup

Vessel at 500m standoff of virtual structure. Before approaching, complete the full zone entry checklist verbally with DPO. Every item, in order, with reasoning for why each item matters.

### Execute

Work through the 500m zone entry checklist item by item. For each item state what you are checking, confirm its status, and explain why it matters before entering the 500m zone. DPO challenges any item you cannot explain. After completing the checklist verbally, execute the approach.

### Professional standard

- All checklist items covered without prompting

- Correct status confirmed for each item

- Clear reasoning given for each item

- No items skipped when challenged

### Debrief questions

1. Which checklist items have the highest safety significance for this specific operation?

2. What happens operationally if you enter the 500m zone without completing this checklist?

3. How does the working side assessment change your risk picture and your approach plan?

4. What SIMOPS information do you need from the installation before entering the 500m zone?

[1 variant omitted from this edition]

## Slot 3 · jc_007 — Blind Stop Challenge

Stopping and speed management · 20 min

### NI task references

Reference

7.5

7.14

7.18

### Setup

DPO sets a random drift duration between 3 and 8 minutes and an environment the trainee has not seen. Trainee does not know duration or conditions in advance.

### Execute

DPO starts the drift without telling the trainee how long it will run. Trainee observes the drift. When DPO gives the instruction to stop — stop the vessel. No preparation time. Residual velocity and time to stop are your score.

### Professional standard

- Residual velocity below 0.2kts

- Vessel stopped within 60 seconds of instruction

- Correct assessment of dominant environmental force stated before acting

- Heading maintained during stop

### Debrief questions

1. How did unknown conditions change your approach compared to prepared drills?

2. What information did you gather in the first 30 seconds of observation — and was it enough?

3. How does this drill simulate real operational conditions when DP fails unexpectedly?

4. What is the difference in your mental process between a prepared stop and a blind stop?

[1 variant omitted from this edition]

## Slot 4 · buffer_025 — buffer

Buffer slot · 60 min — Repeat weakest identified skill from review_003

# Day 26 — Full Operation Without Prompting

120 min across 1 slot · target 120 min

## Slot 1 · ops_001 — Full DP Operation — End to End

Operational planning and execution · 120 min

### NI task references

Reference

7.1

7.4

7.5

7.20

7.24

7.25

8.1

8.2

8.5

8.6

10.1

10.2

2.5

2.6

### Setup

Cold screen. No pre-configuration. This is the full operation with no prompting from DPO unless a genuine safety issue develops.

### Execute

Complete the following sequence without prompting. Phase 1 — Setup: Full system setup from scratch. All checklists completed. Phase 2 — Transit: Move vessel from open water to 500m standoff of virtual structure. Phase 3 — Zone entry: Complete 500m zone entry checklist verbally. Approach to operating radius on JSMH then DP. Phase 4 — Hold: Hold at operating radius for 20 minutes. DPO will trigger one alarm during hold. Phase 5 — Handover: Conduct a full watch handover to DPO. Phase 6 — Departure: Receive the watch back. Execute controlled departure. Return to open water. Save session record.

### Professional standard

- All phases completed in correct sequence without prompting

- No zone breaches during approach or hold

- Alarm during hold responded to correctly

- Handover covers all required items

- Departure executed cleanly

- Session record saved with all observations completed

### Debrief questions

1. Which phase was hardest and what specifically caused the difficulty?

2. What would you do differently on the next full operation?

3. How did the alarm during hold affect your situational awareness for the remainder of the operation?

4. Were your log entries and session record complete enough to reconstruct the operation from the record alone?

[2 variants omitted from this edition]

# Day 27 — Worst Case Failure Near Structure

120 min across 3 slots · target 120 min

## Slot 1 · fail_004 — Worst Case Failure Exercise

Failure management · 40 min

### NI task references

Reference

11.3

11.4

11.10

11.11

11.12

### Setup

Vessel on DP at operating radius of virtual structure. Moderate to heavy conditions. Before starting, state to your DPO what you understand to be the vessels defined worst case failure. DPO will then trigger it without further warning during the hold.

### Execute

When the WCF is triggered, observe the full system response — position excursion, alarm sequence, thruster redistribution. Record the excursion distance. Assess whether the excursion would be within safe limits for a notional subsea operation with divers at 50m depth. Make a safety assessment and communicate it clearly.

### Professional standard

- Correct pre-statement of vessel WCF

- Full alarm sequence observed and recorded

- Position excursion measured accurately

- Correct safety assessment for subsea operation context

- Clear communication of assessment

### Debrief questions

1. What is the defined WCF for this vessel — where is it documented and how was it determined?

2. Was the excursion within acceptable limits for a notional subsea operation and what is that limit based on?

3. What is the FMEA and how does it define the WCF for this vessel class?

4. Under what environmental conditions does the WCF excursion become unacceptable for the operations this vessel typically supports?

[1 variant omitted from this edition]

## Slot 2 · alarm_005 — Drive Off Recognition and Response

Alarm recognition and response · 20 min

### NI task references

Reference

11.6

11.7

11.8

11.10

### Setup

Vessel on DP near virtual structure. DPO will simulate a drive off without warning at an unannounced time.

### Execute

When drive off begins — vessel accelerating away from setpoint with thrusters at maximum in the wrong direction — recognise it, select joystick or manual immediately, and arrest the vessel movement. Record time from drive off initiation to your recognition. Record distance travelled before arrested.

### Professional standard

- Drive off recognised within 30 seconds

- Correct immediate mode selection — joystick or manual

- Vessel movement arrested

- No collision with virtual structure

- Post drive off checks completed

### Debrief questions

1. What are the visual signatures of a drive off on the DP display that distinguish it from a normal position excursion?

2. Why does a drive off require immediate manual intervention rather than waiting for the DP to self-correct?

3. What is the difference between a drive off and a position excursion in terms of cause and response?

4. What checks must be completed after a drive off before you return the vessel to DP control?

[1 variant omitted from this edition]

## Slot 3 · buffer_027 — buffer

Buffer slot · 60 min — Full scenario — approach plus WCF triggered at operating radius plus departure

# Day 28 — Mock Oral Examination

120 min across 3 slots · target 120 min

## Slot 1 · review_004 — Pre-Course Mock Oral Examination

Self assessment · 30 min

### NI task references

None recorded.

### Setup

Vessel on DP in moderate conditions. DPO conducts a verbal knowledge check while the vessel holds position.

### Execute

Hold position on DP while your DPO asks a minimum of 12 questions across system knowledge, failure responses, operational decisions, ASOG application, and watchkeeping. Answer without notes. The NI Simulator Course is a practical skills course not an examination — this is preparation for a productive learning experience, not an exam drill.

### Professional standard

- Correct answer to minimum 10 of 12 questions

- No single topic area completely blank

- Answers specific not vague

- Confidence maintained throughout even when uncertain

### Debrief questions

1. Which topic area was weakest and what is your plan to address it before the real course?

2. Were there any questions where you knew the answer but could not articulate it clearly?

3. How did your confidence hold up under examination conditions — did uncertainty show?

4. Based on this mock examination — are you ready for the NI Simulator Course?

## Slot 2 · buffer_028a — buffer

Buffer slot · 45 min — Revise weakest topic areas from review_004

## Slot 3 · buffer_028b — buffer

Buffer slot · 45 min — Practical drill on weakest practical skill from review_004

# Day 29 — Full Assessment Operation

120 min across 1 slot · target 120 min

## Slot 1 · ops_001 — Full DP Operation — End to End

Operational planning and execution · 120 min

### NI task references

Reference

7.1

7.4

7.5

7.20

7.24

7.25

8.1

8.2

8.5

8.6

10.1

10.2

2.5

2.6

### Setup

Cold screen. No pre-configuration. This is the full operation with no prompting from DPO unless a genuine safety issue develops.

### Execute

Complete the following sequence without prompting. Phase 1 — Setup: Full system setup from scratch. All checklists completed. Phase 2 — Transit: Move vessel from open water to 500m standoff of virtual structure. Phase 3 — Zone entry: Complete 500m zone entry checklist verbally. Approach to operating radius on JSMH then DP. Phase 4 — Hold: Hold at operating radius for 20 minutes. DPO will trigger one alarm during hold. Phase 5 — Handover: Conduct a full watch handover to DPO. Phase 6 — Departure: Receive the watch back. Execute controlled departure. Return to open water. Save session record.

### Professional standard

- All phases completed in correct sequence without prompting

- No zone breaches during approach or hold

- Alarm during hold responded to correctly

- Handover covers all required items

- Departure executed cleanly

- Session record saved with all observations completed

### Debrief questions

1. Which phase was hardest and what specifically caused the difficulty?

2. What would you do differently on the next full operation?

3. How did the alarm during hold affect your situational awareness for the remainder of the operation?

4. Were your log entries and session record complete enough to reconstruct the operation from the record alone?

[2 variants omitted from this edition]

# Day 30 — DPO Sign-Off and Logbook Review

120 min across 2 slots · target 120 min

## Slot 1 · review_004 — Pre-Course Mock Oral Examination

Self assessment · 30 min

### NI task references

None recorded.

### Setup

Vessel on DP in moderate conditions. DPO conducts a verbal knowledge check while the vessel holds position.

### Execute

Hold position on DP while your DPO asks a minimum of 12 questions across system knowledge, failure responses, operational decisions, ASOG application, and watchkeeping. Answer without notes. The NI Simulator Course is a practical skills course not an examination — this is preparation for a productive learning experience, not an exam drill.

### Professional standard

- Correct answer to minimum 10 of 12 questions

- No single topic area completely blank

- Answers specific not vague

- Confidence maintained throughout even when uncertain

### Debrief questions

1. Which topic area was weakest and what is your plan to address it before the real course?

2. Were there any questions where you knew the answer but could not articulate it clearly?

3. How did your confidence hold up under examination conditions — did uncertainty show?

4. Based on this mock examination — are you ready for the NI Simulator Course?

## Slot 2 · buffer_030 — buffer

Buffer slot · 90 min — Logbook review — all NI task sections reviewed with DPO. Confirm all passive-eligible sections addressed. Identify remaining active requirements.