# DPTrainer — Curriculum Review

DPTrainer is a structured session companion for Trainee Dynamic Positioning Operators undertaking Nautical Institute passive simulator days. It gives a TDPO a task to work through, a standard to meet, fields to record against, and questions to answer, for each of the 30 days the scheme allows. It is not a training provider. It does not instruct, assess, certify or sign off. The supervising DPO does all of that; DPTrainer only structures the time.

## Source documents behind the curriculum

- The Nautical Institute — DP Operator’s Certification Scheme (New Offshore Scheme), logbook task sections 1–12

- DP Operator’s Handbook, 3rd Edition — Bray, Daniels, Fiander, Foster (The Nautical Institute)

- IMCA M117 Rev. 3.3 — The Training and Experience of Key DP Personnel

- IMCA M220 Rev. 3.0 (Jan 2025) — Recommended Practice on Operational Activity Planning (ASOG, CAM/TAM, TAGOS)

- IMCA M249 Rev. 1 — DP Practitioner Accreditation Scheme Handbook

- IMCA M273 (Jan 2026) — A Guide to Conducting DP Drills and Ensuring Preparedness for DP Failures

- IMO MSC.1/Circ.1580 (Jun 2017) — Guidelines for vessels and units with dynamic positioning (DP) systems

## Scope

19 domains · 72 tasks · 30 training days

## Reviewer instruction

Please mark anything unrealistic, missing, or wrongly placed within the 30 days. Marginal notes are more useful than a summary.

## Reviewer

| Name |  |
| --- | --- |
| Position |  |
| Organisation |  |
| Date |  |

Generated 10 September 2026

# Reference vessel

Every model answer in this document is written against the vessel below. It is a generic arrangement, not any particular ship.

| Vessel | Generic DP2 PSV |
| --- | --- |
| Heading | 000° True unless a task states otherwise |
| Thrusters | Two bow tunnel, one stern tunnel, two main azimuth aft |
| Power | Two switchboards, bus tie normally open, each board carrying one bow tunnel and one main |
| References | Two DGPS |
| Sensors | Two gyros, two wind, two VRU |

Model answers are written against this reference vessel. Actual values will differ by vessel and conditions — the reasoning is what matters.

# Day 1 — System Setup and Basic Vessel Control

120 min across 5 slots · target 120 min

## Slot 1 · setup_001 — Full System Setup From Scratch

System initialisation · 20 min

### NI task references

Reference

1.1

1.2

4.12

### Setup

Cold screen. No pre-configuration. Start from the beginning.

### Execute

Complete the full simulator setup procedure without using any guide. Enter real vessel position from GPS display, heading from gyrocompass, draft from stability display, water depth from echosounder. Confirm all sensors active and healthy. Confirm all thrusters enabled. Set environment to calm. Run a 3-minute steady state check — vessel must hold position with minimal thruster activity before you declare setup complete.

### Professional standard

- All fields correctly populated from live vessel data

- All sensors showing green — no rejections or warnings

- All thrusters enabled and showing in schematic

- Vessel stable after 3-minute steady state check

- Setup completed within 15 minutes

### Debrief questions

1. What would happen to the vessel model if you entered the wrong water depth — how does an incorrect depth affect the simulation?

Depth feeds the current and shallow-water model. Enter a depth much greater than actual and the model under-estimates the current force and the sluggish shallow-water response, so the controller's feed-forward is wrong before anything else happens: the vessel sits with a steady offset the DPO cannot explain, and thruster demand does not match what the sea state suggests it should be. Too shallow and the model over-reacts. The point is that the whole simulation is only as good as what was typed in — a wrong depth is not a small error, it invalidates the exercise.

2. Why must the initial heading entered match the actual vessel heading precisely — what goes wrong if it does not?

The controller resolves thruster demand into vessel axes using heading. If the entered heading is wrong by even a few degrees, the model's idea of where the bow is points somewhere the vessel is not, so surge demand leaks into sway and the vessel walks off track while the display reports it holding. On the reference vessel, heading is 000° True at setup and the entered value must match the gyro exactly. Wind feed-forward is resolved the same way, so a heading error also puts the wind correction on the wrong axis.

3. What does the 3-minute steady state check confirm — and what does it tell you if the vessel is not stable?

It confirms the vessel has reached equilibrium — that thruster output is balancing the environment and not chasing an error the DPO introduced. On the reference vessel in calm conditions expect low, steady demand on the tunnels with the mains trimming, and position and heading holding within a metre and a fraction of a degree. If it is not stable, something entered at setup is wrong or a sensor is feeding bad data: the answer is to find that cause, not to start the drill and hope it settles.

4. Which sensor is most critical for DP position keeping and why?

Position reference is what DP cannot run without — with no accepted reference the system has nothing to close the loop on and drops to a heading-and-thruster-only mode. But the honest answer names the gyro alongside it: heading is used to resolve every reference and every thruster command into vessel axes, so a gyro fault corrupts good position data as well. On the reference vessel, two DGPS and two gyros mean neither is a single point of failure, and that redundancy is the reason for the pairing.

5. If one gyro was showing a 2 degree difference from the other at setup — would you proceed? What would you do and why?

No. Two degrees is far outside the half-degree agreement expected at setup, and with only two gyros there is no third to vote on which one is right. Report it to the supervising DPO, check both against a known bearing or the vessel's heading on a reference before accepting either, and do not start a drill until the disagreement is understood. Proceeding on a wrongly-resolved heading means every observation recorded that session is against a model that was never correct.

### Expected outcome — reference vessel

| Field | Expected on reference vessel |
| --- | --- |
| Time to complete setup (minutes) | 10–15 for a competent first pass; under 10 by the end of the 30 days |
| Any items missed or incorrect on first attempt | Typically none by day 3; draft and water depth are the two most often left at default |
| Sensor status at completion — all green? | All green — both DGPS accepted with SNR above 35, both gyros within 0.5°, both wind sensors active, both VRUs healthy |
| Thruster status at completion — all enabled? | All five enabled and shown in the schematic — two bow tunnel, one stern tunnel, two main azimuth aft |
| Steady state check result — vessel stable? | Stable: position held within 1 m and heading within 0.5° over the 3 minutes, with low steady thruster demand and no rejections |
| Notes | Bus tie open with one bow tunnel and one main on each board — confirm the split before declaring setup complete |

[2 variants omitted from this edition]

## Slot 2 · jc_001 — Pure Drift Stop

Stopping and speed management · 20 min

### NI task references

Reference

7.5

7.14

### Setup

Note your exact current position on the motion page. This is your datum. You will not be returning to it in this task — the only goal is stopping.

### Execute

Select JSMH and set all joystick inputs to zero. Let the vessel drift freely for exactly 4 minutes. Do not touch any controls. When 4 minutes expires, using joystick only, bring the vessel to a complete stop. Both velocity axes must read below 0.1kts. Do not return to datum — stopping is the entire task.

### Professional standard

- Stopped within 3 deliberate joystick inputs

- Residual velocity below 0.1kts on both axes

- Heading maintained throughout — no yaw during the stop

- No overcorrection after initial input

### Debrief questions

1. Which direction did the vessel drift and which environmental force dominated — wind, wave or current? How do you know?

2. At the speed your vessel was moving when you intervened, and knowing your stopping distance — at what range from a hard exclusion zone would you have needed to begin your deceleration to stop cleanly?

3. Why does stopping a vessel into the weather require different technique to stopping beam-on to it?

4. If wind increased from 15kts to 25kts mid-stop, what would change about your technique and why?

5. The DP mathematical model builds its understanding of environmental forces partly from thruster activity. After a large joystick transit, what does the model incorrectly learn — and why does selecting Monitor briefly before DP help the model reset more accurately than going straight from JSMH to DP?

[Model answers not yet written]

[4 variants omitted from this edition]

## Slot 3 · jc_002 — Controlled Surge Move

Controlled position moves · 25 min

### NI task references

Reference

7.5

7.5.1

### Setup

Confirm your exact datum position. You will return to within 2m of this position at the end of each run.

### Execute

Move the vessel exactly 100m North from datum. Stop with residual velocity below 0.1kts. Return to within 2m of original datum position. This is one run. Complete three runs.

### Professional standard

- Overshoot less than 5m on return each run

- Residual velocity below 0.1kts at stop each run

- Improvement visible between run 1 and run 3

- Heading maintained throughout

### Debrief questions

1. Did your overshoot reduce between run 1 and run 3? What specifically changed in your technique?

2. At what point in each run did you decide to begin decelerating — and in hindsight was that the right point?

3. Why is a 100m surge move (North/South) typically easier to execute cleanly than a 100m sway move (East/West) on most DP vessels? Think about thruster geometry.

4. In a real operation near a rig, what would be the consequence of a 5m overshoot versus a 15m overshoot?

[Model answers not yet written]

[4 variants omitted from this edition]

## Slot 4 · jc_003 — Controlled Sway Move

Controlled position moves · 25 min

### NI task references

Reference

7.5

7.5.1

### Setup

Confirm exact datum position. You will return to within 2m of this position at the end of each run.

### Execute

Move the vessel exactly 100m East from datum. Stop with residual velocity below 0.1kts. Return to within 2m of original datum. This is one run. Complete three runs. Compare your results to jc_002.

### Professional standard

- Overshoot less than 5m on return each run

- Residual velocity below 0.1kts at stop each run

- Improvement visible between run 1 and run 3

- Heading maintained throughout

### Debrief questions

1. Was the sway move harder or easier than the surge move in jc_002? What specifically explains the difference?

2. Which thrusters did the most work during the sway move — and why those ones based on their position on the vessel?

3. What does this tell you about the vessel's athwartship stopping authority compared to fore-aft?

4. In a real operation, when would you need to make a pure sway move rather than repositioning on DP?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 5 · buffer_001 — buffer

Buffer slot · 30 min — Additional joystick practice — repeat jc_001 or jc_002 variant

# Day 2 — Joystick Precision and Environmental Introduction

120 min across 5 slots · target 120 min

## Slot 1 · setup_002 — Checklist Discipline

System initialisation · 15 min

### NI task references

Reference

1.1.4

1.3.4

### Setup

Paper and pen only. App closed.

### Execute

Before opening the app, write down every item on the pre-session checklist from memory. All 16 items, in order, with a one-sentence explanation of why each item matters. Then open the app and compare your list. Note any items missed, any items in wrong order, any explanations that were incomplete.

### Professional standard

- Recall minimum 14 of 16 items without reference

- Correct explanation for each item recalled

- Identify own gaps accurately

### Debrief questions

1. Which checklist items have the highest immediate safety significance if skipped?

2. What is the consequence of starting a drill with a reference system misconfigured?

3. Why does the checklist require DPO presence to be confirmed before anything else — not after setup?

4. If you could only check five items due to time pressure, which five would you choose and why?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · jc_004 — Diagonal Move

Controlled position moves · 25 min

### NI task references

Reference

7.5

7.5.1

### Setup

Confirm datum. Calm conditions only for first run.

### Execute

Move the vessel 100m Northeast simultaneously — not North then East in sequence, but both axes at the same time. Stop with residual below 0.1kts on both axes. Return to datum. Three runs.

### Professional standard

- Heading maintained within 3 degrees throughout

- Both axes within 5m overshoot on return

- Residual velocity below 0.1kts at stop

- Smooth coordinated inputs — no axis fighting

### Debrief questions

1. Which axis was harder to control simultaneously and what caused the difficulty?

2. Did your heading drift during the move — what caused it and how did you correct?

3. In what real operational scenario would a diagonal move be required rather than sequential axis moves?

4. How does the DP thrust allocation algorithm handle simultaneous surge and sway demand — does it prioritise one axis?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · env_001 — Environment Introduction — Progressive Build

Environmental awareness · 30 min

### NI task references

Reference

7.13

7.17

### Setup

Vessel on DP in calm conditions. Record baseline thruster output for all units — this is your zero reference. Before adding each stage, write down your prediction of which thrusters will increase and by how much.

### Execute

Add environmental conditions in three stages. Stage 1: wind 8kts at 037 degrees. Let settle 3 minutes. Record thruster response. Compare to prediction. Stage 2: add current 0.3kts at 090 degrees. Let settle 3 minutes. Record changes. Compare to prediction. Stage 3: add wave 1.0m at 048 degrees. Let settle 3 minutes. Record changes. Compare to prediction.

### Professional standard

- Accurate prediction of which thrusters increase before each stage

- Correct identification of dominant force at each stage

- Explanation of why specific thrusters responded

- Current vector reading compared to set conditions

### Debrief questions

1. Which thruster increased most when you added wind — and does its position on the vessel explain why?

2. Why did the current vector reading change when you added actual current, even though the tidal current was already there?

3. What is the combined environmental force vector direction given wind from 037, wave from 048, and current from 090?

4. How does wave loading differ from wind loading in terms of how the DP responds to it?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · jc_005 — Heading Change at Controlled Rate

Heading control · 20 min

### NI task references

Reference

7.5

7.12

### Setup

Note current heading. This is your start heading. Your target is current heading plus 45 degrees.

### Execute

Change vessel heading by 45 degrees at no more than 2 degrees per minute. Stop exactly on the new heading. Record position excursion during the turn. Then return to original heading at 5 degrees per minute. Record excursion on return.

### Professional standard

- Rate of turn does not exceed 2 degrees per minute on slow turn

- Position excursion under 5m during turn

- Stop within 1 degree of target heading

- Heading maintained on return

### Debrief questions

1. Why did position keeping degrade during the heading change even though you were on DP auto position?

2. Why would you limit rate of turn to 2 degrees per minute near a structure?

3. What happens to thruster allocation geometry as the vessel rotates — which thrusters change role?

4. When would you use a fast heading change versus a slow controlled turn operationally?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 5 · buffer_002 — buffer

Buffer slot · 30 min — Repeat env_001 with different heading to compare thruster response

# Day 3 — Mode Transitions and Environmental Loading

150 min across 6 slots · target 150 min

## Slot 1 · mode_003 — JSAH vs JSMH Comparison

Mode transitions · 20 min

### NI task references

Reference

7.4

7.5

### Setup

Vessel on DP at datum. Moderate conditions settled.

### Execute

Part 1 — Surge move in JSMH: Select JSMH and make a controlled 100m surge move North. Record heading drift during the move. Return to datum. Part 2 — Same surge move in JSAH: Select JSAH and repeat the identical move. Record heading drift. Compare. Part 3 — Heading change in each mode: Attempt a 20 degree heading change in JSMH, then in JSAH. Compare the experience and result.

### Professional standard

- Correctly identify heading behaviour difference between modes

- Articulate when each mode is appropriate

- Complete all three parts with observations recorded

- Correct statement of fundamental difference between modes

### Debrief questions

1. What is the fundamental difference between JSMH and JSAH in terms of what the DP system controls?

2. During the surge move in wind — which mode gave better heading control and why?

3. When would you choose JSMH over JSAH during a real approach to a structure?

4. What is the risk of using JSAH when close to a fixed structure in a cross-wind condition?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · jc_001 — Pure Drift Stop

Stopping and speed management · 20 min

### NI task references

Reference

7.5

7.14

### Setup

Note your exact current position on the motion page. This is your datum. You will not be returning to it in this task — the only goal is stopping.

### Execute

Select JSMH and set all joystick inputs to zero. Let the vessel drift freely for exactly 4 minutes. Do not touch any controls. When 4 minutes expires, using joystick only, bring the vessel to a complete stop. Both velocity axes must read below 0.1kts. Do not return to datum — stopping is the entire task.

### Professional standard

- Stopped within 3 deliberate joystick inputs

- Residual velocity below 0.1kts on both axes

- Heading maintained throughout — no yaw during the stop

- No overcorrection after initial input

### Debrief questions

1. Which direction did the vessel drift and which environmental force dominated — wind, wave or current? How do you know?

2. At the speed your vessel was moving when you intervened, and knowing your stopping distance — at what range from a hard exclusion zone would you have needed to begin your deceleration to stop cleanly?

3. Why does stopping a vessel into the weather require different technique to stopping beam-on to it?

4. If wind increased from 15kts to 25kts mid-stop, what would change about your technique and why?

5. The DP mathematical model builds its understanding of environmental forces partly from thruster activity. After a large joystick transit, what does the model incorrectly learn — and why does selecting Monitor briefly before DP help the model reset more accurately than going straight from JSMH to DP?

[Model answers not yet written]

[4 variants omitted from this edition]

## Slot 3 · env_002 — Minimum Power Heading

Environmental awareness · 25 min

### NI task references

Reference

7.12

7.13

### Setup

Set environment as specified. Allow 5 minutes to settle. Record total thruster demand at current heading — this is your baseline.

### Execute

Test four different headings spaced 45 degrees apart. Hold each heading for 3 minutes and record total thruster demand. Find the heading that gives minimum total demand. Hold that heading for 5 minutes and confirm it is stable.

### Professional standard

- Test minimum four headings systematically

- Correctly identify minimum power heading

- Achieve measurable demand reduction from starting heading

- Hold minimum power heading stably for 5 minutes

### Debrief questions

1. What is the minimum power heading you found and why does that specific heading minimise demand?

2. How does this concept apply to real operations where your heading is partly constrained by the work being done?

3. What is weathervaning and when would you use Minimum Power mode instead of holding a fixed heading?

4. How would the minimum power heading change if the current direction shifted by 45 degrees?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · jc_006 — Emergency Stop from Speed

Stopping and speed management · 20 min

### NI task references

Reference

7.5

7.14

### Setup

Moderate conditions set. Vessel on DP at datum. Identify a clear stopping lane with no constraints.

### Execute

Deliberately build the vessel to 1.0kt forward speed using joystick. Then stop as quickly and cleanly as possible. Record stopping distance. Reset to datum. Repeat three times — try to improve stopping distance each run.

### Professional standard

- Stop within 30m from 1.0kt in moderate conditions

- Residual velocity below 0.1kts at stop

- No heading change greater than 5 degrees during stop

- Improvement between run 1 and run 3

### Debrief questions

1. What was your stopping distance from 1.0kt? Is that a safe margin at 100m from a hard exclusion zone?

2. What specific technique change reduced your stopping distance between runs?

3. How does heading relative to wind affect stopping efficiency — is it easier to stop into the weather or with it?

4. At what speed would you consider the vessel out of safe control in a DP operational context near structure?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 5 · act_001 — ASOG Traffic Light Interpretation

Decision support interpretation · 30 min

### NI task references

Reference

7.21

7.23

7.24

### Setup

Vessel settled on DP holding position at a safe open-water datum with the model built; the vessel remains on a sustained DP hold while the scenarios are worked at the desk. TDPO reviews the vessel's actual ASOG table (or a representative excerpt) prior to the session. DPO or supervisor prepares 3-4 short written scenarios describing a fault, alarm, or environmental change.

### Execute

For each scenario presented, the TDPO must: (1) classify the correct DP status colour, (2) state who must be notified and by when, (3) state the required response action, and (4) explain the reasoning using the ASOG's own criteria rather than guesswork. Throughout, the TDPO references the live thruster loading, position footprint and environmental readings on the DP desk when justifying each answer.

### Professional standard

- Correctly classifies status for at least 3 of 4 scenarios without prompting

- Does not confuse Blue (advisory, operations continue) with Yellow (degraded, prepare to suspend)

- Correctly identifies that a change to Blue does NOT require ceasing ongoing DP operations

- States the notification chain accurately for the vessel's own management structure

### Debrief questions

1. What is the practical difference between the Blue and Yellow response — why does Yellow require preparing to suspend operations while Blue does not?

2. If the DP system generates a consequence-analysis drift-off warning, which status does that typically trigger, and why does that differ from an ordinary equipment alarm?

3. The operation must not resume after a Yellow status until redundancy is regained — who makes that call on your vessel, and what evidence would they want to see first?

4. Can you think of a fault on your vessel's actual configuration that would immediately jump to Red rather than passing through Yellow first? Why does it skip a step?

[Model answers not yet written]

[3 variants omitted from this edition]

## Slot 6 · ds_001 — Model-Build and Settling-Time Observation

System understanding and model behaviour · 35 min

### NI task references

Reference

7.16

7.17

4.12

### Setup

Have the vessel stationary or slow-moving under manual (JSMH) control. Note the exact time on the DP desk clock before you do anything else.

### Execute

Select DP. Do not touch any controls once selected. Watch the position and heading trend on the DP display for the full settling period — typically up to 30 minutes on older systems, sometimes 10-20 on newer ones. Record how the excursions reduce over time and note the point at which the vessel visibly stops hunting and settles to a steady residual.

### Professional standard

- No manual intervention once DP was selected

- Correctly identified the point the model appeared to settle, not just when the timer ran out

- Can explain in own words why a freshly-engaged DP system behaves like a helmsman just taking the wheel

- Logged actual settling time, not the textbook figure

### Debrief questions

1. The DP model takes time to adapt to the vessel and the present environment, in the same way a helmsman takes a few minutes to get the feel of a vessel after taking the wheel. Why would selecting DP straight after a large heading change give you a worse settling period than selecting DP from steady, stationary conditions?

2. Some newer systems have shortened this settling period from 30 minutes to as little as 10. What does a DPO gain from that, and what might a DPO lose by relying on it during a safety-critical operation?

3. Your captain's technique after a large JSMH transit — slow to near zero, brief Monitor selection, then DP — relies on this same model-build principle. In your own words, why does that sequence give the model cleaner data to build from than going straight from JSMH to DP?

4. If you had watched this same exercise in deteriorating weather instead of steady conditions, what would you expect to look different about the settling trace?

[Model answers not yet written]

[2 variants omitted from this edition]

# Day 4 — Model Management and Mode Transitions

120 min across 5 slots · target 120 min

## Slot 1 · mode_001 — Clean Mode Transition — JSMH to Monitor to DP

Mode transitions · 20 min

### NI task references

Reference

7.4

7.16

### Setup

Vessel on DP at datum. Moderate environment settled. Note current vector value before starting.

### Execute

Three runs. Each run: select JSMH with zero inputs and allow vessel to drift for 2 minutes. Then slow to near zero. Select Monitor briefly. Select DP. Record capture quality. Vary the vessel speed at the Monitor transition between runs — Run 1 transition at 0.3kts, Run 2 at 0.1kts, Run 3 at near zero. Compare capture quality across all three runs.

### Professional standard

- Cleanest capture when vessel nearest zero at Monitor transition

- Correct explanation of what Monitor does to the model

- Improvement in capture quality from Run 1 to Run 3

- Time to stable position reducing each run

### Debrief questions

1. Why did DP capture quality improve when the vessel was closer to zero speed at the Monitor transition?

2. What does Monitor actually do to the DP mathematical model — is it a hard reset or something else?

3. How long did Monitor need to be active before DP capture improved — what does this tell you about the model?

4. When would you use this JSMH to Monitor to DP sequence in a real operational scenario?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · env_005 — Model Corruption and Recovery

Model behaviour · 25 min

### NI task references

Reference

7.16

7.17

### Setup

Moderate environment set and settled. Vessel on DP at datum. Note current vector value before starting.

### Execute

Run 1 — Corrupted transition: Make an aggressive 300m JSMH transit with large inputs and abrupt stops. Immediately select DP. Record thruster behaviour and current vector for 2 minutes. Note time to settle. Run 2 — Clean transition: Reset. Make the same 300m transit but slow to near zero before selecting Monitor briefly, then DP. Record settle time and capture quality. Compare both runs.

### Professional standard

- Observe measurable difference in capture quality between runs

- Correctly identify current vector corruption after aggressive transit

- Explain the mechanism of model corruption

- Record settle time difference between methods

### Debrief questions

1. What did the current vector show immediately after the aggressive transit and what caused that reading?

2. Why does slowing to near zero before the Monitor transition improve DP capture quality?

3. What does the Kalman filter inherit from a large JSMH transit — and why is that a problem?

4. How long did the model take to settle in each method — and what explains the difference in time?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · mode_002 — Minimum Power Mode Observation

Mode transitions · 20 min

### NI task references

Reference

7.12

7.15

### Setup

Vessel on DP holding a fixed heading. Record current heading and total thruster demand as baseline.

### Execute

Select Minimum Power mode. Observe as the vessel weathervanes toward the minimum resistance heading. Record what heading it settles on. Compare this to the minimum power heading you found manually in env_002 — do they agree? Hold the settled heading for 10 minutes and record demand reduction achieved compared to the fixed heading baseline.

### Professional standard

- Correctly observe and record weathervane behaviour

- Compare settled heading to env_002 result

- Record quantified demand reduction

- Understand operational limitations of this mode

### Debrief questions

1. Did Minimum Power mode find the same heading you identified manually in env_002 — and if not what explains the difference?

2. When would you use Minimum Power mode operationally and when would you not use it?

3. What are the risks of selecting Minimum Power mode when near a structure?

4. How does weathervaning affect your vessel position relative to any nearby installation or subsea asset?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · env_004 — Current Vector Analysis

Environmental awareness · 20 min

### NI task references

Reference

7.13

7.17

### Setup

Set calm wind and wave. Add only current at 0.5kts from 090 degrees. Allow 10 minutes for model to fully settle before recording anything.

### Execute

After 10 minutes, record the current vector displayed by the DP system. Compare the displayed value to the actual current you set. Then add wind at 15kts from 180 degrees — directly opposing current. Observe how the displayed current vector changes over 5 minutes. Record the new value. Explain the difference.

### Professional standard

- Correctly record displayed vs set current values

- Observe and record change when wind added

- Correctly explain why displayed current changed when wind was added

- Understand that DP current vector represents all unmodelled forces

### Debrief questions

1. Did the displayed current vector match what you set in the simulation? If not — why not and what does this tell you?

2. When you added wind, why did the current vector change even though the tidal current did not change?

3. What does the DP current vector actually represent — is it purely tidal current?

4. How does this understanding affect how you interpret the current vector during a real operation?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 5 · buffer_004 — buffer

Buffer slot · 35 min — Repeat mode_001 after longer JSMH transit

# Day 5 — Foundation Review and Emergency Stopping

170 min across 6 slots · target 170 min

## Slot 1 · jc_006 — Emergency Stop from Speed

Stopping and speed management · 20 min

### NI task references

Reference

7.5

7.14

### Setup

Moderate conditions set. Vessel on DP at datum. Identify a clear stopping lane with no constraints.

### Execute

Deliberately build the vessel to 1.0kt forward speed using joystick. Then stop as quickly and cleanly as possible. Record stopping distance. Reset to datum. Repeat three times — try to improve stopping distance each run.

### Professional standard

- Stop within 30m from 1.0kt in moderate conditions

- Residual velocity below 0.1kts at stop

- No heading change greater than 5 degrees during stop

- Improvement between run 1 and run 3

### Debrief questions

1. What was your stopping distance from 1.0kt? Is that a safe margin at 100m from a hard exclusion zone?

2. What specific technique change reduced your stopping distance between runs?

3. How does heading relative to wind affect stopping efficiency — is it easier to stop into the weather or with it?

4. At what speed would you consider the vessel out of safe control in a DP operational context near structure?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 2 · env_003 — Environmental Ramp to Saturation

Capability and limits · 30 min

### NI task references

Reference

7.13

7.17

7.21

### Setup

Vessel on DP in calm conditions. Enable Ramp mode for wind and wave. Set wind target 35kts at 037 degrees, wave target 3.0m at 048 degrees.

### Execute

Start the ramp and allow conditions to increase gradually. Monitor thruster loading continuously. Record at what wind speed the first thruster reaches saturation. Record at what point position footprint begins to degrade. Record at what point you would recommend to the OIM that operations should be reviewed.

### Professional standard

- Identify first saturation before position degrades

- Correct identification of which thruster saturates first

- Reasoned recommendation point with evidence

- Understanding of relationship between saturation and position accuracy

### Debrief questions

1. Which thruster saturated first and does its position on the vessel explain why it was first?

2. At what sea state would you recommend the OIM consider changing heading or suspending operations on this vessel?

3. What is a capability plot and how would it have predicted what you just observed in this exercise?

4. What is the relationship between thruster saturation and position keeping accuracy — why does one cause the other?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · review_001 — Self-Assessment — End of Foundation Phase

Self assessment · 20 min

### NI task references

None recorded.

### Setup

Vessel on DP in calm conditions. Let the system run while you reflect.

### Execute

Hold position on DP. While the vessel holds, answer the following from memory without reference to notes. What have you learned in sessions 1 to 5 that surprised you? What skills feel genuinely confident? What feels uncertain? What questions remain unanswered? Discuss with your DPO at the end of the hold.

### Professional standard

- Honest assessment — not optimistic, not falsely modest

- Specific skills named not vague categories

- Genuine unanswered questions identified

- Agreed focus areas written down before ending the session

### Debrief questions

1. Are the skills you listed as confident ones you could demonstrate right now without preparation?

2. Are the skills you listed as weak ones you have actually practised or just read about?

3. Which of your unanswered questions is most important to resolve before the next phase?

4. What would a senior DPO say if they watched your best session from days 1 to 5?

[Model answers not yet written]

## Slot 4 · act_002 — CAM vs TAM Configuration Recognition

Decision support interpretation · 30 min

### NI task references

Reference

7.21

7.22

7.24

### Setup

Vessel settled on DP holding position; the hold is sustained while the exercise is worked at the desk. TDPO reviews the vessel's CAM (and TAM, if the vessel operates one) configuration table. DPO presents a description of a planned activity (e.g. within 500m of a structure, IRM work, standby duty outside the zone).

### Execute

For each activity described, TDPO must state whether CAM or TAM applies, justify it against the vessel's documented criteria (not general assumption), and identify the minimum thruster/generator configuration required. When justifying each answer, the TDPO references the live thruster loading, position footprint and environmental readings, comparing the running configuration against the minimum the table requires.

### Professional standard

- Correctly defaults to CAM unless a documented, signed-off risk assessment permits TAM

- Does not treat TAM as merely 'a lower-effort mode' — can state the specific risk-acceptance basis

- Correctly identifies minimum generator/thruster count from the vessel's own CAM/TAM table

- Recognises that TAM does not remove the requirement for mitigating controls

### Debrief questions

1. Why should CAM be the default, and TAM the exception requiring sign-off, rather than the other way round?

2. What's the difference between 'the vessel can lose position in TAM and that's accepted' versus 'TAM removes the need for controls' — why is that distinction important?

3. On your vessel, is there more than one CAM depending on which industrial mission is being performed? If so, what changes between them?

4. If you were only given the CAM/TAM table with no further context, could you tell from the table alone what triggers a return to CAM from TAM?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 5 · ds_002 — Gain and Footprint Comparison

Power precision tradeoff · 40 min

### NI task references

Reference

7.15

8.5

7.17

### Setup

With the vessel settled on DP in the given environment, select low gain and allow ten minutes for the system to settle into that setting before recording anything.

### Execute

Record footprint (position scatter) and average thruster load over a ten-minute period on low gain. Without changing the environment, switch to high gain, allow ten minutes to bed in, and record the same two measures again.

### Professional standard

- Environment held constant between the two recording periods

- Allowed a full settling period after each gain change before recording

- Could state, in figures, the trade-off between footprint tightness and thrust/fuel cost

- Correctly distinguished relaxed gain from simple low gain when asked

### Debrief questions

1. Which gain setting held the tighter footprint, and what did that cost you in thrust and, by extension, fuel? Describe a real operation where paying that cost would clearly be worth it, and one where it wouldn't.

2. Relaxed gain lets the vessel drift freely within a set radius before the system intervenes at all. How is that functionally different from simply running on low gain?

3. If you were holding station 15m from a structure for a lift, which setting would you choose, and why would you accept the higher fuel burn that comes with it?

4. Under relaxed gain, power only increases once the vessel nears the edge of the set circle. What would tell you, before any alarm fires, that the vessel is approaching that edge?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 6 · drill_001 — Auto/Manual Human Intervention Familiarisation

Failure response procedural · 30 min

### NI task references

Reference

7.19

11.10

1.5

### Setup

Vessel settled on auto DP in a safe open area of the simulated environment, no proximity hazards, no risk of unsafe position excursion.

### Execute

One piece of running equipment at a time (thruster, engine, pump, controller) is switched to local/manual control by the supervising DPO while the TDPO watches the DP control screens. Before each switch, TDPO predicts aloud what should happen. After each switch, TDPO records what actually happened, then the DPO reinstates the equipment.

### Professional standard

- Predicts correctly, before the action, whether DP control will be affected

- Correctly identifies which alarms should appear and does appear

- Does not freeze or hesitate when an unexpected effect occurs — verbalises what is being observed

- Can state afterwards which of the tested items rely on operator intervention as part of the vessel's redundancy concept

### Debrief questions

1. Were all the effects you observed ones you actually understood, or were some a surprise even after seeing them happen?

2. What should your response be, as a watchkeeper, if you saw someone switch a running thruster to local without warning you first?

3. Discuss the alternative actions available in response to an unplanned local-control switch — is there one correct response, or several defensible ones?

4. What gap in your own understanding did this drill expose that you'd want to close before doing this for real?

[Model answers not yet written]

[2 variants omitted from this edition]

# Day 6 — Sensor Management and Reference Systems

120 min across 5 slots · target 120 min

## Slot 1 · sen_001 — Reference System Weighting

Reference management · 20 min

### NI task references

Reference

5.1

5.2

7.9

7.10

### Setup

Two DGPS references active. Vessel stable on DP. Open the Sensors tab and observe the weighting assigned to each reference before doing anything.

### Execute

Record initial weights for both references. Ask your DPO to introduce simulated HDOP degradation on one reference. Observe how the weighting changes. Record the new weights. Note how position quality changes. Assess at what point you would manually inhibit the degraded reference rather than waiting for auto-rejection. Re-enable and confirm both references return to healthy status.

### Professional standard

- Correctly record initial reference weights

- Observe and record weight change after degradation

- Correct threshold assessment for manual inhibit

- Correct procedure for re-enabling recovered reference

### Debrief questions

1. How does the DP Kalman filter decide how much weight to assign each reference — what factors influence this?

2. What HDOP value would concern you and at what value would you act?

3. At what point would you manually inhibit a reference rather than waiting for auto-rejection — and what is the risk of waiting?

4. What is the difference between a reference being auto-rejected and manually inhibited — and does it matter which way it happens?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · sen_004 — Wind Feedforward Disable

Sensor management · 20 min

### NI task references

Reference

6.3

7.8

### Setup

Wind 20kts with ramp variation to simulate gusting. Vessel stable on DP with wind feedforward enabled. Record position footprint and thruster activity as baseline.

### Execute

With feedforward enabled, observe and record position footprint size and thruster response character during gusting for 3 minutes. Then disable wind feedforward. Observe the change in position keeping. Record how the position footprint changes and how thruster response changes from anticipatory to reactive. Re-enable feedforward and compare recovery.

### Professional standard

- Observe and record quantifiable difference in position footprint

- Correctly characterise thruster response as anticipatory vs reactive

- Understand the source of feedforward data

- Correct explanation of why feedforward improves gusty condition performance

### Debrief questions

1. What specifically changed about the thruster response pattern when feedforward was disabled?

2. Why does feedforward give better position keeping in gusty conditions compared to feedback alone?

3. What is the source of wind feedforward data — and what happens to position keeping if that sensor fails mid-operation?

4. In what sea conditions would losing wind feedforward be most operationally significant?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · env_004 — Current Vector Analysis

Environmental awareness · 20 min

### NI task references

Reference

7.13

7.17

### Setup

Set calm wind and wave. Add only current at 0.5kts from 090 degrees. Allow 10 minutes for model to fully settle before recording anything.

### Execute

After 10 minutes, record the current vector displayed by the DP system. Compare the displayed value to the actual current you set. Then add wind at 15kts from 180 degrees — directly opposing current. Observe how the displayed current vector changes over 5 minutes. Record the new value. Explain the difference.

### Professional standard

- Correctly record displayed vs set current values

- Observe and record change when wind added

- Correctly explain why displayed current changed when wind was added

- Understand that DP current vector represents all unmodelled forces

### Debrief questions

1. Did the displayed current vector match what you set in the simulation? If not — why not and what does this tell you?

2. When you added wind, why did the current vector change even though the tidal current did not change?

3. What does the DP current vector actually represent — is it purely tidal current?

4. How does this understanding affect how you interpret the current vector during a real operation?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · sen_002 — GPS Failure Response

Reference management · 20 min

### NI task references

Reference

5.1

5.2

7.9

### Setup

Two DGPS references active. Vessel stable on DP in moderate conditions. Note current position quality.

### Execute

Without warning your DPO will disable one GPS reference. Observe the alarm sequence. Record which alarms triggered and in what order. Assess your remaining reference configuration. Decide whether to continue or go off DP. Re-enable the failed GPS and confirm re-acceptance procedure.

### Professional standard

- Alarm acknowledged within 15 seconds

- Correct assessment of remaining reference configuration

- Correct go or no-go decision with reasoning

- Correct re-acceptance procedure for recovered reference

### Debrief questions

1. What is the minimum reference configuration for your vessel's typical operations and where is this defined?

2. Why does losing one GPS not automatically mean you must go off DP?

3. What changed about your position quality with only one reference active?

4. When re-enabling a recovered reference — why must you confirm it is healthy before accepting it back into the solution?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 5 · buffer_006 — buffer

Buffer slot · 40 min — Sensor observation — document all reference system details for this vessel

# Day 7 — Drift Assessment and Recovery

120 min across 5 slots · target 120 min

## Slot 1 · jc_001 — Pure Drift Stop

Stopping and speed management · 20 min

### NI task references

Reference

7.5

7.14

7.18

### Setup

Note your exact current position on the motion page. This is your datum. You will not be returning to it in this task — the only goal is stopping.

### Execute

Select JSMH and set all joystick inputs to zero. Let the vessel drift freely for exactly 4 minutes. Do not touch any controls. When 4 minutes expires, using joystick only, bring the vessel to a complete stop. Both velocity axes must read below 0.1kts. Do not return to datum — stopping is the entire task.

### Professional standard

- Stopped within 3 deliberate joystick inputs

- Residual velocity below 0.1kts on both axes

- Heading maintained throughout — no yaw during the stop

- No overcorrection after initial input

### Debrief questions

1. Which direction did the vessel drift and which environmental force dominated — wind, wave or current? How do you know?

2. At the speed your vessel was moving when you intervened, and knowing your stopping distance — at what range from a hard exclusion zone would you have needed to begin your deceleration to stop cleanly?

3. Why does stopping a vessel into the weather require different technique to stopping beam-on to it?

4. If wind increased from 15kts to 25kts mid-stop, what would change about your technique and why?

5. The DP mathematical model builds its understanding of environmental forces partly from thruster activity. After a large joystick transit, what does the model incorrectly learn — and why does selecting Monitor briefly before DP help the model reset more accurately than going straight from JSMH to DP?

[Model answers not yet written]

[4 variants omitted from this edition]

## Slot 2 · jc_001 — Pure Drift Stop

Stopping and speed management · 20 min

### NI task references

Reference

7.5

7.14

7.18

### Setup

Note your exact current position on the motion page. This is your datum. You will not be returning to it in this task — the only goal is stopping.

### Execute

Select JSMH and set all joystick inputs to zero. Let the vessel drift freely for exactly 4 minutes. Do not touch any controls. When 4 minutes expires, using joystick only, bring the vessel to a complete stop. Both velocity axes must read below 0.1kts. Do not return to datum — stopping is the entire task.

### Professional standard

- Stopped within 3 deliberate joystick inputs

- Residual velocity below 0.1kts on both axes

- Heading maintained throughout — no yaw during the stop

- No overcorrection after initial input

### Debrief questions

1. Which direction did the vessel drift and which environmental force dominated — wind, wave or current? How do you know?

2. At the speed your vessel was moving when you intervened, and knowing your stopping distance — at what range from a hard exclusion zone would you have needed to begin your deceleration to stop cleanly?

3. Why does stopping a vessel into the weather require different technique to stopping beam-on to it?

4. If wind increased from 15kts to 25kts mid-stop, what would change about your technique and why?

5. The DP mathematical model builds its understanding of environmental forces partly from thruster activity. After a large joystick transit, what does the model incorrectly learn — and why does selecting Monitor briefly before DP help the model reset more accurately than going straight from JSMH to DP?

[Model answers not yet written]

[4 variants omitted from this edition]

## Slot 3 · jc_001 — Pure Drift Stop

Stopping and speed management · 20 min

### NI task references

Reference

7.5

7.14

7.18

### Setup

Note your exact current position on the motion page. This is your datum. You will not be returning to it in this task — the only goal is stopping.

### Execute

Select JSMH and set all joystick inputs to zero. Let the vessel drift freely for exactly 4 minutes. Do not touch any controls. When 4 minutes expires, using joystick only, bring the vessel to a complete stop. Both velocity axes must read below 0.1kts. Do not return to datum — stopping is the entire task.

### Professional standard

- Stopped within 3 deliberate joystick inputs

- Residual velocity below 0.1kts on both axes

- Heading maintained throughout — no yaw during the stop

- No overcorrection after initial input

### Debrief questions

1. Which direction did the vessel drift and which environmental force dominated — wind, wave or current? How do you know?

2. At the speed your vessel was moving when you intervened, and knowing your stopping distance — at what range from a hard exclusion zone would you have needed to begin your deceleration to stop cleanly?

3. Why does stopping a vessel into the weather require different technique to stopping beam-on to it?

4. If wind increased from 15kts to 25kts mid-stop, what would change about your technique and why?

5. The DP mathematical model builds its understanding of environmental forces partly from thruster activity. After a large joystick transit, what does the model incorrectly learn — and why does selecting Monitor briefly before DP help the model reset more accurately than going straight from JSMH to DP?

[Model answers not yet written]

[4 variants omitted from this edition]

## Slot 4 · sen_003 — Gyro Failure Response

Sensor management · 20 min

### NI task references

Reference

6.1

7.8

### Setup

Both gyros active. Note heading agreement between gyro 1 and gyro 2 — should be within 0.5 degrees. Vessel stable on DP.

### Execute

Without warning your DPO will introduce a 5 degree heading error on one gyro. Observe how the DP responds. Does it auto-reject? Does heading control degrade? Record the heading excursion. Manually inhibit the faulty gyro. Observe recovery. Record time from fault introduction to stable heading.

### Professional standard

- Identify faulty gyro within 30 seconds

- Correct inhibit procedure executed

- Recovery confirmed before continuing

- Time from fault to stable heading recorded

### Debrief questions

1. How does the DP decide which gyro to trust when two gyros disagree — what is the voting logic?

2. What heading excursion is acceptable during a gyro failure response near a structure?

3. Why is gyro failure more immediately dangerous to vessel position than a GPS failure?

4. How does heading error propagate into position error — explain the mechanism?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 5 · buffer_007 — buffer

Buffer slot · 40 min — Drift stop with position constraint — stop within 50m of datum

# Day 8 — First Approach to Structure

170 min across 6 slots · target 170 min

## Slot 1 · ops_002 — 500m Zone Entry Checklist Walk-Through

Operational planning and execution · 20 min

### NI task references

Reference

7.20

7.24

7.25

### Setup

Vessel at 500m standoff of virtual structure. Before approaching, complete the full zone entry checklist verbally with DPO. Every item, in order, with reasoning for why each item matters.

### Execute

Work through the 500m zone entry checklist item by item. For each item state what you are checking, confirm its status, and explain why it matters before entering the 500m zone. DPO challenges any item you cannot explain. After completing the checklist verbally, execute the approach.

### Professional standard

- All checklist items covered without prompting

- Correct status confirmed for each item

- Clear reasoning given for each item

- No items skipped when challenged

### Debrief questions

1. Which checklist items have the highest safety significance for this specific operation?

2. What happens operationally if you enter the 500m zone without completing this checklist?

3. How does the working side assessment change your risk picture and your approach plan?

4. What SIMOPS information do you need from the installation before entering the 500m zone?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · approach_001 — Leeward Approach to Structure

Approach and proximity · 35 min

### NI task references

Reference

8.1

8.2

8.3

8.4

8.5

8.6

### Setup

Place virtual structure 500m on 037 degrees True from your current position. You are on the leeward side — the dominant environmental force is pushing you away from the structure. Confirm your abort heading before starting. Write it down.

### Execute

Enter the 500m zone on a heading of approximately 037 degrees True. Reduce to maximum 0.5kts by the 500m boundary. Reduce to maximum 0.3kts by the 200m boundary. Reduce to maximum 0.1kts by the 100m boundary. Transition from JSMH to DP at 100m. Hold at the DP operating radius for 10 minutes. Record approach track, speed at each zone boundary, and DP capture quality.

### Professional standard

- No hard exclusion zone breach

- Speed below 0.5kts at 500m boundary

- Speed below 0.3kts at 200m boundary

- Speed below 0.1kts at 100m boundary

- DP capture within operating radius

- Abort heading confirmed before entry

### Debrief questions

1. This was a leeward approach — what is the failure mode if DP fails while you are at the operating radius?

2. At what point during the approach was the environment helping your deceleration and at what point was it not?

3. What is your abort heading from the operating radius and how long would it take to clear the 500m zone?

4. How did thruster demand change as you closed distance to the structure during the hold?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 3 · approach_004 — Controlled Departure from Structure

Approach and proximity · 20 min

### NI task references

Reference

10.1

10.2

10.3

### Setup

Vessel on DP at operating radius of virtual structure. Moderate conditions. This task follows an approach drill — do not reset between approach and departure.

### Execute

Plan and execute a controlled departure. Select JSMH and move to the 500m boundary at appropriate speed — no more than 0.5kts. Once clear of 500m zone, return to open water DP. Complete the departure checklist verbally with your DPO. Record the mode transition point and speed management throughout.

### Professional standard

- Speed below 0.5kts throughout departure

- Mode change to JSMH executed at appropriate time

- 500m zone cleared cleanly

- Departure checklist completed verbally

- Navigation equipment status confirmed on clearing zone

### Debrief questions

1. Why must you manoeuvre well clear of the structure before changing mode during departure?

2. The NI checklist states 1.5 to 2.5 ship lengths before mode change depending on drift-on or drift-off — which applied here and why?

3. How does the drift-on vs drift-off situation affect the urgency and direction of your departure?

4. What navigation equipment must be restored to correct status once clear of the 500m zone?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · act_003 — Personal ASOG Quick-Reference Build

Decision support application · 35 min

### NI task references

Reference

7.20

7.21

7.24

### Setup

TDPO has access to the vessel's full ASOG document for the operational area currently relevant (e.g. 500m zone entry near a fixed structure).

### Execute

TDPO condenses the ASOG's Green/Blue/Yellow/Red criteria for the systems most relevant to the current activity into a single-page quick-reference card, in their own words. DPO or supervisor reviews it for accuracy and gaps.

### Professional standard

- Quick-reference is accurate against the source document, not just plausible-sounding

- Covers the specific systems relevant to the current operation, not a generic copy of the whole ASOG

- Correctly separates 'what triggers the status change' from 'what the response action is'

- TDPO can explain any abbreviation or threshold used on their own card without referring back to the source

### Debrief questions

1. What did you leave off the card, and was that a deliberate scoping decision or an oversight?

2. If a new crew member picked up your card cold, would they be able to use it correctly, or does it rely on knowledge you already had in your head?

3. Which threshold on the card surprised you most once you looked at the actual numbers — thruster loading, heading excursion, position excursion?

[Model answers not yet written]

## Slot 5 · er_003 — DP Event Investigation, Classification and Report

Event classification and reporting · 30 min

### NI task references

Reference

1.3.5

1.3.3

11.4

### Setup

Arrange with the supervising DPO for one degradation to be injected at some point during a 15-minute hold — a reference deselection, a sensor deviation, a thruster deselection — without telling you the time or the nature. If working alone, use the simulator's scenario or failure injection function on a delay. Have the three IMCA station keeping event category definitions written in front of you: DP incident (total loss of DP capability), DP undesired event (loss of redundancy and/or compromised capability), DP observation (neither, but worth sharing). Start a written timeline with the session start time as entry one.

### Execute

Hold position. When the event occurs, respond to it correctly and log your actions with times as you go. Once the vessel is stable, reconstruct the full event timeline from the DP alarm and event log, compare it against your own handwritten timeline, classify the event into one of the three IMCA categories, identify the initiating event as distinct from its consequences, and complete a draft station keeping event report containing: timeline, initiating event, consequences, response actions taken, and classification with one sentence of justification.

### Professional standard

- Handwritten timeline captures the event onset within 30 seconds of the system log time

- First correct response action taken before turning to documentation — vessel first, paperwork second

- Initiating event and consequences separated correctly in the report — the alarm you saw first is often not the initiating event

- Classification defensible against the written definitions, stated in one sentence

- Report complete enough that a DPO who was not present could reconstruct the event from it alone

### Debrief questions

1. Walk through why you chose the classification you did — and what single change to the scenario would have pushed it into the category above or below?

2. What is the difference between the initiating event and the first alarm you noticed, and why do reports that confuse the two mislead the industry?

3. Why does the IMCA scheme publish events anonymously, and what reporting behaviours does that anonymity exist to protect against?

4. How does an initiating event differ from a root cause, and which one does your report identify?

5. On your vessel, who does a station keeping event report go to, and what should happen to it after that?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 6 · ds_003 — Capability Plot vs Reality Cross-Check

Critical use of decision support tools · 30 min

### NI task references

Reference

7.21

7.17

9.15

### Setup

With the vessel settled on DP, open the live DP capability plot for the present conditions.

### Execute

Record what the plotted capability envelope shows for the current heading and wind. Separately, read the actual thrust percentage being used by each thruster from the propulsion readout. Compare the two.

### Professional standard

- Recorded actual per-thruster load, not just an overall average

- Correctly identified whether the plotted envelope and actual load were consistent

- Could explain what the capability plot does NOT account for

- Knew the difference between Live and Theoretical modes without being told

### Debrief questions

1. Capability plots always assume full power is available. If a generator were offline right now, would the plot in front of you reflect that? What could go wrong if a DPO trusted the plot without checking against actual thrust loads?

2. How would you independently verify whether today's capability plot is trustworthy, using only the real thrust readings on your desk?

3. The plot has Live and Theoretical modes. When would a DPO deliberately switch to Theoretical during passive time rather than just reading Live?

4. If your actual thrust usage on the critical heading is already close to the plotted limit in benign weather, what does that tell you about your reserve capacity if weather deteriorates?

[Model answers not yet written]

# Day 9 — Windward Approach and Environmental Awareness

125 min across 4 slots · target 125 min

## Slot 1 · approach_002 — Windward Approach to Structure

Approach and proximity · 35 min

### NI task references

Reference

8.1

8.2

8.3

8.4

8.5

8.6

### Setup

Place virtual structure 500m on 217 degrees True from your current position. You are now on the windward side — the dominant environmental force is pushing you toward the structure. Confirm your abort heading before starting. This is more complex on the windward side — think carefully before writing it down.

### Execute

Same speed profile as approach_001. Enter at 217 degrees True, reduce speed at each zone boundary, transition to DP at 100m, hold for 10 minutes. Pay particular attention to how the environment affects your closure rate and thruster demand compared to the leeward approach.

### Professional standard

- No hard exclusion zone breach

- All speed limits observed at each boundary

- Abort heading correctly identified for windward side

- Recognition of increased risk compared to leeward approach

- Higher thruster demand noted and recorded

### Debrief questions

1. What is the failure mode on the windward side compared to the leeward side — why is windward higher risk?

2. How much higher was thruster demand during the windward approach and hold compared to leeward?

3. Why is the abort heading more complex to plan on the windward side?

4. Under what operational circumstances would you accept a windward approach and what additional safeguards would you put in place?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · env_005 — Model Corruption and Recovery

Model behaviour · 25 min

### NI task references

Reference

7.16

7.17

### Setup

Moderate environment set and settled. Vessel on DP at datum. Note current vector value before starting.

### Execute

Run 1 — Corrupted transition: Make an aggressive 300m JSMH transit with large inputs and abrupt stops. Immediately select DP. Record thruster behaviour and current vector for 2 minutes. Note time to settle. Run 2 — Clean transition: Reset. Make the same 300m transit but slow to near zero before selecting Monitor briefly, then DP. Record settle time and capture quality. Compare both runs.

### Professional standard

- Observe measurable difference in capture quality between runs

- Correctly identify current vector corruption after aggressive transit

- Explain the mechanism of model corruption

- Record settle time difference between methods

### Debrief questions

1. What did the current vector show immediately after the aggressive transit and what caused that reading?

2. Why does slowing to near zero before the Monitor transition improve DP capture quality?

3. What does the Kalman filter inherit from a large JSMH transit — and why is that a problem?

4. How long did the model take to settle in each method — and what explains the difference in time?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · watch_001 — Sustained Position Keeping Observation

Watchkeeping · 30 min

### NI task references

Reference

9.1

9.2

9.3

9.4

9.5

### Setup

Vessel on DP in moderate conditions. DPO will gradually increase environmental conditions during the observation period. Your role is monitor and assess — do not intervene unless a genuine safety issue develops.

### Execute

Monitor the DP system holding position for 30 minutes while conditions increase. Record position footprint, thruster demand, reference quality, and current vector at 5-minute intervals. Identify any trends. Assess at what point intervention would be required. At each 5-minute mark write your observation before looking at the previous entry — do not let previous readings anchor your assessment.

### Professional standard

- All parameters recorded at every 5-minute interval without prompting

- Trends correctly identified from data

- Correct intervention threshold identified with reasoning

- Assessment does not simply mirror the previous reading

### Debrief questions

1. What trends did you identify across the 30-minute period — were conditions genuinely deteriorating or just varying?

2. At what point would you have recommended intervention and what specific evidence supported that recommendation?

3. What is the difference between active watchkeeping and passive observation — are you doing one or both?

4. What DP log entries would you have made during this 30-minute period?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · drill_002 — Loss of Seawater Cooling Pump Redundancy

Failure response procedural · 35 min

### NI task references

Reference

11.3

11.12

4.4

### Setup

Vessel in full auto DP, power plant configured per the vessel's DP operations manual for the current DP class. Simulated location and activity agreed and stated aloud before starting.

### Execute

Supervising DPO simulates loss of a duty seawater pump. TDPO observes the standby pump start and confirms alarms and equipment temperatures remain within normal limits. Test is then repeated with the standby pump isolated, so the TDPO can observe what happens when the redundancy itself is unavailable.

### Professional standard

- Correctly distinguishes the two scenarios: standby available vs standby isolated

- Notices and reports any DP equipment temperature trend, not just the immediate alarm

- Recognises which system components are on which side of the redundancy split

- Communicates the fault and status clearly rather than working silently through it

### Debrief questions

1. What risks arise from 'multi-tasking' during DP operations that could delay noticing a scenario like this — radio traffic, deck operations, routine paperwork?

2. Following this drill, would your response change if you faced the same fault for real, given what you now know about the vessel's reaction?

3. How is the seawater system's segregation related to the vessel's redundancy groups, and how would you check that on your own vessel's diagrams?

4. What would you look for on the operator station specifically to distinguish 'duty pump failed, standby running fine' from 'both pumps degraded'?

[Model answers not yet written]

# Day 10 — Extended Hold and Alarm Introduction

160 min across 6 slots · target 160 min

## Slot 1 · approach_003 — Extended Hold at Operating Radius

Approach and proximity · 30 min

### NI task references

Reference

8.5

8.6

9.1

9.2

### Setup

Position vessel at DP operating radius of virtual structure. Moderate conditions initially. DPO will ramp wind gradually during the hold.

### Execute

Hold position at the DP operating radius for 20 minutes. During the hold your DPO will gradually increase wind from 15kts to 30kts. Record position footprint at 15, 20, 25, and 30kts. Record thruster demand at each interval. Identify the point at which you would recommend moving further from structure or reviewing operations with the OIM.

### Professional standard

- Position maintained throughout hold

- Correct identification of degrading margin

- Reasoned recommendation point with evidence

- Correct communication chain identified

### Debrief questions

1. At what wind speed did your position footprint become unacceptable at this operating radius?

2. What is the ASOG concept and how would a properly defined ASOG have defined your action limit here?

3. Who do you inform and in what order when you identify a deteriorating situation during a hold?

4. What is the difference between moving further from the structure and going fully off DP — when would you choose each?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · approach_004 — Controlled Departure from Structure

Approach and proximity · 20 min

### NI task references

Reference

10.1

10.2

10.3

### Setup

Vessel on DP at operating radius of virtual structure. Moderate conditions. This task follows an approach drill — do not reset between approach and departure.

### Execute

Plan and execute a controlled departure. Select JSMH and move to the 500m boundary at appropriate speed — no more than 0.5kts. Once clear of 500m zone, return to open water DP. Complete the departure checklist verbally with your DPO. Record the mode transition point and speed management throughout.

### Professional standard

- Speed below 0.5kts throughout departure

- Mode change to JSMH executed at appropriate time

- 500m zone cleared cleanly

- Departure checklist completed verbally

- Navigation equipment status confirmed on clearing zone

### Debrief questions

1. Why must you manoeuvre well clear of the structure before changing mode during departure?

2. The NI checklist states 1.5 to 2.5 ship lengths before mode change depending on drift-on or drift-off — which applied here and why?

3. How does the drift-on vs drift-off situation affect the urgency and direction of your departure?

4. What navigation equipment must be restored to correct status once clear of the 500m zone?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · alarm_001 — Position Deviation Alarm Response

Alarm recognition and response · 20 min

### NI task references

Reference

11.1

11.2

11.4

### Setup

Vessel on DP in moderate conditions. DPO will trigger a position deviation alarm without warning at an unannounced time. Do not anticipate — watch the display normally.

### Execute

When the alarm sounds, respond correctly. Acknowledge the alarm. Identify the cause — is it environmental, reference, or thruster related? Take appropriate action. Record your response time and the steps you took. DPO repeats with a different cause. Minimum two runs with different causes.

### Professional standard

- Alarm acknowledged within 15 seconds

- Cause identified within 60 seconds

- Correct action taken for the identified cause

- Correct escalation if required

- Time from alarm to acknowledgement recorded honestly

### Debrief questions

1. What is the first thing you check when a position deviation alarm sounds — before anything else?

2. How do you distinguish between an environmental cause and a reference cause from the DP display alone?

3. At what point does a position alert become a position action alarm and what does that change?

4. Who needs to be informed of a position action alarm and in what order?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · er_004 — Emergency Procedure Touch Drill Circuit

Touch drill procedure recall · 30 min

### NI task references

Reference

11.13

11.4

7.19

1.3.2

### Setup

Vessel settled in DP at datum and holding throughout — the touch drill runs over a live station keeping picture, not a frozen screen. Select three emergency procedures from your vessel's DP operations manual (for example: loss of a position reference, loss of a thruster, initiation of the drive off response). For each, have the written procedure available but face down. A touch drill means physically touching or pointing to each control, page, or indicator in sequence — never actuating anything. The simulator state must be identical at the end to the start.

### Execute

Round one: for each of the three procedures in turn, execute the full response as a touch drill from memory — touch each control in order, say each step aloud, and time yourself. Then turn the written procedure over and mark every step you missed, added, or performed out of sequence. Round two: repeat all three drills. Record time and error count per drill per round. Confirm at the end that nothing was actuated and the DP picture is unchanged.

### Professional standard

- Every step physically located — touched or pointed to — not just named from the chair

- Nothing actuated at any point; vessel position, heading and system state identical before and after

- Round two error count lower than round one on every drill

- Round two times faster than round one without steps being skipped to achieve it

- Missed steps identified honestly against the written procedure, not rationalised away

### Debrief questions

1. Which steps did you miss in round one, and what do they have in common — are they the steps you understand least, or the ones that feel too obvious to rehearse?

2. Why does physically touching each control build a different kind of recall than reading the procedure or reciting it?

3. In a real emergency, which of your three procedures gives you the least time to think, and does your round two time for it reflect that?

4. What is the risk of touch drilling a procedure incorrectly and never checking against the written version — and what does that say about how this task should always be run?

5. Which procedure on your vessel would you touch drill next, and why that one?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 5 · dc_001 — Pre-Drill Briefing and Expected Behaviour

Drill briefing and behaviour prediction · 30 min

### NI task references

Reference

1.3.4

2.3

7.25

11.9

### Setup

Choose a single failure you have already practiced — one position reference loss is the baseline. Write a briefing covering, in order: drill type and boundaries; purpose and the failure mode under test; current system status; the scenario step by step, stating what is done for real and what is mimicked; hazards; roles, communications, and who may stop the drill; expected behaviour — the alarms you predict, what the DP system should do, and how long stabilisation should take; and a two-to-three point summary. Deliver the briefing aloud and time it. Target five to ten minutes.

### Execute

Run the failure exactly as briefed. Respond, recover, restore. Then score your briefing against reality: how many of your predicted alarms appeared, whether the system behaved as you said it would, and whether recovery took the time you estimated.

### Professional standard

- Briefing delivered in full before any action, inside five to ten minutes

- All eight briefing sections addressed

- Expected behaviour stated in specific, checkable terms before execution

- Every mismatch between briefed and actual behaviour noted in the record

### Debrief questions

1. Why does a briefing spell out which actions are real and which are mimic-only, and what kind of incident does that distinction prevent?

2. What did predicting the alarms in advance force you to know that simply reacting to them would not?

3. Why must the authority to stop a drill be stated aloud before it starts?

4. Which of your predictions missed, and what does the miss tell you about your mental model of the system?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 6 · prs_002 — Working Near a Fixed Structure

Environmental and prs degradation awareness · 30 min

### NI task references

Reference

5.1

6.3

8.1

### Setup

Position the vessel at moderate range from a virtual platform structure, with DGPS enabled as one of the active PRS and the vessel positioned downwind of the structure.

### Execute

Close on the structure per the exercise plan while watching DGPS position quality/scatter. Note any degradation as range decreases, and compare your wind sensor reading against the wind you'd expect given the logged environment.

### Professional standard

- Detected DGPS degradation from the data, not just from an alarm

- Recognised the wind sensor reading as suspect before it caused a visible position problem

- Could state which PRS types would NOT be expected to degrade in this scenario

- Maintained a direct visual reference to the structure throughout, not just instrument watch

### Debrief questions

1. Why does a large fixed structure nearby specifically degrade DGPS, while an acoustic or laser-based PRS might be unaffected by the same proximity?

2. Working close-in on the downwind side of a platform, your wind sensor may be actively wind-shadowed rather than simply unreliable. What's the practical difference between those two failure types for a DPO trying to trust the reading?

3. What visual cues should tell you a positioning problem is developing near a structure before any instrument alarm fires?

4. If your only degraded PRS near the structure is DGPS, and you still have two other independent references online, should that change how concerned you are? Why or why not?

[Model answers not yet written]

# Day 11 — Sensor Failures and Watch Handover

120 min across 4 slots · target 120 min

## Slot 1 · sen_003 — Gyro Failure Response

Sensor management · 20 min

### NI task references

Reference

6.1

7.8

11.9

### Setup

Both gyros active. Note heading agreement between gyro 1 and gyro 2 — should be within 0.5 degrees. Vessel stable on DP.

### Execute

Without warning your DPO will introduce a 5 degree heading error on one gyro. Observe how the DP responds. Does it auto-reject? Does heading control degrade? Record the heading excursion. Manually inhibit the faulty gyro. Observe recovery. Record time from fault introduction to stable heading.

### Professional standard

- Identify faulty gyro within 30 seconds

- Correct inhibit procedure executed

- Recovery confirmed before continuing

- Time from fault to stable heading recorded

### Debrief questions

1. How does the DP decide which gyro to trust when two gyros disagree — what is the voting logic?

2. What heading excursion is acceptable during a gyro failure response near a structure?

3. Why is gyro failure more immediately dangerous to vessel position than a GPS failure?

4. How does heading error propagate into position error — explain the mechanism?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · watch_001 — Sustained Position Keeping Observation

Watchkeeping · 30 min

### NI task references

Reference

9.1

9.2

9.7

9.8

### Setup

Vessel on DP in moderate conditions. DPO will gradually increase environmental conditions during the observation period. Your role is monitor and assess — do not intervene unless a genuine safety issue develops.

### Execute

Monitor the DP system holding position for 30 minutes while conditions increase. Record position footprint, thruster demand, reference quality, and current vector at 5-minute intervals. Identify any trends. Assess at what point intervention would be required. At each 5-minute mark write your observation before looking at the previous entry — do not let previous readings anchor your assessment.

### Professional standard

- All parameters recorded at every 5-minute interval without prompting

- Trends correctly identified from data

- Correct intervention threshold identified with reasoning

- Assessment does not simply mirror the previous reading

### Debrief questions

1. What trends did you identify across the 30-minute period — were conditions genuinely deteriorating or just varying?

2. At what point would you have recommended intervention and what specific evidence supported that recommendation?

3. What is the difference between active watchkeeping and passive observation — are you doing one or both?

4. What DP log entries would you have made during this 30-minute period?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · watch_002 — DP Watch Handover

Watch handover · 25 min

### NI task references

Reference

2.5

2.6

2.7

### Setup

Vessel on DP in moderate conditions with a virtual structure active at 100m. A realistic handover scenario — the incoming DPO needs everything.

### Execute

Part 1 — Give handover: You are the outgoing DPO. Brief your DPO on full system status covering every checklist item. Vessel position and heading, mode and setpoint, reference systems and quality, sensor status, thruster configuration, power configuration, environmental conditions, nearby structures and working side, alarms active or recently cleared, any system degradation, operational requirements for the next watch. Part 2 — Receive handover: Swap roles. Receive a handover from your DPO. Ask all questions you would ask as an incoming DPO. Identify any gaps in what you were told.

### Professional standard

- All handover items covered without prompting

- Minimum 5 questions asked as incoming DPO

- Gaps in received handover correctly identified

- Refusal to accept an incomplete handover demonstrated

### Debrief questions

1. What items did you miss in the handover you gave — and are those items safety-critical?

2. Under what circumstances would you refuse to accept a watch handover?

3. What is the single most important piece of information in a DP watch handover?

4. How does a DP watch handover differ from a standard bridge watchkeeping handover?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · buffer_011 — buffer

Buffer slot · 45 min — Repeat sen_002 near structure variant

# Day 12 — Alarms and Single Failures

165 min across 7 slots · target 165 min

## Slot 1 · alarm_002 — Reference System Rejection Response

Alarm recognition and response · 20 min

### NI task references

Reference

11.2

11.3

11.9

### Setup

Two DGPS references active. Vessel stable on DP. DPO will trigger reference rejection without warning.

### Execute

Run 1 — Single rejection: DPO triggers auto-rejection of one reference. Respond correctly. Assess remaining configuration. Make go or no-go decision. Run 2 — Compound: After restoring Run 1, DPO degrades a second reference shortly after restoring the first. Assess minimum reference threshold and make go or no-go decision with reasoning.

### Professional standard

- Alarm acknowledged within 15 seconds

- Correct minimum reference assessment

- Correct go or no-go decision with clear reasoning

- Correct re-acceptance procedure

- Appropriate escalation

### Debrief questions

1. What is the difference between auto-rejection and manual inhibit — and does it matter which way a reference is removed from the solution?

2. What is the minimum reference configuration for your vessel's operations and where is this defined?

3. When re-enabling a recovered reference why must you verify it is healthy before accepting it — what is the risk of premature acceptance?

4. How does being near a structure change the minimum reference requirement compared to open water operations?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · alarm_003 — Thruster Fault Response

Alarm recognition and response · 20 min

### NI task references

Reference

11.3

11.4

11.10

### Setup

All thrusters active. Vessel on DP in moderate conditions. DPO will fault one thruster without warning.

### Execute

When thruster fault alarm sounds, identify which thruster has faulted. Assess your remaining thrust capability. Assess the new worst case failure given the reduced thruster complement. Inform the appropriate parties. DPO asks: can you continue operations? Give a reasoned answer. Repeat with a different thruster faulted.

### Professional standard

- Correct thruster identified within 30 seconds

- Correct capability assessment

- Correct new worst case failure identified

- Appropriate parties informed in correct order

- Reasoned continue or abort decision

### Debrief questions

1. How does losing this specific thruster change your worst case failure scenario?

2. Which direction is position keeping most degraded with this thruster lost — and why?

3. At what point does a thruster fault require going off DP rather than continuing with reduced capability?

4. What is the correct communication chain for a thruster fault during DP operations near a structure?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · watch_003 — DP Log Entry Discipline

Watchkeeping · 20 min

### NI task references

Reference

9.12

9.13

9.14

### Setup

Vessel on DP. Paper and pen ready for log entries. DPO will trigger events during a 20-minute period without announcing what or when. Your job is to log everything correctly.

### Execute

During the 20-minute period your DPO will trigger several events — a reference quality degradation, a wind increase, a thruster loading change, and a position alert. For each event make a DP log entry in the correct format: time, event description, action taken, system status after action. DPO assesses completeness and accuracy of all entries at the end.

### Professional standard

- Every event logged within 2 minutes of occurrence

- Timestamp accurate to within 1 minute

- All required fields completed for each entry

- Action taken clearly stated for each entry

- No events missed

### Debrief questions

1. What information is mandatory in every DP log entry — what are the minimum required fields?

2. Why is timestamp accuracy critical in a DP log — what happens to your log if a DP incident occurs?

3. What constitutes an event worthy of a log entry versus routine monitoring that does not need logging?

4. Who reads your DP log entries after an incident — and how should that knowledge affect how you write them?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · fail_001 — Single Thruster Failure Near Structure

Failure management · 30 min

### NI task references

Reference

11.3

11.4

11.10

### Setup

Vessel on DP at operating radius of virtual structure. Moderate conditions settled. DPO will trigger a single thruster failure at an unannounced time during the hold.

### Execute

When the thruster failure occurs, hold position with remaining thrusters. Identify the faulted thruster. Assess new worst case failure. Assess remaining capability for current conditions. Decide whether to maintain position or increase standoff distance. Communicate your decision and reasoning to DPO as if communicating to OIM.

### Professional standard

- Position maintained within double the normal footprint

- Faulted thruster identified within 30 seconds

- Correct new worst case failure identified

- Correct capability assessment for current conditions

- Clear reasoned decision with communication

### Debrief questions

1. How did your worst case failure change when one thruster was lost — and why does that matter?

2. Was your decision to hold position or increase standoff correct given remaining capability and conditions?

3. What would the ASOG say about continuing operations with this degraded thruster complement?

4. What is your new abort threshold with reduced thrusters — at what environmental condition would you now go off DP?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 5 · act_004 — Consequence Analysis Alarm Response

Decision support application · 25 min

### NI task references

Reference

11.5

11.4

9.11

### Setup

Applicable where the vessel's DP system has consequence analysis fitted and enabled (Class 2/3). Vessel settled on DP holding position; the hold is sustained while the scenario is worked. DPO presents a scenario where a consequence-analysis drift-off warning has been triggered.

### Execute

TDPO must state: what the warning actually means (predicted loss of position/heading if the next failure occurred now, not an actual failure), what status it should trigger per the ASOG, and what the correct operator response is — including whether it is appropriate to continue working under the alarm. While answering, the TDPO references the live thruster loading, position footprint and environmental readings to ground each judgement in the vessel's present state.

### Professional standard

- Does not confuse a consequence-analysis warning with an actual equipment failure

- Correctly states that continuing to work while the alarm is active is a Yellow-status decision requiring justification, not a default

- Can describe what would be happening in the ECR (power plant under stress) that corroborates the alarm

- States the correct action if a further failure occurred while the alarm was already active

### Debrief questions

1. Is it ever appropriate to keep working while a consequence-analysis drift-off warning is active? What would need to be true for that to be acceptable?

2. Why does the DP system give priority to heading over position when it starts shedding capability — and what does that mean practically for you as the watchkeeper?

3. How would you tell the difference between the DP system being genuinely at its limit versus a false or over-sensitive alarm?

[Model answers not yet written]

## Slot 6 · er_001 — Backup DP Station Changeover

Control station transfer · 25 min

### NI task references

Reference

4.2

3.1

11.10

7.19

### Setup

Vessel settled in DP at datum with the model built. Confirm the backup DP station is powered, available, and showing valid data. Locate your vessel's control transfer procedure and read it once before starting. Note which station currently holds command and record your settled position and heading.

### Execute

Hold position for 5 minutes and record your maximum excursion as a baseline. Then treat the primary station as unavailable: transfer command to the backup DP station following the vessel procedure step by step. Continue holding position from the backup station for 10 minutes. Record the time from decision to completed transfer, the position and heading excursion during the transfer, and every alarm generated.

### Professional standard

- Transfer completed as a continuous deliberate sequence — no backtracking through the procedure

- Command confirmed at the backup station and stated out loud (or logged) before releasing attention from the primary

- Position excursion during transfer no greater than the baseline excursion recorded in the first 5 minutes

- Heading held within 2 degrees throughout

- All alarms generated by the transfer acknowledged and understood, not just silenced

### Debrief questions

1. What checks would you make at the backup station before accepting command, and what could go wrong if you skipped them?

2. Which failure modes could take out the primary station while leaving the backup healthy? Which failures would take out both?

3. What happens to the vessel's station keeping in the seconds between the primary station being lost and command being established at the backup?

4. How would working at 50m from a structure change your decision about whether to transfer command or take the vessel on joystick first?

5. Why does DP class notation require an independent backup control station at all — what redundancy principle is it protecting?

[Model answers not yet written]

[3 variants omitted from this edition]

## Slot 7 · prs_003 — Equipment Class and Worst-Case Failure Identification

Redundancy concept application · 25 min

### NI task references

Reference

11.3

11.12

3.1

### Setup

Using the vessel's own FMEA or class documentation where available (or the exercise's provided vessel specification), identify the assigned equipment class and the worst-case failure it's designed to survive without losing position.

### Execute

With the vessel settled on DP, talk through what would actually happen to position-keeping if that specific worst-case failure occurred right now. Use the consequence analysis display if the system is fitted with one.

### Professional standard

- Correctly stated the vessel's equipment class from documentation, not assumption

- Could describe the worst-case failure in specific terms (which thruster group, which bus section), not just 'a failure'

- Correctly interpreted whether consequence analysis was currently showing margin or risk

- Distinguished single-fault criteria from a total, unrealistic failure of everything at once

### Debrief questions

1. In your own words: what actually separates equipment class 1, 2 and 3, in terms of the kind of failure the vessel must survive without losing position?

2. Consequence analysis continuously checks that the vessel could survive its worst-case failure right now, under present conditions. What's the practical difference in a real emergency between a vessel that has this running and one that doesn't?

3. A single inadvertent act by someone on board counts as a single fault under class 2/3 criteria. Can you think of a plausible inadvertent act on this vessel that could, on its own, cause the worst-case failure?

4. If the consequence analysis showed marginal capability on your present heading, what are your options, and which would you reach for first?

[Model answers not yet written]

[1 variant omitted from this edition]

# Day 13 — Failure Scenarios

120 min across 4 slots · target 120 min

## Slot 1 · fail_001 — Single Thruster Failure Near Structure

Failure management · 30 min

### NI task references

Reference

11.3

11.10

11.12

### Setup

Vessel on DP at operating radius of virtual structure. Moderate conditions settled. DPO will trigger a single thruster failure at an unannounced time during the hold.

### Execute

When the thruster failure occurs, hold position with remaining thrusters. Identify the faulted thruster. Assess new worst case failure. Assess remaining capability for current conditions. Decide whether to maintain position or increase standoff distance. Communicate your decision and reasoning to DPO as if communicating to OIM.

### Professional standard

- Position maintained within double the normal footprint

- Faulted thruster identified within 30 seconds

- Correct new worst case failure identified

- Correct capability assessment for current conditions

- Clear reasoned decision with communication

### Debrief questions

1. How did your worst case failure change when one thruster was lost — and why does that matter?

2. Was your decision to hold position or increase standoff correct given remaining capability and conditions?

3. What would the ASOG say about continuing operations with this degraded thruster complement?

4. What is your new abort threshold with reduced thrusters — at what environmental condition would you now go off DP?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 2 · fail_002 — Power Failure Response

Failure management · 30 min

### NI task references

Reference

11.3

11.11

11.12

### Setup

Vessel on DP. DPO will trigger a generator dropout on one bus without warning. Know your vessel power configuration — which thrusters are on which bus — before this drill.

### Execute

When the generator dropout occurs, observe which thrusters lose power. Observe auto load shedding. Assess remaining propulsion capability. Prevent blackout. Either hold position or initiate controlled departure. Communicate to DPO as OIM.

### Professional standard

- Correct identification of thrusters lost on faulted bus

- Auto load shedding observed and understood

- Blackout prevention demonstrated

- Correct hold or depart decision

- Clear communication of situation

### Debrief questions

1. Which thrusters lost power and does the bus configuration of your vessel explain why those ones specifically?

2. What is the role of blackout prevention in this scenario — what would happen without it?

3. Why does the bus tie position matter so much in a power failure scenario?

4. What is the worst case failure for this vessel and how does this event relate to it — was this the WCF or something different?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · drill_003 — Loss of Main DP System UPS

Failure response procedural · 35 min

### NI task references

Reference

3.2

4.8

11.3

### Setup

Vessel in full auto DP, power plant and equipment configured per the vessel's DP operations manual and applicable checklists.

### Execute

Supervising DPO simulates loss of a main DP system UPS by isolating its outputs. TDPO observes and reports the vessel's ability to maintain position using remaining online equipment, and identifies what specifically was lost.

### Professional standard

- Correctly identifies which DP-critical equipment was fed from the affected UPS

- Does not assume the DP system is unaffected just because position is being maintained — checks what redundancy remains

- Recognises that an internal UPS fault can sometimes prevent its own bypass from working, and knows to check for this

- Reports the fault and its implications clearly and promptly

### Debrief questions

1. What DP-specific equipment on your vessel is fed by each main UPS, and could you locate that on a single-line diagram if asked?

2. Why might an internal UPS failure prevent a bypass supply from working, even though the bypass circuit itself is intact?

3. What alarms or events specifically indicate a UPS failure rather than a downstream equipment failure?

4. How would you methodically fault-find this rather than just reacting to the first alarm you see?

[Model answers not yet written]

## Slot 4 · buffer_013 — buffer

Buffer slot · 25 min — Power failure near structure variant plus debrief

# Day 14 — Consequence Alarms and ASOG

120 min across 5 slots · target 120 min

## Slot 1 · alarm_004 — Consequence Alarm Recognition

Alarm recognition and response · 20 min

### NI task references

Reference

11.4

11.5

11.6

### Setup

Vessel on DP near virtual structure. Moderate to heavy conditions. DPO will configure conditions to trigger consequence alarm.

### Execute

When consequence alarm sounds, respond immediately. Identify the alarm type. Understand its significance. Take appropriate immediate actions. Escalate to OIM immediately. Assess abort criteria. Record everything.

### Professional standard

- Immediate recognition as highest priority alarm

- OIM informed within 60 seconds

- Abort criteria assessed immediately

- Correct understanding of what the alarm predicts

- No hesitation in escalation

### Debrief questions

1. What does a consequence alarm predict — and why is this alarm categorically different from all other DP alarms?

2. Why is OIM notification the immediate first action rather than trying to resolve the situation yourself?

3. What immediate actions follow a consequence alarm before you have spoken to the OIM?

4. Under what circumstances if any would you accept a consequence alarm and continue operations?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · ops_003 — ASOG Application

Operational decision making · 25 min

### NI task references

Reference

7.21

7.22

7.23

7.24

### Setup

DPO will present three operational scenarios. For each scenario you must state the ASOG status, the action required, and who must be informed. No reference material — knowledge only.

### Execute

Scenario 1 — Green status: Conditions and system configuration within all ASOG limits. State status and confirm operation proceeds. Scenario 2 — Yellow status: One parameter approaching its ASOG limit. State status, required action, and communication chain. Scenario 3 — Red or abort criteria: A parameter has exceeded its ASOG limit during an active operation. State status, immediate actions, and who decides whether to continue.

### Professional standard

- Correct ASOG status assessment all three scenarios

- Correct action identified for each status

- Correct communication chain for each scenario

- Correct identification of who has authority to override

### Debrief questions

1. What is the ASOG and who produces it for this vessel and operation?

2. What is the difference between a yellow and red ASOG status in terms of required actions?

3. Who has authority to continue operations outside ASOG limits and what documentation is required?

4. How does the ASOG relate to the vessel FMEA — are they the same document or different?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · approach_003 — Extended Hold at Operating Radius

Approach and proximity · 30 min

### NI task references

Reference

8.5

8.6

9.1

9.2

### Setup

Position vessel at DP operating radius of virtual structure. Moderate conditions initially. DPO will ramp wind gradually during the hold.

### Execute

Hold position at the DP operating radius for 20 minutes. During the hold your DPO will gradually increase wind from 15kts to 30kts. Record position footprint at 15, 20, 25, and 30kts. Record thruster demand at each interval. Identify the point at which you would recommend moving further from structure or reviewing operations with the OIM.

### Professional standard

- Position maintained throughout hold

- Correct identification of degrading margin

- Reasoned recommendation point with evidence

- Correct communication chain identified

### Debrief questions

1. At what wind speed did your position footprint become unacceptable at this operating radius?

2. What is the ASOG concept and how would a properly defined ASOG have defined your action limit here?

3. Who do you inform and in what order when you identify a deteriorating situation during a hold?

4. What is the difference between moving further from the structure and going fully off DP — when would you choose each?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · er_002 — Collision Threat Escape Move

Time critical escape · 20 min

### NI task references

Reference

8.6

7.13

11.12

### Setup

Vessel settled in DP at datum. Before anything else, plan your escape: assess the environment and decide, out loud or on paper, the direction you would move the vessel if a drifting vessel or object threatened your position, and whether you would use a DP position move or take the vessel on joystick. Then set a random trigger — either the supervising DPO calls it, or use a timer set between 3 and 10 minutes that you cannot see.

### Execute

Hold position normally. On the trigger, move the vessel 100m in your pre-planned escape direction as quickly as you can do it safely and in control. Record the time from trigger to first commanded input, time to reach 50m and 100m from datum, maximum speed achieved, and maximum heading deviation. Settle the vessel at the new position with residual velocity below 0.1kts.

### Professional standard

- First commanded input within 15 seconds of the trigger

- Escape direction matches the plan made in setup — no improvising a new direction under pressure

- Heading either held throughout or altered deliberately as part of the plan, not lost

- Speed built decisively but deceleration planned — vessel settled at 100m without a large overshoot

- Residual velocity below 0.1kts at the settled position

### Debrief questions

1. Why did you choose the escape direction you planned — which environmental force were you working with, and which were you accepting working against?

2. DP position move versus joystick takeover for an escape: what does each give you, what does each cost you, and what would tip your choice one way or the other?

3. At the maximum speed you reached, roughly what stopping distance did you need — and how does that number change what '100m clear' actually buys you?

4. How is this manoeuvre different from a planned departure from a worksite, in both technique and mindset?

5. If the threat direction had been the same as your planned escape direction, what would your alternative have been?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 5 · dc_002 — Observer Discipline

Drill observation and timeline recording · 25 min

### NI task references

Reference

2.2

2.3

11.3

11.12

### Setup

This task is best run with a second person — a supervising DPO or another trainee — inducing the failure while you observe. Working alone, induce a familiar single failure yourself and switch immediately into the observer role. Prepare a blank timeline sheet before starting: one column for elapsed time, one for what happened, one for what the operator did.

### Execute

From the moment the failure is applied, keep a continuous timestamped log: first alarm, alarm count, first operator action, every subsequent input, and the moment the vessel is stable again. Nothing is reconstructed afterwards — if you missed an entry, log that you missed it. Finish with maximum excursion and total recovery time.

### Professional standard

- Timeline continuous and written in real time

- Every operator action timestamped

- Missed observations admitted in the record, not backfilled

- Log legible to a third party without verbal explanation

### Debrief questions

1. Why do properly run drills assign an observer who is not the operator?

2. What does the interval between first alarm and first action tell you, and what in training or ergonomics would shorten it?

3. Which of your observations would justify a change to a checklist or a familiarisation programme?

4. Why is an honest record with gaps worth more than a tidy record written afterwards?

[Model answers not yet written]

# Day 15 — Mid Programme Review and Advanced Joystick

120 min across 5 slots · target 120 min

## Slot 1 · jc_007 — Blind Stop Challenge

Stopping and speed management · 20 min

### NI task references

Reference

7.5

7.14

7.18

### Setup

DPO sets a random drift duration between 3 and 8 minutes and an environment the trainee has not seen. Trainee does not know duration or conditions in advance.

### Execute

DPO starts the drift without telling the trainee how long it will run. Trainee observes the drift. When DPO gives the instruction to stop — stop the vessel. No preparation time. Residual velocity and time to stop are your score.

### Professional standard

- Residual velocity below 0.2kts

- Vessel stopped within 60 seconds of instruction

- Correct assessment of dominant environmental force stated before acting

- Heading maintained during stop

### Debrief questions

1. How did unknown conditions change your approach compared to prepared drills?

2. What information did you gather in the first 30 seconds of observation — and was it enough?

3. How does this drill simulate real operational conditions when DP fails unexpectedly?

4. What is the difference in your mental process between a prepared stop and a blind stop?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · alarm_005 — Drive Off Recognition and Response

Alarm recognition and response · 20 min

### NI task references

Reference

11.6

11.7

11.8

11.10

### Setup

Vessel on DP near virtual structure. DPO will simulate a drive off without warning at an unannounced time.

### Execute

When drive off begins — vessel accelerating away from setpoint with thrusters at maximum in the wrong direction — recognise it, select joystick or manual immediately, and arrest the vessel movement. Record time from drive off initiation to your recognition. Record distance travelled before arrested.

### Professional standard

- Drive off recognised within 30 seconds

- Correct immediate mode selection — joystick or manual

- Vessel movement arrested

- No collision with virtual structure

- Post drive off checks completed

### Debrief questions

1. What are the visual signatures of a drive off on the DP display that distinguish it from a normal position excursion?

2. Why does a drive off require immediate manual intervention rather than waiting for the DP to self-correct?

3. What is the difference between a drive off and a position excursion in terms of cause and response?

4. What checks must be completed after a drive off before you return the vessel to DP control?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · review_002 — Self-Assessment — Mid Programme

Self assessment · 20 min

### NI task references

None recorded.

### Setup

Vessel on DP in moderate conditions. Run a sustained hold while you reflect.

### Execute

Hold position for 20 minutes. While monitoring the system, answer the following. Has your stopping distance improved since day 1? Has your approach discipline improved? Can you set up the simulator without reference in under 15 minutes? What can you do on day 15 that you could not do on day 1? Discuss with DPO.

### Professional standard

- Measurable comparisons to day 1 performance

- Honest gap assessment relative to NI Simulator Course standard

- Specific focus areas for days 16-25 agreed

- No generic statements — specific evidence only

### Debrief questions

1. Is the improvement you identified real and measurable or do you just feel more comfortable?

2. What is the single biggest gap between your current standard and the NI Simulator Course standard?

3. If days 16 to 25 go well what will you be able to do on day 25 that you cannot do today?

4. What does your DPO think your biggest weakness is — and do you agree?

[Model answers not yet written]

## Slot 4 · ta_001 — Trials Test Sheet Discipline

Test documentation and witnessing · 30 min

### NI task references

Reference

1.1

1.1.4

6.1

11.9

### Setup

Choose a single sensor failure test — one gyro is the baseline. Before touching anything, write a formal test sheet: test objective, method steps in order, the alarms you expect, the system response you expect, and the position and heading effect you expect. The sheet is complete before the first action. This is how a trials practitioner works: expected results are committed to paper before the failure is applied, never after.

### Execute

With the model settled, execute your test sheet exactly as written. Deselect the gyro. Record every alarm as it appears, the actual system response, and the actual position and heading excursion. Against each expected item mark confirmed, not confirmed, or unexpected. Reinstate the gyro, verify the system is fully restored, and close the sheet with date, session identifier and a signature line.

### Professional standard

- Test sheet complete before execution begins

- Every expected item has an actual result recorded against it

- Deviations and unexpected events documented, not omitted

- System restored and restoration verified before the sheet is closed

### Debrief questions

1. Why must expected results be written before the test rather than reconstructed afterwards?

2. If an actual result did not match your expectation, what does that gap mean for the vessel's assurance documentation, and who would need to know on a real vessel?

3. In formal trials, the person witnessing a test cannot only be the author of the test programme. What failure of assurance does that rule protect against?

4. What is the difference between this exercise run as a trial and the same failure run as a drill — what is each one for?

5. Your test produced a measurable heading excursion. How would you decide whether that excursion is an acceptable result or a finding?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 5 · buffer_015 — buffer

Buffer slot · 30 min — Repeat weakest skill identified in review_002

# Day 16 — Compound Failures and Risk Assessment

165 min across 5 slots · target 165 min

## Slot 1 · fail_003 — Compound Failure — Reference Loss During Approach

Failure management · 35 min

### NI task references

Reference

11.3

11.9

11.12

### Setup

Begin an active approach to virtual structure from 500m. DPO will trigger reference loss at an unannounced point during the approach — inside the 500m zone.

### Execute

Continue the approach normally. When reference loss occurs inside the 500m zone, make an immediate decision — continue approach on remaining references or abort. If continuing, hold position and reassess. If aborting, execute a controlled departure. There is no correct answer predetermined — your decision must be reasoned and clearly communicated.

### Professional standard

- Immediate recognition of reference loss

- Correct minimum reference assessment at moment of failure

- Clear reasoned decision within 30 seconds

- Correct execution of chosen action

- Clear communication throughout

### Debrief questions

1. What was your reference configuration at the moment of failure and was minimum configuration still met?

2. Was your decision to continue or abort correct given your proximity to structure at the moment of failure?

3. How does being inside the 500m zone change the minimum reference requirement compared to open water?

4. What would the ASOG say about this scenario — does it define a minimum reference requirement for inside 500m?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · ops_004 — Risk Assessment and Toolbox Talk

Operational planning and execution · 25 min

### NI task references

Reference

7.24

7.25

9.11

9.13

### Setup

DPO describes a notional DP operation: vessel holding position near a fixed platform, ROV deployed to 50m depth, a second vessel on standby 300m away, wind 20kts and increasing. You must conduct a risk assessment and toolbox talk before the operation proceeds.

### Execute

Conduct a verbal risk assessment covering all major risk categories: environmental conditions and forecast, vessel capability and ASOG status, reference configuration and quality, SIMOPS considerations, escape route and abort criteria, communication plan, personnel briefing requirements. Then conduct a one-minute toolbox talk covering the top three risks and the actions planned for each.

### Professional standard

- All major risk categories addressed

- Specific risks identified not just categories

- Toolbox talk clear and under 2 minutes

- Top three risks correctly prioritised

- Action plan stated for each top risk

### Debrief questions

1. What risk categories did you miss and why are they significant for this type of operation?

2. What makes a toolbox talk effective versus one that people switch off during?

3. How does the risk assessment change if wind increases by 10kts during the operation?

4. Who must be present for the toolbox talk and what happens if a key person is absent?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · drill_004 — Loss of All Position References

Failure response procedural · 35 min

### NI task references

Reference

11.6

11.9

5.1

### Setup

Vessel on full auto DP, mathematical model allowed to build and stabilise for at least 30 minutes before the drill begins. Environmental conditions noted beforehand.

### Execute

Supervising DPO deselects all Position Reference Systems from DP control. TDPO monitors the vessel's dead-reckoning behaviour using an independent position measuring method (waypoint range/bearing, or UTM northings/eastings) at one-minute intervals for 5 minutes, and reports the drift observed.

### Professional standard

- Correctly sets up an independent monitoring method before the PRSs are deselected, not after

- Recognises the DP system is now on dead-reckoning, not "no control"

- Position change of no more than roughly 20m over 5 minutes is treated as the expected benchmark, and deviation from that prompts further investigation

- Can discuss what should happen next given the vessel's Time to Safely Terminate for the current activity

### Debrief questions

1. This drill connects directly to your earlier drift-and-recovery drill design — how does dead-reckoning behaviour here compare to what you saw during a genuine drift-off?

2. Given your vessel's Time to Safely Terminate, at what point after losing all references would you need to have already started terminating the operation?

3. How would this scenario play out differently in benign versus deteriorating weather?

4. What's the very first practical action you'd take, before touching any controls, on realising all references were lost?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · dc_003 — Structured Debrief and Drill Record

Post drill debrief and record keeping · 30 min

### NI task references

Reference

1.3.3

1.3.4

1.3.5

7.24

### Setup

Run any familiar single-failure drill on the simulator. The drill itself is not the task — the fifteen minutes after it is. Before starting, prepare a debrief sheet with six headings: factual review; comparison to expected behaviour; communication and decisions; human factors; procedural or training needs; actions and follow-up.

### Execute

Execute the drill and recover. Immediately after recovery, work the debrief sheet aloud, heading by heading. Then complete a formal drill record: type, date, scenario, participants, timings, equipment behaviour, deviations, lessons, and action points — every action point with an owner and a close-out date. The record is finished before the session ends, not later.

### Professional standard

- Debrief conducted immediately after the drill, not at end of session

- At least one honest item recorded under what did not go well

- Every action point has an owner and a close-out date

- Record filed within the same session

### Debrief questions

1. Why is the value of a drill said to sit in the debrief rather than in the simulation itself?

2. What happens to the honesty of debriefs the first time one is used to assign blame?

3. Pick one lesson from today: which document on a real vessel would it change — a checklist, the familiarisation programme, or a decision support tool — and how?

4. What does an action point without an owner and a date actually achieve?

[Model answers not yet written]

## Slot 5 · tg_001 — Build and Validate a TAGOS Row

Post wcf limit calculation and validation · 40 min

### NI task references

Reference

4.4

4.6

7.6

7.17

### Setup

For your simulator vessel's configuration, write one TAGOS-style row for normal DP operation: thrusters online, generators online, bus-tie state, and — the core of the exercise — the maximum permitted pre-failure thrust limit. Compute it from the governing principle: the units surviving the worst case failure must replace the lost capacity. If the worst case failure removes two of four thrusters, the survivors are half the fleet, so thrust must be held low enough beforehand that the remaining half can carry the total demand. Write the number down before executing anything.

### Execute

Set the configuration and settle in DP. Confirm pre-failure thrust sits inside your computed limit. Then apply the worst case failure: remove the entire redundancy group at once. Hold position for 5 minutes. Record peak survivor thrust, peak generator load, and maximum position excursion. Reinstate and stabilise.

### Professional standard

- Thrust limit computed and written before execution

- Pre-failure thrust confirmed inside the computed limit before the failure is applied

- Position held throughout the post-failure period

- Any exceedance of the computed limits explained in the record, not ignored

### Debrief questions

1. Why are thrust limits set against post-failure capacity rather than installed capacity?

2. Your post-failure survivor thrust peaked at a specific number. At roughly what environmental increase would this operation have become unsafe to start?

3. Why does generator step-load capability belong in the same calculation as thruster limits?

4. If a vessel routinely operates above its strategy limits in benign weather without incident, what has actually been lost?

[Model answers not yet written]

[1 variant omitted from this edition]

# Day 17 — Heavy Weather Approaches

120 min across 3 slots · target 120 min

## Slot 1 · approach_001 — Leeward Approach to Structure

Approach and proximity · 35 min

### NI task references

Reference

8.1

8.2

8.3

8.4

8.5

8.6

### Setup

Place virtual structure 500m on 037 degrees True from your current position. You are on the leeward side — the dominant environmental force is pushing you away from the structure. Confirm your abort heading before starting. Write it down.

### Execute

Enter the 500m zone on a heading of approximately 037 degrees True. Reduce to maximum 0.5kts by the 500m boundary. Reduce to maximum 0.3kts by the 200m boundary. Reduce to maximum 0.1kts by the 100m boundary. Transition from JSMH to DP at 100m. Hold at the DP operating radius for 10 minutes. Record approach track, speed at each zone boundary, and DP capture quality.

### Professional standard

- No hard exclusion zone breach

- Speed below 0.5kts at 500m boundary

- Speed below 0.3kts at 200m boundary

- Speed below 0.1kts at 100m boundary

- DP capture within operating radius

- Abort heading confirmed before entry

### Debrief questions

1. This was a leeward approach — what is the failure mode if DP fails while you are at the operating radius?

2. At what point during the approach was the environment helping your deceleration and at what point was it not?

3. What is your abort heading from the operating radius and how long would it take to clear the 500m zone?

4. How did thruster demand change as you closed distance to the structure during the hold?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 2 · approach_002 — Windward Approach to Structure

Approach and proximity · 35 min

### NI task references

Reference

8.1

8.2

8.3

8.4

8.5

8.6

### Setup

Place virtual structure 500m on 217 degrees True from your current position. You are now on the windward side — the dominant environmental force is pushing you toward the structure. Confirm your abort heading before starting. This is more complex on the windward side — think carefully before writing it down.

### Execute

Same speed profile as approach_001. Enter at 217 degrees True, reduce speed at each zone boundary, transition to DP at 100m, hold for 10 minutes. Pay particular attention to how the environment affects your closure rate and thruster demand compared to the leeward approach.

### Professional standard

- No hard exclusion zone breach

- All speed limits observed at each boundary

- Abort heading correctly identified for windward side

- Recognition of increased risk compared to leeward approach

- Higher thruster demand noted and recorded

### Debrief questions

1. What is the failure mode on the windward side compared to the leeward side — why is windward higher risk?

2. How much higher was thruster demand during the windward approach and hold compared to leeward?

3. Why is the abort heading more complex to plan on the windward side?

4. Under what operational circumstances would you accept a windward approach and what additional safeguards would you put in place?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · buffer_017 — buffer

Buffer slot · 50 min — Emergency departure drill — alarm triggered at operating radius

# Day 18 — Worst Case Failure

130 min across 4 slots · target 130 min

## Slot 1 · fail_004 — Worst Case Failure Exercise

Failure management · 40 min

### NI task references

Reference

11.3

11.4

11.10

11.11

11.12

### Setup

Vessel on DP at operating radius of virtual structure. Moderate to heavy conditions. Before starting, state to your DPO what you understand to be the vessels defined worst case failure. DPO will then trigger it without further warning during the hold.

### Execute

When the WCF is triggered, observe the full system response — position excursion, alarm sequence, thruster redistribution. Record the excursion distance. Assess whether the excursion would be within safe limits for a notional subsea operation with divers at 50m depth. Make a safety assessment and communicate it clearly.

### Professional standard

- Correct pre-statement of vessel WCF

- Full alarm sequence observed and recorded

- Position excursion measured accurately

- Correct safety assessment for subsea operation context

- Clear communication of assessment

### Debrief questions

1. What is the defined WCF for this vessel — where is it documented and how was it determined?

2. Was the excursion within acceptable limits for a notional subsea operation and what is that limit based on?

3. What is the FMEA and how does it define the WCF for this vessel class?

4. Under what environmental conditions does the WCF excursion become unacceptable for the operations this vessel typically supports?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · ops_003 — ASOG Application

Operational decision making · 25 min

### NI task references

Reference

7.21

7.22

7.23

7.24

### Setup

DPO will present three operational scenarios. For each scenario you must state the ASOG status, the action required, and who must be informed. No reference material — knowledge only.

### Execute

Scenario 1 — Green status: Conditions and system configuration within all ASOG limits. State status and confirm operation proceeds. Scenario 2 — Yellow status: One parameter approaching its ASOG limit. State status, required action, and communication chain. Scenario 3 — Red or abort criteria: A parameter has exceeded its ASOG limit during an active operation. State status, immediate actions, and who decides whether to continue.

### Professional standard

- Correct ASOG status assessment all three scenarios

- Correct action identified for each status

- Correct communication chain for each scenario

- Correct identification of who has authority to override

### Debrief questions

1. What is the ASOG and who produces it for this vessel and operation?

2. What is the difference between a yellow and red ASOG status in terms of required actions?

3. Who has authority to continue operations outside ASOG limits and what documentation is required?

4. How does the ASOG relate to the vessel FMEA — are they the same document or different?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · ta_002 — FMEA Prediction and Verification

Fmea verification and gap analysis · 35 min

### NI task references

Reference

1.2

4.6

7.6

11.4

### Setup

From your knowledge of the vessel's DP layout, write down the redundancy groups as you understand them. For the loss of each complete group, predict in writing: which thrusters and generators remain, the peak thrust you expect on the survivors, and whether the current environment sits inside the post-failure capability. All predictions are on paper before anything is executed.

### Execute

Allow the model to settle. Remove every element of the first redundancy group from DP. Hold position for 5 minutes. Record peak thruster load on the survivors, maximum position and heading excursion, and alarm count. Reinstate, stabilise fully, then repeat for the second group. Finish by writing one line per group: prediction confirmed, or gap found — and exactly what the gap was.

### Professional standard

- Predictions written in full before execution

- Both groups tested, with full stabilisation between tests

- Every gap between predicted and observed behaviour explicitly documented

- Environment assessed against post-failure capability before each removal

### Debrief questions

1. What is an FMEA gap analysis, and why is a difference between documented failure effects and observed behaviour treated as a serious finding rather than a curiosity?

2. Why do formal trials require the witnessing practitioner to be independent of the vessel?

3. Which group loss produced the worse station-keeping outcome, and does that ranking match your pre-test capability assessment?

4. How would rotating the environment 90 degrees change which group loss is worst?

5. Why must both the prediction and the result survive in the record even when the prediction was wrong?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · tg_003 — Force Off Recognition

Environmental exceedance recognition · 30 min

### NI task references

Reference

7.17

7.21

9.2

9.12

### Setup

Settle in DP in the starting environment and log baseline thrust and generator load. Force off is being pushed outside the capability envelope by environment alone, with nothing failed — the loss-of-position mode this task teaches you to see coming. Before the first wind step, decide and write down the mean thrust percentages at which you would declare an advisory status and a degraded status. Note: the power and thrust monitoring habits this task builds map to NI logbook Section 9 references, which can only be signed off during active DP sea time — here you are building the habit, not the sign-off.

### Execute

Increase simulated wind in 5 knot steps, allowing two minutes of settling at each step. At each step record wind speed, mean thrust percentage, and generator load. Declare your pre-decided status changes aloud as their thresholds are crossed. Stop the escalation once mean thrust exceeds 75 percent or position becomes unstable, then step the environment back down in a controlled de-escalation and record the recovery.

### Professional standard

- Status thresholds written before the first wind step

- Every step settled and recorded before the next increase

- Declarations made when thresholds are crossed, not retrospectively

- Controlled de-escalation completed with the vessel stable at every step down

### Debrief questions

1. Define drift off, drive off and force off in one sentence each. Which of the three gives the least warning per unit of attention, and why?

2. Why do operating guidelines put status triggers on thrust and generator load rather than on absolute wind speeds?

3. How does heading relative to the environment move the wind speed at which force off occurs?

4. At your advisory declaration point, what should a DPO on a working vessel actually be doing — list the concrete actions, not the sentiment?

[Model answers not yet written]

[1 variant omitted from this edition]

# Day 19 — Watchkeeping Standards

120 min across 4 slots · target 120 min

## Slot 1 · watch_001 — Sustained Position Keeping Observation

Watchkeeping · 30 min

### NI task references

Reference

9.1

9.2

9.3

9.4

9.5

9.7

9.8

### Setup

Vessel on DP in moderate conditions. DPO will gradually increase environmental conditions during the observation period. Your role is monitor and assess — do not intervene unless a genuine safety issue develops.

### Execute

Monitor the DP system holding position for 30 minutes while conditions increase. Record position footprint, thruster demand, reference quality, and current vector at 5-minute intervals. Identify any trends. Assess at what point intervention would be required. At each 5-minute mark write your observation before looking at the previous entry — do not let previous readings anchor your assessment.

### Professional standard

- All parameters recorded at every 5-minute interval without prompting

- Trends correctly identified from data

- Correct intervention threshold identified with reasoning

- Assessment does not simply mirror the previous reading

### Debrief questions

1. What trends did you identify across the 30-minute period — were conditions genuinely deteriorating or just varying?

2. At what point would you have recommended intervention and what specific evidence supported that recommendation?

3. What is the difference between active watchkeeping and passive observation — are you doing one or both?

4. What DP log entries would you have made during this 30-minute period?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · watch_002 — DP Watch Handover

Watch handover · 25 min

### NI task references

Reference

2.5

2.6

2.7

### Setup

Vessel on DP in moderate conditions with a virtual structure active at 100m. A realistic handover scenario — the incoming DPO needs everything.

### Execute

Part 1 — Give handover: You are the outgoing DPO. Brief your DPO on full system status covering every checklist item. Vessel position and heading, mode and setpoint, reference systems and quality, sensor status, thruster configuration, power configuration, environmental conditions, nearby structures and working side, alarms active or recently cleared, any system degradation, operational requirements for the next watch. Part 2 — Receive handover: Swap roles. Receive a handover from your DPO. Ask all questions you would ask as an incoming DPO. Identify any gaps in what you were told.

### Professional standard

- All handover items covered without prompting

- Minimum 5 questions asked as incoming DPO

- Gaps in received handover correctly identified

- Refusal to accept an incomplete handover demonstrated

### Debrief questions

1. What items did you miss in the handover you gave — and are those items safety-critical?

2. Under what circumstances would you refuse to accept a watch handover?

3. What is the single most important piece of information in a DP watch handover?

4. How does a DP watch handover differ from a standard bridge watchkeeping handover?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · watch_003 — DP Log Entry Discipline

Watchkeeping · 20 min

### NI task references

Reference

9.12

9.13

9.14

9.15

### Setup

Vessel on DP. Paper and pen ready for log entries. DPO will trigger events during a 20-minute period without announcing what or when. Your job is to log everything correctly.

### Execute

During the 20-minute period your DPO will trigger several events — a reference quality degradation, a wind increase, a thruster loading change, and a position alert. For each event make a DP log entry in the correct format: time, event description, action taken, system status after action. DPO assesses completeness and accuracy of all entries at the end.

### Professional standard

- Every event logged within 2 minutes of occurrence

- Timestamp accurate to within 1 minute

- All required fields completed for each entry

- Action taken clearly stated for each entry

- No events missed

### Debrief questions

1. What information is mandatory in every DP log entry — what are the minimum required fields?

2. Why is timestamp accuracy critical in a DP log — what happens to your log if a DP incident occurs?

3. What constitutes an event worthy of a log entry versus routine monitoring that does not need logging?

4. Who reads your DP log entries after an incident — and how should that knowledge affect how you write them?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · buffer_019 — buffer

Buffer slot · 45 min — Sustained observation near structure — watch_001a variant

# Day 20 — First Full Operation

200 min across 3 slots · target 200 min

## Slot 1 · ops_001 — Full DP Operation — End to End

Operational planning and execution · 120 min

### NI task references

Reference

7.1

7.4

7.5

7.20

7.24

7.25

8.1

8.2

8.5

8.6

10.1

10.2

2.5

2.6

### Setup

Cold screen. No pre-configuration. This is the full operation with no prompting from DPO unless a genuine safety issue develops.

### Execute

Complete the following sequence without prompting. Phase 1 — Setup: Full system setup from scratch. All checklists completed. Phase 2 — Transit: Move vessel from open water to 500m standoff of virtual structure. Phase 3 — Zone entry: Complete 500m zone entry checklist verbally. Approach to operating radius on JSMH then DP. Phase 4 — Hold: Hold at operating radius for 20 minutes. DPO will trigger one alarm during hold. Phase 5 — Handover: Conduct a full watch handover to DPO. Phase 6 — Departure: Receive the watch back. Execute controlled departure. Return to open water. Save session record.

### Professional standard

- All phases completed in correct sequence without prompting

- No zone breaches during approach or hold

- Alarm during hold responded to correctly

- Handover covers all required items

- Departure executed cleanly

- Session record saved with all observations completed

### Debrief questions

1. Which phase was hardest and what specifically caused the difficulty?

2. What would you do differently on the next full operation?

3. How did the alarm during hold affect your situational awareness for the remainder of the operation?

4. Were your log entries and session record complete enough to reconstruct the operation from the record alone?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 2 · tg_002 — Maintenance Configuration and Go/No-Go

Degraded configuration planning · 35 min

### NI task references

Reference

4.5

4.6

7.6

11.3

### Setup

Take one thruster out of service — deselected and treated as under maintenance for the whole task. Recompute the operating strategy row for this degraded configuration: which units now survive the worst case failure, and what the new pre-failure thrust limit must be. Then make a written go or no-go call: is the current environment still acceptable for DP operations in this state? Write the reasoning, not just the answer.

### Execute

If your call is go: settle in the degraded configuration, confirm thrust inside the recomputed limit, apply the worst case failure to the remaining worst group, and hold for 5 minutes. If your call is no-go: reduce the simulated environment step by step until the answer becomes go, record that threshold, then execute the failure at the threshold. Either way, record survivor thrust, maximum excursion, and heading deviation.

### Professional standard

- Recomputed limit and go/no-go decision written before operating

- Threshold reasoning recorded, not just the number

- Post-failure control assessed honestly against the recomputed limits

- Heading held throughout the post-failure period

### Debrief questions

1. Operating with a thruster offline is a textbook task appropriate mode decision. What must exist on paper, and who must sign it, before a real vessel operates that way?

2. Who is notified, and what operating-guideline status change occurs, when DP equipment comes out of service mid-operation?

3. Why is reinstating equipment done outside the work area wherever possible?

4. Your go/no-go threshold was a single number. Describe what sits on each side of it.

[Model answers not yet written]

## Slot 3 · dv_001 — The Paperwork Holding You in Position

Dp documentation and verification · 45 min

### NI task references

Reference

1.1

1.1.5

1.2

1.3.1

### Setup

Establish a stable DP hold at your datum with the model fully built. The vessel does nothing but hold position for this entire task. That is the point: while she holds, you audit the documentation that certifies she can. Have a notebook ready — you will be moving around the bridge, not sitting at the desk.

### Execute

While the vessel holds position, locate aboard — or, where the simulator vessel's set differs, name and state the correct location of — each of the following: the DPVAD; the FMEA and its proving trials results; the DP operations manual; annual survey and periodic (5-year) test records; DP capability plots (fully operational AND post-worst-case-failure); the location checklist; the watchkeeping checklist; the blackout recovery procedure; the list of critical components; and the ASOG if the vessel carries one. For each, record where it lives, its date or revision, and one sentence on what it proves about the system holding position in front of you right now. Then make three live cross-checks: (1) confirm the equipment class stated on the DPVAD matches the redundancy actually set up on the desk — bus configuration, thruster split across power systems, number and diversity of position references; (2) plot the current wind and sea state against the post-worst-case-failure capability plot and state whether you are inside the envelope, and by how much; (3) confirm the watchkeeping checklist in use at the desk is the version referenced in the operations manual.

### Professional standard

- Every listed document located, or its absence positively identified and recorded — no line left blank

- DPVAD equipment class correctly matched against the live desk configuration, with the specific evidence stated (bus state, thruster allocation, PRS count and diversity)

- Current conditions correctly placed inside or outside the post-worst-case-failure capability envelope, with the margin stated

- Currency correctly assessed for each record: annual survey within three months either side of the DPVAD anniversary date, periodic testing within five years, FMEA revision reflecting the vessel as she currently is

### Debrief questions

1. The annual survey does not just confirm the equipment works — it tests the vessel's ability to keep position after single failures for her assigned equipment class, and validates the FMEA and operations manual. Why is that distinction the entire point of the survey, and what would a survey that only function-tested equipment fail to prove?

2. Name three things that cause a DPVAD to cease to be valid, and explain for each one exactly why it breaks the chain of assurance between the paperwork and the vessel holding position right now.

3. Your online consequence analysis and the post-worst-case-failure capability plot answer the same question — 'will she hold if the worst single failure happens now?' — in two different ways. What does each rely on, what does each assume, and why does the guidance require both rather than either one alone?

4. The FMEA must be kept updated so that it remains current, and no significant change may be made to the DP system after survey without sanction, except direct like-for-like replacement for repair. A yard fitted a different-model thruster drive last refit. What has to happen to the FMEA, the proving trials and the DPVAD before this vessel legitimately works a Class 2 operation again?

5. You join this vessel cold tomorrow as an incoming DPO. Of everything you handled in this task, which single document do you read first, and why does it beat the others?

[Model answers not yet written]

[1 variant omitted from this edition]

# Day 21 — Advanced Sensor Management

120 min across 4 slots · target 120 min

## Slot 1 · sen_001 — Reference System Weighting

Reference management · 20 min

### NI task references

Reference

5.1

5.2

7.9

7.10

### Setup

Two DGPS references active. Vessel stable on DP. Open the Sensors tab and observe the weighting assigned to each reference before doing anything.

### Execute

Record initial weights for both references. Ask your DPO to introduce simulated HDOP degradation on one reference. Observe how the weighting changes. Record the new weights. Note how position quality changes. Assess at what point you would manually inhibit the degraded reference rather than waiting for auto-rejection. Re-enable and confirm both references return to healthy status.

### Professional standard

- Correctly record initial reference weights

- Observe and record weight change after degradation

- Correct threshold assessment for manual inhibit

- Correct procedure for re-enabling recovered reference

### Debrief questions

1. How does the DP Kalman filter decide how much weight to assign each reference — what factors influence this?

2. What HDOP value would concern you and at what value would you act?

3. At what point would you manually inhibit a reference rather than waiting for auto-rejection — and what is the risk of waiting?

4. What is the difference between a reference being auto-rejected and manually inhibited — and does it matter which way it happens?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · sen_002 — GPS Failure Response

Reference management · 20 min

### NI task references

Reference

5.1

5.2

7.9

11.9

### Setup

Two DGPS references active. Vessel stable on DP in moderate conditions. Note current position quality.

### Execute

Without warning your DPO will disable one GPS reference. Observe the alarm sequence. Record which alarms triggered and in what order. Assess your remaining reference configuration. Decide whether to continue or go off DP. Re-enable the failed GPS and confirm re-acceptance procedure.

### Professional standard

- Alarm acknowledged within 15 seconds

- Correct assessment of remaining reference configuration

- Correct go or no-go decision with reasoning

- Correct re-acceptance procedure for recovered reference

### Debrief questions

1. What is the minimum reference configuration for your vessel's typical operations and where is this defined?

2. Why does losing one GPS not automatically mean you must go off DP?

3. What changed about your position quality with only one reference active?

4. When re-enabling a recovered reference — why must you confirm it is healthy before accepting it back into the solution?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 3 · sen_003 — Gyro Failure Response

Sensor management · 20 min

### NI task references

Reference

6.1

7.8

11.9

### Setup

Both gyros active. Note heading agreement between gyro 1 and gyro 2 — should be within 0.5 degrees. Vessel stable on DP.

### Execute

Without warning your DPO will introduce a 5 degree heading error on one gyro. Observe how the DP responds. Does it auto-reject? Does heading control degrade? Record the heading excursion. Manually inhibit the faulty gyro. Observe recovery. Record time from fault introduction to stable heading.

### Professional standard

- Identify faulty gyro within 30 seconds

- Correct inhibit procedure executed

- Recovery confirmed before continuing

- Time from fault to stable heading recorded

### Debrief questions

1. How does the DP decide which gyro to trust when two gyros disagree — what is the voting logic?

2. What heading excursion is acceptable during a gyro failure response near a structure?

3. Why is gyro failure more immediately dangerous to vessel position than a GPS failure?

4. How does heading error propagate into position error — explain the mechanism?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · buffer_021 — buffer

Buffer slot · 60 min — Combined sensor degradation scenario — reference plus gyro

# Day 22 — Compound Failure Scenarios

120 min across 4 slots · target 120 min

## Slot 1 · fail_001 — Single Thruster Failure Near Structure

Failure management · 30 min

### NI task references

Reference

11.3

11.10

11.12

### Setup

Vessel on DP at operating radius of virtual structure. Moderate conditions settled. DPO will trigger a single thruster failure at an unannounced time during the hold.

### Execute

When the thruster failure occurs, hold position with remaining thrusters. Identify the faulted thruster. Assess new worst case failure. Assess remaining capability for current conditions. Decide whether to maintain position or increase standoff distance. Communicate your decision and reasoning to DPO as if communicating to OIM.

### Professional standard

- Position maintained within double the normal footprint

- Faulted thruster identified within 30 seconds

- Correct new worst case failure identified

- Correct capability assessment for current conditions

- Clear reasoned decision with communication

### Debrief questions

1. How did your worst case failure change when one thruster was lost — and why does that matter?

2. Was your decision to hold position or increase standoff correct given remaining capability and conditions?

3. What would the ASOG say about continuing operations with this degraded thruster complement?

4. What is your new abort threshold with reduced thrusters — at what environmental condition would you now go off DP?

[Model answers not yet written]

[2 variants omitted from this edition]

## Slot 2 · fail_002 — Power Failure Response

Failure management · 30 min

### NI task references

Reference

11.3

11.11

11.12

### Setup

Vessel on DP. DPO will trigger a generator dropout on one bus without warning. Know your vessel power configuration — which thrusters are on which bus — before this drill.

### Execute

When the generator dropout occurs, observe which thrusters lose power. Observe auto load shedding. Assess remaining propulsion capability. Prevent blackout. Either hold position or initiate controlled departure. Communicate to DPO as OIM.

### Professional standard

- Correct identification of thrusters lost on faulted bus

- Auto load shedding observed and understood

- Blackout prevention demonstrated

- Correct hold or depart decision

- Clear communication of situation

### Debrief questions

1. Which thrusters lost power and does the bus configuration of your vessel explain why those ones specifically?

2. What is the role of blackout prevention in this scenario — what would happen without it?

3. Why does the bus tie position matter so much in a power failure scenario?

4. What is the worst case failure for this vessel and how does this event relate to it — was this the WCF or something different?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · ta_003 — Mini Trials Programme

Trials programme design and record · 45 min

### NI task references

Reference

1.1

1.1.4

1.3.3

7.20

### Setup

Design a three-test programme covering one power system test, one thruster or propulsion test, and one reference or sensor test. Write a full test sheet for each, then sequence them deliberately — decide which runs first and why. Prepare a single trials record header: date, session, environment, vessel configuration.

### Execute

Run the programme in your planned sequence. Between tests: reinstate, verify restoration, and allow full stabilisation before the next test begins. Log actual start and finish times per test, results against expected, and any deviation from your written method the moment it happens. Close out the record with a summary of findings and any items you would carry forward to a future session.

### Professional standard

- Programme fully written and sequenced before the first test

- Restoration verified — not assumed — before each subsequent test

- Deviations from written method logged at the time, not afterwards

- Record complete enough that another TDPO could repeat the programme without asking a single question

### Debrief questions

1. What logic did you use to sequence the three tests, and what would go wrong if you ran them in reverse order?

2. Why must restoration be verified rather than assumed before the next test starts?

3. What makes a trials record repeatable, and why does repeatability matter more than neatness?

4. In real annual trials, the trials document exists in a pre-trials revision written before anyone steps aboard. Why is the authorship done in advance?

5. Which of your three tests would you promote into a recurring personal drill, and why that one?

[Model answers not yet written]

## Slot 4 · buffer_022 — buffer

Buffer slot · 15 min — Compound failure — thruster loss plus reference degradation simultaneously

# Day 23 — Heavy Weather Approaches and Compound Failures

120 min across 3 slots · target 120 min

## Slot 1 · approach_003 — Extended Hold at Operating Radius

Approach and proximity · 30 min

### NI task references

Reference

8.5

8.6

9.1

9.2

### Setup

Position vessel at DP operating radius of virtual structure. Moderate conditions initially. DPO will ramp wind gradually during the hold.

### Execute

Hold position at the DP operating radius for 20 minutes. During the hold your DPO will gradually increase wind from 15kts to 30kts. Record position footprint at 15, 20, 25, and 30kts. Record thruster demand at each interval. Identify the point at which you would recommend moving further from structure or reviewing operations with the OIM.

### Professional standard

- Position maintained throughout hold

- Correct identification of degrading margin

- Reasoned recommendation point with evidence

- Correct communication chain identified

### Debrief questions

1. At what wind speed did your position footprint become unacceptable at this operating radius?

2. What is the ASOG concept and how would a properly defined ASOG have defined your action limit here?

3. Who do you inform and in what order when you identify a deteriorating situation during a hold?

4. What is the difference between moving further from the structure and going fully off DP — when would you choose each?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · fail_003 — Compound Failure — Reference Loss During Approach

Failure management · 35 min

### NI task references

Reference

11.3

11.9

11.12

### Setup

Begin an active approach to virtual structure from 500m. DPO will trigger reference loss at an unannounced point during the approach — inside the 500m zone.

### Execute

Continue the approach normally. When reference loss occurs inside the 500m zone, make an immediate decision — continue approach on remaining references or abort. If continuing, hold position and reassess. If aborting, execute a controlled departure. There is no correct answer predetermined — your decision must be reasoned and clearly communicated.

### Professional standard

- Immediate recognition of reference loss

- Correct minimum reference assessment at moment of failure

- Clear reasoned decision within 30 seconds

- Correct execution of chosen action

- Clear communication throughout

### Debrief questions

1. What was your reference configuration at the moment of failure and was minimum configuration still met?

2. Was your decision to continue or abort correct given your proximity to structure at the moment of failure?

3. How does being inside the 500m zone change the minimum reference requirement compared to open water?

4. What would the ASOG say about this scenario — does it define a minimum reference requirement for inside 500m?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · buffer_023 — buffer

Buffer slot · 55 min — Full scenario — approach in heavy weather with unannounced failure at operating radius

# Day 24 — Watchkeeping and Operational Decision Making

120 min across 4 slots · target 120 min

## Slot 1 · watch_003 — DP Log Entry Discipline

Watchkeeping · 20 min

### NI task references

Reference

9.12

9.13

9.14

9.15

### Setup

Vessel on DP. Paper and pen ready for log entries. DPO will trigger events during a 20-minute period without announcing what or when. Your job is to log everything correctly.

### Execute

During the 20-minute period your DPO will trigger several events — a reference quality degradation, a wind increase, a thruster loading change, and a position alert. For each event make a DP log entry in the correct format: time, event description, action taken, system status after action. DPO assesses completeness and accuracy of all entries at the end.

### Professional standard

- Every event logged within 2 minutes of occurrence

- Timestamp accurate to within 1 minute

- All required fields completed for each entry

- Action taken clearly stated for each entry

- No events missed

### Debrief questions

1. What information is mandatory in every DP log entry — what are the minimum required fields?

2. Why is timestamp accuracy critical in a DP log — what happens to your log if a DP incident occurs?

3. What constitutes an event worthy of a log entry versus routine monitoring that does not need logging?

4. Who reads your DP log entries after an incident — and how should that knowledge affect how you write them?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · ops_004 — Risk Assessment and Toolbox Talk

Operational planning and execution · 25 min

### NI task references

Reference

7.24

7.25

9.11

9.13

### Setup

DPO describes a notional DP operation: vessel holding position near a fixed platform, ROV deployed to 50m depth, a second vessel on standby 300m away, wind 20kts and increasing. You must conduct a risk assessment and toolbox talk before the operation proceeds.

### Execute

Conduct a verbal risk assessment covering all major risk categories: environmental conditions and forecast, vessel capability and ASOG status, reference configuration and quality, SIMOPS considerations, escape route and abort criteria, communication plan, personnel briefing requirements. Then conduct a one-minute toolbox talk covering the top three risks and the actions planned for each.

### Professional standard

- All major risk categories addressed

- Specific risks identified not just categories

- Toolbox talk clear and under 2 minutes

- Top three risks correctly prioritised

- Action plan stated for each top risk

### Debrief questions

1. What risk categories did you miss and why are they significant for this type of operation?

2. What makes a toolbox talk effective versus one that people switch off during?

3. How does the risk assessment change if wind increases by 10kts during the operation?

4. Who must be present for the toolbox talk and what happens if a key person is absent?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · watch_002 — DP Watch Handover

Watch handover · 25 min

### NI task references

Reference

2.5

2.6

2.7

### Setup

Vessel on DP in moderate conditions with a virtual structure active at 100m. A realistic handover scenario — the incoming DPO needs everything.

### Execute

Part 1 — Give handover: You are the outgoing DPO. Brief your DPO on full system status covering every checklist item. Vessel position and heading, mode and setpoint, reference systems and quality, sensor status, thruster configuration, power configuration, environmental conditions, nearby structures and working side, alarms active or recently cleared, any system degradation, operational requirements for the next watch. Part 2 — Receive handover: Swap roles. Receive a handover from your DPO. Ask all questions you would ask as an incoming DPO. Identify any gaps in what you were told.

### Professional standard

- All handover items covered without prompting

- Minimum 5 questions asked as incoming DPO

- Gaps in received handover correctly identified

- Refusal to accept an incomplete handover demonstrated

### Debrief questions

1. What items did you miss in the handover you gave — and are those items safety-critical?

2. Under what circumstances would you refuse to accept a watch handover?

3. What is the single most important piece of information in a DP watch handover?

4. How does a DP watch handover differ from a standard bridge watchkeeping handover?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · buffer_024 — buffer

Buffer slot · 50 min — Full watch cycle — hold, log entries, alarm, handover

# Day 25 — Pre-Completion Review and Open Water Drills

120 min across 4 slots · target 120 min

## Slot 1 · review_003 — Self-Assessment — Pre-Completion Review

Self assessment · 20 min

### NI task references

None recorded.

### Setup

Vessel on DP. Run the system while you conduct this assessment.

### Execute

Hold position and reflect honestly. Are you ready for the NI Simulator Course — yes or no, with evidence. What is the one skill you are least confident about? What scenario would most expose that weakness? What is the one thing you most wish you had more time to practise? Discuss with DPO.

### Professional standard

- Honest yes or no to readiness question with evidence

- One specific skill identified as weakest

- Specific scenario identified as most exposing

- Final five sessions have a concrete plan

### Debrief questions

1. Is your readiness assessment consistent with what your session records show — or are you being optimistic?

2. Does your DPO agree with your self-assessment — and if not who is more likely to be right?

3. What is the single most important thing you can do in days 26 to 30 to close the biggest gap?

4. If the NI Simulator Course started tomorrow what would be the first thing that tripped you up?

[Model answers not yet written]

## Slot 2 · ops_002 — 500m Zone Entry Checklist Walk-Through

Operational planning and execution · 20 min

### NI task references

Reference

7.20

7.24

7.25

### Setup

Vessel at 500m standoff of virtual structure. Before approaching, complete the full zone entry checklist verbally with DPO. Every item, in order, with reasoning for why each item matters.

### Execute

Work through the 500m zone entry checklist item by item. For each item state what you are checking, confirm its status, and explain why it matters before entering the 500m zone. DPO challenges any item you cannot explain. After completing the checklist verbally, execute the approach.

### Professional standard

- All checklist items covered without prompting

- Correct status confirmed for each item

- Clear reasoning given for each item

- No items skipped when challenged

### Debrief questions

1. Which checklist items have the highest safety significance for this specific operation?

2. What happens operationally if you enter the 500m zone without completing this checklist?

3. How does the working side assessment change your risk picture and your approach plan?

4. What SIMOPS information do you need from the installation before entering the 500m zone?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · jc_007 — Blind Stop Challenge

Stopping and speed management · 20 min

### NI task references

Reference

7.5

7.14

7.18

### Setup

DPO sets a random drift duration between 3 and 8 minutes and an environment the trainee has not seen. Trainee does not know duration or conditions in advance.

### Execute

DPO starts the drift without telling the trainee how long it will run. Trainee observes the drift. When DPO gives the instruction to stop — stop the vessel. No preparation time. Residual velocity and time to stop are your score.

### Professional standard

- Residual velocity below 0.2kts

- Vessel stopped within 60 seconds of instruction

- Correct assessment of dominant environmental force stated before acting

- Heading maintained during stop

### Debrief questions

1. How did unknown conditions change your approach compared to prepared drills?

2. What information did you gather in the first 30 seconds of observation — and was it enough?

3. How does this drill simulate real operational conditions when DP fails unexpectedly?

4. What is the difference in your mental process between a prepared stop and a blind stop?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 4 · buffer_025 — buffer

Buffer slot · 60 min — Repeat weakest identified skill from review_003

# Day 26 — Full Operation Without Prompting

120 min across 1 slot · target 120 min

## Slot 1 · ops_001 — Full DP Operation — End to End

Operational planning and execution · 120 min

### NI task references

Reference

7.1

7.4

7.5

7.20

7.24

7.25

8.1

8.2

8.5

8.6

10.1

10.2

2.5

2.6

### Setup

Cold screen. No pre-configuration. This is the full operation with no prompting from DPO unless a genuine safety issue develops.

### Execute

Complete the following sequence without prompting. Phase 1 — Setup: Full system setup from scratch. All checklists completed. Phase 2 — Transit: Move vessel from open water to 500m standoff of virtual structure. Phase 3 — Zone entry: Complete 500m zone entry checklist verbally. Approach to operating radius on JSMH then DP. Phase 4 — Hold: Hold at operating radius for 20 minutes. DPO will trigger one alarm during hold. Phase 5 — Handover: Conduct a full watch handover to DPO. Phase 6 — Departure: Receive the watch back. Execute controlled departure. Return to open water. Save session record.

### Professional standard

- All phases completed in correct sequence without prompting

- No zone breaches during approach or hold

- Alarm during hold responded to correctly

- Handover covers all required items

- Departure executed cleanly

- Session record saved with all observations completed

### Debrief questions

1. Which phase was hardest and what specifically caused the difficulty?

2. What would you do differently on the next full operation?

3. How did the alarm during hold affect your situational awareness for the remainder of the operation?

4. Were your log entries and session record complete enough to reconstruct the operation from the record alone?

[Model answers not yet written]

[2 variants omitted from this edition]

# Day 27 — Worst Case Failure Near Structure

120 min across 3 slots · target 120 min

## Slot 1 · fail_004 — Worst Case Failure Exercise

Failure management · 40 min

### NI task references

Reference

11.3

11.4

11.10

11.11

11.12

### Setup

Vessel on DP at operating radius of virtual structure. Moderate to heavy conditions. Before starting, state to your DPO what you understand to be the vessels defined worst case failure. DPO will then trigger it without further warning during the hold.

### Execute

When the WCF is triggered, observe the full system response — position excursion, alarm sequence, thruster redistribution. Record the excursion distance. Assess whether the excursion would be within safe limits for a notional subsea operation with divers at 50m depth. Make a safety assessment and communicate it clearly.

### Professional standard

- Correct pre-statement of vessel WCF

- Full alarm sequence observed and recorded

- Position excursion measured accurately

- Correct safety assessment for subsea operation context

- Clear communication of assessment

### Debrief questions

1. What is the defined WCF for this vessel — where is it documented and how was it determined?

2. Was the excursion within acceptable limits for a notional subsea operation and what is that limit based on?

3. What is the FMEA and how does it define the WCF for this vessel class?

4. Under what environmental conditions does the WCF excursion become unacceptable for the operations this vessel typically supports?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 2 · alarm_005 — Drive Off Recognition and Response

Alarm recognition and response · 20 min

### NI task references

Reference

11.6

11.7

11.8

11.10

### Setup

Vessel on DP near virtual structure. DPO will simulate a drive off without warning at an unannounced time.

### Execute

When drive off begins — vessel accelerating away from setpoint with thrusters at maximum in the wrong direction — recognise it, select joystick or manual immediately, and arrest the vessel movement. Record time from drive off initiation to your recognition. Record distance travelled before arrested.

### Professional standard

- Drive off recognised within 30 seconds

- Correct immediate mode selection — joystick or manual

- Vessel movement arrested

- No collision with virtual structure

- Post drive off checks completed

### Debrief questions

1. What are the visual signatures of a drive off on the DP display that distinguish it from a normal position excursion?

2. Why does a drive off require immediate manual intervention rather than waiting for the DP to self-correct?

3. What is the difference between a drive off and a position excursion in terms of cause and response?

4. What checks must be completed after a drive off before you return the vessel to DP control?

[Model answers not yet written]

[1 variant omitted from this edition]

## Slot 3 · buffer_027 — buffer

Buffer slot · 60 min — Full scenario — approach plus WCF triggered at operating radius plus departure

# Day 28 — Mock Oral Examination

120 min across 3 slots · target 120 min

## Slot 1 · review_004 — Pre-Course Mock Oral Examination

Self assessment · 30 min

### NI task references

None recorded.

### Setup

Vessel on DP in moderate conditions. DPO conducts a verbal knowledge check while the vessel holds position.

### Execute

Hold position on DP while your DPO asks a minimum of 12 questions across system knowledge, failure responses, operational decisions, ASOG application, and watchkeeping. Answer without notes. The NI Simulator Course is a practical skills course not an examination — this is preparation for a productive learning experience, not an exam drill.

### Professional standard

- Correct answer to minimum 10 of 12 questions

- No single topic area completely blank

- Answers specific not vague

- Confidence maintained throughout even when uncertain

### Debrief questions

1. Which topic area was weakest and what is your plan to address it before the real course?

2. Were there any questions where you knew the answer but could not articulate it clearly?

3. How did your confidence hold up under examination conditions — did uncertainty show?

4. Based on this mock examination — are you ready for the NI Simulator Course?

[Model answers not yet written]

## Slot 2 · buffer_028a — buffer

Buffer slot · 45 min — Revise weakest topic areas from review_004

## Slot 3 · buffer_028b — buffer

Buffer slot · 45 min — Practical drill on weakest practical skill from review_004

# Day 29 — Full Assessment Operation

120 min across 1 slot · target 120 min

## Slot 1 · ops_001 — Full DP Operation — End to End

Operational planning and execution · 120 min

### NI task references

Reference

7.1

7.4

7.5

7.20

7.24

7.25

8.1

8.2

8.5

8.6

10.1

10.2

2.5

2.6

### Setup

Cold screen. No pre-configuration. This is the full operation with no prompting from DPO unless a genuine safety issue develops.

### Execute

Complete the following sequence without prompting. Phase 1 — Setup: Full system setup from scratch. All checklists completed. Phase 2 — Transit: Move vessel from open water to 500m standoff of virtual structure. Phase 3 — Zone entry: Complete 500m zone entry checklist verbally. Approach to operating radius on JSMH then DP. Phase 4 — Hold: Hold at operating radius for 20 minutes. DPO will trigger one alarm during hold. Phase 5 — Handover: Conduct a full watch handover to DPO. Phase 6 — Departure: Receive the watch back. Execute controlled departure. Return to open water. Save session record.

### Professional standard

- All phases completed in correct sequence without prompting

- No zone breaches during approach or hold

- Alarm during hold responded to correctly

- Handover covers all required items

- Departure executed cleanly

- Session record saved with all observations completed

### Debrief questions

1. Which phase was hardest and what specifically caused the difficulty?

2. What would you do differently on the next full operation?

3. How did the alarm during hold affect your situational awareness for the remainder of the operation?

4. Were your log entries and session record complete enough to reconstruct the operation from the record alone?

[Model answers not yet written]

[2 variants omitted from this edition]

# Day 30 — DPO Sign-Off and Logbook Review

120 min across 2 slots · target 120 min

## Slot 1 · review_004 — Pre-Course Mock Oral Examination

Self assessment · 30 min

### NI task references

None recorded.

### Setup

Vessel on DP in moderate conditions. DPO conducts a verbal knowledge check while the vessel holds position.

### Execute

Hold position on DP while your DPO asks a minimum of 12 questions across system knowledge, failure responses, operational decisions, ASOG application, and watchkeeping. Answer without notes. The NI Simulator Course is a practical skills course not an examination — this is preparation for a productive learning experience, not an exam drill.

### Professional standard

- Correct answer to minimum 10 of 12 questions

- No single topic area completely blank

- Answers specific not vague

- Confidence maintained throughout even when uncertain

### Debrief questions

1. Which topic area was weakest and what is your plan to address it before the real course?

2. Were there any questions where you knew the answer but could not articulate it clearly?

3. How did your confidence hold up under examination conditions — did uncertainty show?

4. Based on this mock examination — are you ready for the NI Simulator Course?

[Model answers not yet written]

## Slot 2 · buffer_030 — buffer

Buffer slot · 90 min — Logbook review — all NI task sections reviewed with DPO. Confirm all passive-eligible sections addressed. Identify remaining active requirements.